3GPP TSG-SA WG6 Meeting #56	S6-232395

# Gothenburg, Sweden, 21th August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0002 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding missing A-ADRF API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The list of A-ADRF APIs does not currently include the SS_AADRF_Location_Accuracy_Data API, though it is supported in the procedure for location accuracy analytics. The API needs to be added or the list will be incomplete. The naming of API operations in the list of ADAES and A-ADRF APIs are updated, to align with the existing naming convention. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The API SS_AADRF_Location_Accuracy_Data and its description is included in the A-ADRF APIs list. API operations in the list of ADAES and A-ADRF APIs are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of A-ADRF API is incomplete (due to missing SS_AADRF_Location_Accuracy_Data API and its description) leading to incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2, 9.2.9.4, 9.2.10.2, 9.3.2, 9.3.3, 9.3.4, 9.3.5, 9.3.6, 9.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |

* * * Next Change * * * *

#### 9.2.9.4	Get

API operation name: edge_analytics_get

Description: The consumer requests edge analytics data.

Inputs: See clause 8.8.3.8.

Outputs: See clause 8.8.3.9.

See clause 8.8.2.2 for details of usage of this operation.

* * * Next Change * * * *

#### 9.2.10.2	Get

API operation name: slice_usage_stats_get

Description: The consumer requests and receives slice usage statistics from ADAE server.

Inputs: See clause 8.7.4.7.

Outputs: See clause 8.7.4.8.

See clause 8.7.3 for details of usage of this operation.

* * * Next Change * * * *

### 9.3.2	A-ADRF APIs

Table 9.3.2-1 illustrates the A-ADRF APIs.

Table 9.3.2-1: List of A-ADRF APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_AADRF_Data_Collection | Subscribe | ADAES | Subscribe / Notify |
|  | Notify |  |  |
| SS_ AADRF_Historical_ServiceAPI_Logs | Get | ADAES | Request / Response |
| SS_AADRF_NetworkSlice_Data | Get | ADAES | Request / Response |
| SS_AADRF_Location_Accuracy_Data | Get | ADAES | Request / Response |
| SS_AADRF_EdgeData_Collection | Subscribe | ADAES | Subscribe / Notify |
|  | Notify | ADAES |  |

* * * Next Change * * * *

### 9.3.3	SS_AADRF_Data_Collection API

#### 9.3.3.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF for subscribing for offline data collection and for getting notified about the offline data/statistics.

#### 9.3.3.2	Subscribe

API operation name: Data_Collection_Subscribe

Description: The consumer subscribes for offline data from A-ADRF.

Inputs: See clause 8.2.4.4.

Outputs: See clause 8.2.4.5.

See clause 8.2.2 for details of usage of this operation.

#### 9.3.3.3	Notify

API operation name:  Data_Collection_Notify

Description: The consumer is receiving the offline data from A-ADRF as notification, based on subscription.

Inputs: -

Outputs: See clause 8.2.4.6.

See clause 8.2.2 for details of usage of this operation.

* * * Next Change * * * *

### 9.3.4	SS_ AADRF_Historical_serviceAPI_logs API

#### 9.3.4.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF for requesting service API logs and receiving the offline data/statistics on API logs.

#### 9.3.4.2	Get

API operation name:  Historical_ServiceAPI_Logs_Get

Description: The consumer requests API logs from A-ADRF.

Inputs: See clause 8.6.3.4.

Outputs: See clause 8.6.3.5.

See clause 8.6.2 for details of usage of this operation.

* * * Next Change * * * *

### 9.3.5	SS_ AADRF_NetworkSlice_data API

#### 9.3.5.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF for requesting and receiving network slice data.

#### 9.3.5.2	Get

API operation name:  NetworkSlice_Data_Get

Description: The consumer requests network slice data from A-ADRF.

Inputs: See clause 8.7.3.5.

Outputs: See clause 8.7.3.6.

See clause 8.7.2 for details of usage of this operation.

* * * Next Change * * * *

### 9.3.6	SS_AADRF_EdgeData_Collection API

#### 9.3.6.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF for subscribing for edge data collection and for getting notified about the offline data/statistics for the EDN and/or EAS/EES.

#### 9.3.6.2	Subscribe

API operation name: EdgeData_Collection_Subscribe

Description: The consumer subscribes for offline edge data from A-ADRF.

Inputs: See clause 8.8.3.4.

Outputs: See clause 8.8.3.5.

See clause 8.8.2 for details of usage of this operation.

#### 9.3.6.3	Notify

API operation name:  EdgeData_Collection_Notify

Description: The consumer is receiving the offline edge data from A-ADRF as notification, based on subscription.

Inputs: -

Outputs: See clause 8.8.3.6.

See clause 8.8.2 for details of usage of this operation.

* * * Next Change * * * *

### 9.3.x	SS_AADRF_Location_Accuracy API

#### 9.3.x.1	General

API description: This API enables the ADAE server to communicate with the A-ADRF to request location analytics/data for VAL UEs or VAL service area.

#### 9.3.x.2	Get

API operation name: Location_Accuracy_Data_Get

Description: The consumer is receiving offline location analytics/data from A-ADRF.

Inputs: See clause 8.5.3.4.

Outputs: See clause 8.5.3.5.

See clause 8.5.2 for details of usage of this operation.

* * * End of changes * * * *