3GPP TSG-SA WG6 Meeting #56	S6-232397

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0225 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct location monitoring verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, location mismatch is reported by LM server when: the NEF reported Area of Interest monitoring result does match with the determined VAL UE location status (present/absence) in LM server. This logic is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The location mismatch is reported to VAL server when the LM server determines UE location status does not match with the NEF reported AoI result. Adding step 6c in Figure 9.3.11.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect logic in location verification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The LM server authorized to consume the 3GPP core network service (Monitoring events provided by NEF as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to LM server including VAL UE Identifier, predetermined area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LM server regarding VAL UE's presence in a given area.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters.

-	"Notify_Interval" represents the periodic interval in which the LM server needs to notify VAL UE's location information to the VAL server. When the VAL UE moves away from the "Area of interest", then the LM server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

2.	LM server processes the Area of interest information in the request, and then subscribes to UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the LM server receives the VAL UE location information periodically from the 3GPP core network. The LM server also invokes NEF service for Area of Interest monitoring as specified in clause 5.3.3 of 3GPP TS 23.256 [52] to know whether the VAL UE is in the VAL server’s predetermined area of interest.

3.	LM server shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the VAL UE location information. Based on the geographic information from the VAL server, the LM server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4.	LMS may periodically obtain the VAL UE location information from the 3rd party location management server based on the geographic information from the VAL server.

NOTE 1:	How the LMS obtains the UE location from the 3rd party location management server is out of scope of this specification.

5.	LM server, after successful subscription according to steps 2~4, sends Monitor Location Subscription response, indicating that the LM server accepts VAL server's request and will monitor the location of the VAL UE to verify if the VAL UE is in the area of interest.

6a and 6b. LM server processes the location information received from SEAL Location Information procedures, the 3GPP core network and/or the 3rd party location management server, and validates the information. If the location information is matching, then the LM server shall check if the VAL UE's current location is within the predetermined area of interest received in step 1. The LM server will continue with step 7, step 8 and step 9 as applicable.

NOTE 2:	The location information received from the 3GPP core network and from LM client does not necessarily be exactly the same, the LM server can aggregate the location information if there is no big deviation.

6c.	LM server verifies its determined VAL UE location status (i.e. presence or absence with regard to the predetermined area of interest) with the retrieved 3GPP core network result from the NEF monitoring service for Area of Interest.

7.	If the location information received from Location management client, the 3GPP core network and/or the 3rd party location management server do not match or the NEF reported Area of Interest monitoring result does not match with the determined VAL UE location status (present/absence) in LM server, then the LM server shall consider the VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including VAL UE ID and the location information from LM server,the 3GPP core network and/or the 3rd party location management server in the notification message.

8.	If the VAL UE's current location is from Location management client, the 3GPP core network and/or the 3rd party location management server matches, and not in the area of interest received from VAL server in Monitor Location Subscription Request message, and such out of area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server considers the VAL UE as outside from its specified area of interest and shall notify the VAL server that the VAL UE's current location is outside of area of interest and VAL UE ID in "Notify Absence" message.

9.	When the VAL UE's current location is in area of interest, and such in area of interest result is also aligned with the NEF reported area of interest monitoring result, then the LM server shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the VAL UE is within the area of interest, along with VAL UE's current location information.

* * * End of changes * * * *