3GPP TSG-SA WG6 Meeting #56	S6-232400

# Goteborg, Sweden, 21st August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0227 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service area triggering criteria in the SS_LocationReporting API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The received LS S6-231661 asking SA6 to clarify the VAL service area based triggering criteria. This CR proposes to specify the VAL service area based triggering criteria in explicit form to address the questions in the received LS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the VAL service area based triggering criteria in the SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of the VAL service area functionality cannot be completed in the stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1, 9.3.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR addresses LS S6-231661. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

The SS_LocationReporting API does not specify clearly the triggering criteria for the location reporting based on the VAL service area.

This CR and the corresponding LS reply propose to specify a flexible VAL service area triggering criteria that supports both periodic and event-triggered location reporting.

We would like to discuss the following key aspects of the VAL service area triggering criteria:

- the triggering criteria enable the reporting inside and outside the locations defined in the VAL service area (the possibility of the UE location monitoring inside and outside the locations defined in the VAL service area dramatically improves the application usability, i.e., the application does not need to create and synchronize any changes between definition two VAL service areas that represent the locations inside and outside);

- the triggering criteria support a custom time range for the location reporting, e.g., the reporting shall occur each working day during the working hours.

- the triggering criteria support a configurable granularity of the location change conditions, e.g., cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates.

The VAL service area based triggering criteria with the key aspects listed above enable a wide variety of the use cases of the SS_LocationReporting API. For example:

- tracking assert in case of theft or inproper use: the factory owner would like to track the GPS coordinates (aspect 3) of a factory loader outside (aspect 1) the factory area in non-working hours (e.g., after 5:00 PM during working days and weekends) (aspect 2).

- the HR system would like to receive an indication if a worker is outside of the factory area (aspect 1) at 8:00 AM (aspect 2).

- the security team wants to receive a periodical report of the VAL user/UE location that is inside the forbidden areas identified by the VAL service area.

23.273 18.2.0 defines the reporting indication with event reporting allows the reporting outside the geographical location, i.e., clause 5.1 states, the event report allowed area can also be used differently if an area usage indication is provided together with the power saving area by the UE. A reporting indication that is determined by GMLC based on the area usage indication means when the UE detects the triggered or periodic event happens, if it is outside the event report allowed area, the UE is allowed to generate and send the event report to network to reduce UE power consumption.

To avoid non-efficient energy consumption at VAL UE side, an addititonal indication of the time range is added to the location reporting configuration response, i.e., the VAL UE/VAL Group shall report the location during the given time periods to reduce energy consumption and prolong the productive time of the VAL UE.

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Requested location access type (see NOTE 2) | O (see NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested positioning method | O (see NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| > Time range | O | It indicates the requested time intervals when the reporting shall occur, e.g., days of the week and/or time period for the location reporting. |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE 1: If none of the information element is present, this represents a cancellation for location reporting. NOTE 2: The non-3GPP access as defined in 23.273[50] and TS 29.572 [51] is out of scope of the present specification. |  |  |

* * * Next change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| > VAL service area ID | O (see NOTE 3) | Identifies the VAL service area ID. |
| > VAL service area based triggering criteria | O (see NOTE 3) | Identifies the VAL service area based triggering criteria. If VAL service area ID is provided, the VAL service area based triggering criteria shall be provided. |
| >> Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered. |
| >> Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| >> Location change condition | O | It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates. If the Frequency of reporting is event-triggered, this IE shall be provided. |
| >> Inside or outside indication | O | It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations defined in the VAL service area. |
| >> Time range | O | It indicates the requested time intervals when the reporting shall occur, e.g., days of the week and/or time period for the location reporting. |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. NOTE 3: This information element may be provided when this information flow is from VAL server to the location management server. |  |  |

* * * End of changes * * * *