3GPP TSG-SA WG6 Meeting #56	S6-232403

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0414 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Initial SIP registration for MC service clients behind MC GW UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The initial SIP registration for MC service client hosted by an MC service UE can be done during PDN establishment towards the MC services APN. However, initial SIP registration for the case of MC service client hosted by a non-3GPP device is not clearly described. Therefore, this CR provides clarification related to initial SIP registration for MC service clients hosted by non-3GPPdevices. For this case, the MC gateway UE establishes a PDN connection towards the MC services APN on behalf of the MC service client behind it, and requests initial SIP registration on its behalf. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a paragraph related to initial SIP registration for MC service clients behind MC gateway UE (i.e., hosted by non-3GPP devices). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Initial SIP registration for MC service clients behind MC gateway UE is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.2.7.2.1	Considerations for the EPS bearer to the MC services PDN

If the PDN connection established during the initial attach by the MC service UE is to an APN other than the MC services APN, then prior to user authentication, the MC service UE shall establish another PDN connection to the MC services APN. PDN connection establishment can also be caused by a SIP registration request for one or more MC services.

In the case of MC gateway UE, which supports MC service clients hosted by non-3GPP devices behind it, the MC gateway UE establishes a PDN connection to the MC services APN on behalf of the MC service clients behind it, and their initial SIP registration is requested via the MC gateway UE.

The QCI value of 69 (as specified in 3GPP TS 23.203 [8]) shall be used for the EPS bearer that transports SIP-1 reference point messaging.

# * * * * First change * * * *