3GPP TSG-SA WG6 Meeting #56	S6-232415

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0419 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IE related to resources management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current resources management procedures described in clause 10.11 do not include any information element to describe the information sent with the messages and requests. Therefore, this CR introduces a new clause to include the information elements associated with the procedures in clause 10.11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - rename clause 10.11.1 to be 10.11.0 - adding clause 10.11.1 including the information elements associated with the procedures in clauses 10.11.2 – 10.11.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information and procedures related to resources management are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11.1, 10.11.1.1. – 10.11.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.11.0	General

The following subclauses specify the procedures for resource management for mission critical services. The procedures are utilised by the following MC services:

-	MCPTT (as specified in 3GPP TS 23.379 [16]);

-	MCVideo (as specified in 3GPP TS 23.281 [12]); and

-	MCData (as specified in 3GPP TS 23.282 [13]).

The MC service server sets up media bearers and may need to modify the bearers for an already established MC communication.

Media characteristics that may need to be modified include:

-	activation and deactivation of the bearer;

-	modification of the QoS characteristics of the bearer (e.g. bearer priority adjustment); and

-	modification of GBR due to media codec change

NOTE 1:	A group call can consist of both unicast and multicast bearers which can all need modification due to the same event.

NOTE 2:	The MC service server can retry the establishment of a media bearer upon failure based on information received, or other conditions.

# * * * * Second change * * * *

### 10.11.1	Information flows

#### 10.11.1.1	Session establishment request

Table 10.11.1.1-1 describes the session establishment request sent from the MC service client to MC service server.

Table 10.11.1.1-1: Session establishment request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service client sending the request |
| SDP offer | M | Requested SDP, including among others, network resources type (e.g., bandwidth, media type), and resources access details of the MC service client and media anchoring service (e.g., IP address and ports) |
| Priority information | M | Indicate the priority of the established session |

#### 10.11.1.2	Session progress request

Table 10.11.1.2-1 describes the session progress request sent from the MC service server to SIP core, and from SIP core to MC service client.

Table 10.11.1.2-1: Session progress request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service client associated to the requested session |
| SDP answer | M | Requested SDP, including among others, network resources type (e.g., bandwidth, media type), and resources access details of the MC service client and media anchoring service (e.g., IP address and ports) |
| Priority information | M | Indicate the priority of the session |

#### 10.11.1.3	Session establishment acknowledgement

Table 10.11.1.3-1 describes the session establishment acknowledgment sent from the MC service client to MC service server.

Table 10.11.1.3-1: Session establishment acknowledgment

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service client sending the request |
| Session ID | M | Identity of the session to be established |
| Final negotiated service information | M | Final negotiated service information, e.g., resources access details of the MC service client and media anchoring service (e.g., IP address and ports) |

#### 10.11.1.4	Session update request

Table 10.11.1.4-1 describes the session update request sent from the MC service server to SIP core, and from SIP core to MC service client.

Table 10.11.1.4-1: Session update request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service client associated to the requested session |
| Session ID | M | Identity of the session to be updated |
| Session related aspects to be updated | M | Session aspects to be updated, such as details related to SDP or priority |

#### 10.11.1.5	Session establishment response

Table 10.11.1.5.-1 describes the session establishment response sent from the SIP core to the MC service client.

Table 10.11.1.5-1: Session establishment response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service client associated to the requested session |
| Session ID | M | Identity of the session to be established |
| Final negotiated service information | M | Final negotiated service information, e.g., resources access details of the MC service client and media anchoring service (e.g., IP address and ports) |

#### 10.11.1.6	Session release request

Table 10.11.1.6.-1 describes the session release request sent from the MC service server to the EPS (PCRF).

Table 10.11.1.6-1: Session release request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | Identity of the session to be released |

#### 10.11.1.7	Session release response

Table 10.11.1.7.-1 describes the session release request sent from the EPS (PCRF) to the MC service server.

Table 10.11.1.7-1: Session release response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | Identity of the session to be established |
| Result | M | Successful or failure of session release |

# * * * * End of changes * * * *