3GPP TSG-SA WG6 Meeting #56	S6-232416

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0078 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add VAE server API in clause 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the VAE server APIs corresponding to clause 9.20, 9.21 and 9.22 are missing. This should be added so that the CT3 can work on the server API in stage-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new APIs related to procedures in clauses 9.20-9.22 in clause 10, and update the API table accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The server APIs for the Rel-18 features are not specified and the downstream group cannot start their work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.1, (new)10.2.x, (new)10.2.x.1, (new)10.2.x.2, (new)10.2.x.3, 10.2.y, (new)10.2.y.1, (new)10.2.y.2, (new)10.2.y.3, 10.2.z, (new)10.2.z.1, (new)10.2.z.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /****************************** 1st change ********************/

## 10.2	VAE server APIs

### 10.2.1	General

Table 10.2.1-1 illustrates the VAE server APIs.

Table 10.2.1-1: List of VAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| VAE_MessageDelivery API | Deliver_DL_Message | V2X application specific server | Request/ Response |
|  | Deliver_UL_Message | V2X application specific server | Subscribe/notify |
| VAE_FileDistribution API | Distribute_File | V2X application specific server | Request/ Response |
| VAE_ApplicationRequirement API | Reserve_NetworkResource | V2X application specific server | Subscribe/notify |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup API | Configure_DynamicGroup | V2X application specific server | Request/Response |
|  | Notify_DynamicGroup |  |  |
| VAE_ServiceContinuity API | Query_ServiceContinuity | VAE server | Request/Response |
| VAE_HDMapDynamicInfo API | Subscribe_HDMapDynamicInfo | V2X application specific server | Subscribe/notify |
|  | Notify_HDMapDynamicInfo |  |  |
| VAE_SessionOrientedService API | Establish_Session | V2X application specific server | Subscribe/notify |
|  | Notify_Establish_Session |  |  |
|  | Update_Session |  |  |
|  | Notify_Update_Session |  |  |
|  | Terminate_Session |  |  |
|  | Notify_Terminate_Session |  |  |
| VAE_PC5ProvisioningRequirement API | Config_PC5ProvisioningRequirement | V2X application specific server | Subscribe/notify |
|  | Notify_PC5ProvisioningRequirement |  |  |
| VAE_V2VConfigRequirement API | Request_V2VConfigRequirement | V2X application specific server | Request/Response |
| VAE_ServiceAdaptationQoSControlInfo API _ | Subscribe_ ServiceandQoSControlInfo | V2X application specific server | Subscribe/notify |
|  | Notify_ ServicandQoSControlInfo |  |  |
| VAE_ VRUzoneManagement API | Subscribe_VRUzoneManagement operation | V2X application specific server | Subscribe/notify |
|  | Notify_ Enter/Leave |  |  |
| VAE_V2PApplicationRequirement API | Request_ V2PApplicationRequirement | V2X application specific server, V2X application specific client | Request/Response |

The SEAL server(s) APIs available via SEAL-S reference point as specified in 3GPP TS 23.434 [6] are consumed by the V2X application specific server via the VAE server (acting as a VAL server) over the Vs reference point. When V2X application specific server invokes a SEAL server API via the VAE server over the Vs reference point, the VAE server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response.

# /****************************** 2nd change ********************/

### 10.2.x	VAE_ ServiceAndQoSControlInfo API

#### 10.2.x.1	General

API description: This API enables the V2X application specific server to communicate with the VAE server to subscribe and receive notifications for service adaptation and QoS control information.

#### 10.2.x.2	Subscribe_ ServiceAndQoSControlInfo operation

API operation name: Subscribe_ ServiceAndQoSControlInfo

Description: Subscribes for the service adaptation and QoS control information at the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.20.2.1.

Outputs: Refer subclause 9.20.2.2.

See subclause 9.20.3 for the details of usage of this API operation.

This service operation also enables to update/delete an existing subscription at the VAE server. For the update case, the parameters that may be updated are V2X group ID, V2X service ID, and V2X UE ID.

#### 10.2.x.3	Notify_ ServiceAndQoSControlInfo operation

API operation name: Notify_ ServiceAndQoSControlInfo

Description: Notify the service adaptation and QoS control information at the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.20.2.3.

Outputs: None.

See subclause 9.20.4 for the details of usage of this API operation.

### 10.2.y	VAE_ VRUzoneManagement API

#### 10.2.y.1	General

API description: This API enables the V2X application specific server to communicate with the VAE server to subscribe and receive notifications for VRU zone management.

#### 10.2.y.2	Subscribe_VRUzoneManagement operation

API operation name: Subscribe_VRUzoneManagement

Description: Subscribes for the VRU zone management / configuration at the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.21.2.1.

Outputs: Refer subclause 9.21.2.2.

See subclause 9.21.3 for the details of usage of this API operation.

#### 10.2.y.3	Notify_ Enter/Leave operation

API operation name: Notify_ Enter/Leave

Description: Notify the entry or leave of the V2X UE to / from the VRU zone area, based on the subscription.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.21.2.4.

Outputs: None.

See subclause 9.21.3 for the details of usage of this API operation.

### 10.2.z	VAE_ V2PApplicationRequirement

#### 10.2.z.1	General

API description: This API enables the V2X application specific server or the V2X application specific client to communicate with the VAE server to provide a requirement to manage the support functionality for enabling V2P applications.

#### 10.2.z.2	Request_V2PApplicationRequirement

API operation name: Request_V2PApplicationRequirement

Description: The V2X application specific server or client provides a V2P application requirement request to the VAE server to provide the support functionality for enabling V2P applications.

Known Consumers: V2X application specific server, V2X application specific client.

Inputs: Refer subclause 9.22.2.1.

Outputs: Refer subclause 9.22.2.2.

See subclause 9.22.3.1 and 9.22.3.2 for the details of usage of this API operation.