3GPP TSG-SA WG6 Meeting #55	S6-232442

Berlin, 22th – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0425 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on connection authorisation of an MC gateway client hosted by an MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 11.5.1.3.3 Connection authorisation for non-3GPP devices that do not host an MC client, this procedure requires the MC gateway client is also within the MC gateway UE as indicated in figure 11.5.1.3.3-1 and the descriptions. However, in figure 11.2.1-1 functional model, there is no such MC gateway client in the MC gateway UE. Such misalignment makes the following text in 11.5.1.3 ambiguous (i.e., may indicate the MC gateway client hosted by the MG gateway UE may connect to another MC gateway UE). The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Add the MC gateway client in the MC gateway UE as an optional box to align with clause 11.5.1.3 2) Update the above text from 11.5.1.2.1, 11.5.3.1, 11.5.1.3.3, 11.5.4.2.1, 11.5.4.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion and misunderstandings will be caused.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1, 11.5.1.2.1, 11.5.3.1, 11.5.1.3.3, 11.5.4.2.1, 11.5.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

### 11.2.1	Functional model

For the application-level signalling necessary for the association between the MC client on the non-3GPP device and the MC gateway UE reference point Gateway-local (GW-local) is used. The MC gateway UE uses the Gateway-Core (GW-Core) reference point towards the MC service server to announce the association between MC gateway client and MC gateway UE server. Once the use of the MC gateway UE has been authenticated and authorized by the corresponding MC service server, the MC service user is authorized MC gateway user.

The MC gateway UE sever enables the following functions:

-	Authentication of the MC gateway clients including the MC gateway UE function of the corresponding MC service server

-	The following functions are available when the connection authorization of the MC gateway client is successful:

-	Relay of signalling between MC client and MC service server/CSC server

-	Relay of signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	Relay of signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media plane forwarding between the MC service server and the MC clients

The MC gateway UE function as part of the corresponding MC service server enables the use of an MC gateway UE with a corresponding MC gateway UE function that authorizes the association between the MC gateway client/MC client and the corresponding MC gateway UE to enable forwarding of the media plane to MC clients via the MC gateway UE.

MC gateway UE

MC client hosted by a non-3GPP device

Common Service Core

GW-local

CSC client

CSC server

CSC-n

CSC-n

CSC-n

Signalling user agent

Signalling user agent

SIP Core

SIP-1

SIP-1

SIP-1

HTTP proxy

HTTP client

HTTP client

HTTP-1

HTTP-1

HTTP-1

MC gateway UE

server

MC service servers

MC client

MCX-n

MC gateway client

MCX-n

GW-Core

MC gateway UE function

MC service clients

MCX-n

MC gateway client

Figure 11.2.1-1: Functional model of MC gateway UE signalling plane

# /********************Next Changes ********************/

##### 11.5.1.2.1	General

The solution is applied to non-3GPP devices which can host an MC client and the MC gateway client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC gateway client is authorized and controlled by the MC server. The MC gateway client informs MC clients about the connection status.

For the period of association between MC server, MC gateway client and MC gateway UE, the MC server maintains the assignment between MC clients to the MC gateway UE used. This assignment is cancelled again with the disconnection.

# /********************Next Changes ********************/

##### 11.5.1.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client and the MC gateway client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC client is authorized and controlled by the MC server.

NOTE:	The interworking between the MC gateway client hosted at the MC gateway UE and an MC service user is out of scope of the present document, nevertheless, the connection authorisation performed by the MC gateway UE shall enable the non-3GPP devices to get the access to MC services requested by the service user.

# /********************Next Changes ********************/

##### 11.5.1.3.3	Connection authorisation procedure

The procedure for connection authorisation of an MC gateway client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device, where the MC gateway client and MC clients are hosted by the MC gateway UE.

-	The MC gateway client has selected an MC gateway UE or alternatively, the non-3GPP has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

-	The MC gateway client, which is hosted by the MC gateway UE, has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

Figure 11.5.1.3.3-1: Connection authorisation of an MC gateway client hosted by an MC gateway UE

1.	The MC client, hosted by the MC gateway UE, requests connection authorization with an MC server by providing the GW MC service ID. The MC gateway UE sends the connection authorization request to the MC server.

2.	The MC server performs an authorization check, to verify that access using the MC gateway UE is permitted. An MC server shall reject the connection authorization when the MC server receives connection authorization from a MC gateway client for a particular MC service for which the connection already exists with the same or different MC gateway UE.

3.	The MC server sends the connection authorization response to the MC gateway client residing on the MC gateway UE.

The MC gateway client has now access to the MC server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE hosting the MC gateway client for the new MC service, if multiple MC gateway UEs are available.

# /******************** Next Changes ********************/

##### 11.5.4.2.1	General

The solution is applied to non-3GPP devices which can host an MC client and the MC gateway client. The MC gateway UE forwards the disconnection request to the corresponding MC server to disconnect the MC gateway UE to MC client connection.

# /******************** Next Changes ********************/

##### 11.5.4.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client and the MC gateway client. The MC server is requested to disconnect the MC gateway UE to MC client connection on demand.