3GPP TSG-SA WG6 Meeting #56	S6-232449

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0106 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of multicast MBS data reception in RRC inactive state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, the multicast MBS data reception in RRC inactive state is introduced at the network layer to enable simultaneous reception of MBS session data for a higher number of UEs in a cell than can be operating in RRC_CONNECTED state, to participate in public safety group calls using MBS-based service. However, due to timeline issue among RAN, SA2, SA6, this feature is not supported in rel-18/TS23.289. This feature enhances the 5GC-AF interactions, which requires the AF/AS to provide the MBS assistance information indicating that a UE is preferred to be kept connected when the related MBS Session the UE joined is active. The MC service server acting as the AF needs to be enhanced to support this feature: - The MC service server needs to select the UE(s) being preferred to be kept connected when the related MBS Session the UE joined is active; SA2 has some instructions in clause 7.2.9a of 23.247 as follows: NOTE 2: The AF can provide the MBS Session assistance information for the UEs based on observed or expected UE data transmission behaviour, e.g., a frequent talker and/or the group leader of a multicast group represented by an MBS Session ID. - After the AF obtains the MBS Session ID of a multicast MBS Session the MC service server needs to determine when to provison such MBS assistance information to the 5GC network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (1) Update the description in clause 4.7.1 to allow the UE to receive the multicast MBS data in RRC-INACTIVE state. (2) Introduce a new clause to cover the support of multicast MBS data reception in RRC inactive state including the following aspects: a. the selection of UE is preferred to be kept connected when the related MBS Session the UE joined is active. b. when to provide the MBS assistance information to the 5GC (e.g., UDM via the NEF). (3) Update the SIP registration to allow the UE to report the reception capability of multicast data in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The multicast MBS data reception in RRC inactive state can not be used by MC service and higher number of UEs in a cell to receive multicast MBS data is not well supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1, 5.3.3.3.2, 5.3.3.3.3, (new)7.3.x, (new)7.3.x.1, (new)7.3.x.2, (new)7.3.x.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /****************************** First changes **************************/

### 4.7.1	General

The main purpose of 5G Multicast-Broadcast Service (MBS) use by mission critical services is to provide efficient downlink delivery of user traffic in group calls and communications. The architectural figures in this clause are aligned  with the 5GS architecture for MBS shown in Figure 5.1-2 of 3GPP TS 23.247 [15], which identifies both mandatory and optional functional entities and interfaces, in reference point representation, available for use by the MC services.

Multicast and broadcast communication services in 5G for MC group communications rely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the MC service server to multiple MC users (i.e., users affiliated to a certain MC group) is supported either as point-to-point or point-to-multipoint over the radio. The MBS sessions are either broadcast or multicast type and consist of one or multiple QoS flows for different service requirements. For the broadcast MBS session or local MBS session, the MBS service area is configured with the MBS session.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the MC service server decides whether to create broadcast or multicast MBS sessions to be associated with certain MC groups. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions only. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the MC service UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED and RRC-INACTIVE state.

The following capabilities (non-exhaustive list) provided by MBS could be used by MC service servers:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Transparent MBS Data forwarding;

-	Dynamic PCC control for MBS session;

-	UE’s MBS assistance information provision.

# /****************************** Second changes **************************/

##### 5.3.3.3.2	Reference point SIP-1(between the signalling user agent and the SIP core)

The SIP-1 reference point, which exists between the signalling user agent and the SIP core for establishing a session in support of MC service, shall use the Gm reference point as defined in 3GPP TS 23.002 [8] (with necessary enhancements to support MC service requirements and profiled to meet the minimum requirements for support of MC services). The SIP-1 reference point fulfils the requirements of the 5G-GC1 reference point, and is used for:

-	SIP registration (including the UE's capabilities, for example MBMS capable, or MBS capable UE, capability to receive multicast MBS data in RRC_INACTIVE state);

-	authentication and security to the service layer;

-	event subscription and event notification;

-	overload control;

-	MBS session management; e.g., MBS session announcement and de-announcement; and

-	media negotiation.

NOTE 1:	The reference point 5G-GC1 is defined within SA6, and outside of SA2 scope.

NOTE 2:	Information related to the UE´s MBS/MBMS capabilities can be exchanged during either registration or service authorization procedure.

##### 5.3.3.3.3	Reference point SIP-2 (between the SIP core and the SIP AS)

The SIP-2 reference point, which exists between the SIP core and the SIP AS for establishing a session in support of MC service, shall use the ISC and Ma reference points as defined in 3GPP TS 23.002 [8]. The SIP-2 reference point is used for:

-	notification to the MC service server(s) of SIP registration (including the UE's capabilities, for example MBMS capable, or MBS capable UE, capability to receive multicast MBS data in RRC_INACTIVE state) by the MC service UE;

-	authentication and security to the service layer;

-	event subscription and event notification;

-	session management; and

-	media negotiation.

NOTE:	Information related to the UE´s MBS/MBMS capabilities can be exchanged during either registration or service authorization procedure.

# /****************************** Third changes **************************/

### 7.3.x	Support of Multicast MBS session data reception in UE with RRC_INACTIVE state

#### 7.3.x.1	General

To enable simultaneous reception of MBS session data for a higher number of UEs in a cell than can be operating in RRC_CONNECTED state, to participate in public safety group calls using MBS-based service, the MC service server may provision the MBS assistance information indicating that a UE is preferred to be kept connected when the related MBS session the UE joined is active.

#### 7.3.x.2	Selection of UEs preferred to receive multicast data in RRC_CONNECTED state

Receive multicast data in RRC_INACTIVE state may not have the same experience as receiving in RRC_CONNECTED state, due to the HARQ feedback and PTP transmission are not supported and seamless/lossless mobility is not required for multicast reception in RRC_INACTIVE, interruptions and data loss during state transition between RRC_INACTIVE and RRC_CONNECT may occur. So the frequent talker, the critical member, or the UE suffering bad receptions quality is more appropriate to be kept in connected to receive the multicast MBS data.

The MC service server determines a UE is preferred to be kept connected to receive the multicast MBS data and provision the corresponding MBS assistance information (i.e., MBS session ID(s)) to the 5GC. The MC service server may select such UE considering both the static characteristics (e.g., UE’s capability to receiving multicast MBS data in RRC_INACTIVE state in the group, user’s role (e.g., commander)) and dynamic information (e.g., UE location, floor request, the number of simultaneous joined MBS session, number of group communications over multicast MBS session, receiption quality of the multicast MBS session) of the UE(s) in one or more group where the multicast MBS session is to be used/used.

The MC service server may prefer to select a UE that is preferred to be kept connected to receive the multicast MBS data e.g., the UE is only capable of receiving multicast MBS data in RRC_CONNECTED state in a group, requesting the floor, bad receiption quallity of the mutlicast MBS session, near the incident scene, several simultaneous joined MBS session/group communication.

NOTE:	The above information is optional input for the UE selection the MC service server, the final algorithm is implementation specific.

#### 7.3.x.3	Provisioning of the MBS assistance information

After the MC service server obtains the MBS Session ID of a multicast MBS Session via the TMGI allocation or the MBS session creation procedure, and the group and MBS session mapping is determined, the MC service server may provision the MBS assistance information of a UE to the 5GC as described in 3GPP TS 23.247 [15].

Before the MBS session is deleted, such provisioning may be performed and updated. If the MBS session ID(s) are well planned and statically assigned to the groups, and UEs preferred to receive multicast data in RRC_CONNECTED state is determined based on static information (e.g., user’s role in the group), the MC service server may provision the UE’s MBS assistance information before the group call setup. If the MBS session is dynamically assigned to a group call, or dynamic information (e.g., location or floor requset) is considered to determine the UEs preferred to receive multicast data in RRC_CONNECTED state, such provisioning may be deferred after the group call setup (e.g., after the MapGroupToSessionStream or even floor request).