3GPP TSG-SA WG6 Meeting #56	S6-232450

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0005 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the SEALDD server discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the SEALDD server discovery procedure in clause 9.4.3.2.3, the EES can identify the VAL server and the associated SEALDD server based on the performance of VAL server and the performance of SEALDD server. However, there is no clear description of what the specific performances are collected from ADAE server. From the role of SEALDD server and VAL server, the SEALDD server provides the generic 5G communication funcationality, including the storage service and data/signalling tranmission, and the VAL server is responsible for application logic processing, such as the image rendering. Therefore, the performance of SEALDD server can be represented as the transmission quality information (e.g. E2E latency), and the performance of VAL server can be identified by the computing capability information (e.g. VAL server load). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add the related descriptions for the performances of SEALDD server and VAL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no clear descriptions about what the specific performances are collected from ADAE server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.4.3.2.3	VAL server and SEALDD server registered to EES

Figure 9.4.3.2.3-1: VAL server and SEALDD server registered to EES

1. The VAL server (acting as an EAS) registers to the EES.

2. The SEALDD server (acting as an EAS) registers to the EES.

3. The EEC performs service provisioning procedure as described in clause 8.3.3.2.2 of 3GPP TS 23.558 [10]. The EEC is responded with the EES which supports the SEALDD server and the VAL server. The EEC sends the EAS discovery request for EAS bundle including VAL service and SEALDD service. The EES may collect the performance of VAL server (computing capability information, e.g. VAL server load) and the performance of SEALDD server (transmission quality information, e.g. E2E latency) from the ADAE server when receiving the EAS discovery request.

NOTE: It is assumed that the ADAE sever can collect load and transmission quality information from EAS.

4. The EES identifies the VAL server and the associated SEALDD server based on the performance of VAL server and the performance of SEALDD server (e.g. to satisfy the AC service KPI).

5. The EES sends the EAS discovery response to EEC, including the SEALDD server address, the VAL server address.

Upon receiving the SEALDD server address, the VAL server address, the VAL client can initiate SEALDD service via SEALDD client with the SEALDD server's address. The data transmission between the VAL client and the VAL server can be enabled via the SEALDD connection, as specified in clause 9.2.2.