3GPP TSG-SA WG6 Meeting #56	S6-232453

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0008 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the SEALDD policy termination procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the SEALDD policy configuration, the VAL server used for SEALDD policy configuration can dertermine to terminate the existing SEALDD policy that has been configured in SEALDD server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to add the SEALDD policy configuration termination procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing SEALDD policy configuration peocedure is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.2.3, 9.10.3.5, 9.10.3.6, 9.10.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.10	SEALDD policy configuration

9.10.1	General

The following clauses specify procedures, information flow for SEALDD policy configuration. In clause 9.10, the VAL server is a specific server for configurating SEALDD policy, and is different from the VAL server used for VAL application processing in other clauses.

9.10.2	Procedures

9.10.2.1	SEALDD policy configuration

Figure 9.10.2.1-1 illustrates the procedure for SEALDD policy configuration from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.1-1: SEALDD policy configuration

1.  The VAL server sends the SEALDD policy configuration request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE identify, and the SEALDD policy.

2.  The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to configure the SEALDD policy.

3.  Upon successful authorization, the SEALDD server stores the SEALDD policy for later use (e.g. for bandwidth control, transmission quality guarantee) and replies to the VAL server with the SEALDD policy configuration response.

9.10.2.2	SEALDD policy configuration update

Figure 9.10.2.2-1 illustrates the procedure for SEALDD policy configuration update from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.2-1: SEALDD policy configuration update

1.  The VAL server used for SEALDD policy configuration determines that the existing SEALDD policy needs to be updated, the VAL server sends the SEALDD policy configuration update request to the SEALDD server.

2. The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to update the SEALDD policy configuration.

3.  Upon successful authorization, the SEALDD server updates the SEALDD policy configuration and replies to the VAL server with the SEALDD policy configuration update response.

9.10.2.3	SEALDD policy configuration termination

Figure 9.10.2.3-1 illustrates the procedure for SEALDD policy configuration termination from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.3-1: SEALDD policy configuration termination

1.  The VAL server used for SEALDD policy configuration determines that the existing SEALDD policy needs to be terminated, the VAL server sends the SEALDD policy configuration delete request to the SEALDD server.

2. The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to delete the SEALDD policy configuration.

3.  Upon successful authorization, the SEALDD server deletes the SEALDD policy configuration and replies to the VAL server with the SEALDD policy configuration delete response.

9.10.3	Information flows

9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| VAL UE identity | O | Identifier of the VAL UE for which SEALDD policy applies |
| SEALDD policy | M | The SEALDD policy associated with application traffic identifiers, VAL UE identify |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the action to be performed when the measurement event (e.g. measurement threshold) occurs, in order to meet the quality guarantee. |
| > Bandwidth control policy | O (See NOTE 2) | The bandwidth control policy from VAL server, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. |  |  |

9.10.3.2	SEALDD policy configuration response

Table 9.10.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration.

Table 9.10.3.2-1: SEALDD policy configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Configuration ID | O (See NOTE) | Identifier of the SEALDD policy configuration. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |
| NOTE: These IEs are used for the successful case for SEALDD policy configuration request. |  |  |

9.10.3.3	SEALDD policy configuration update request

Table 9.10.3.3-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration update.

Table 9.10.3.3-1: SEALDD policy configuration update request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |
| Updated SEALDD policy | O | The updated SEALDD policy as described in Table 9.10.3.1-1. |

9.10.3.4	SEALDD policy configuration update response

Table 9.10.3.4-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration update.

Table 9.10.3.4-1: SEALDD policy configuration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured SEALDD policy |
| NOTE: This IEs is used for the successful case for SEALDD policy configuration update request. |  |  |

9.10.3.5	SEALDD policy configuration delete request

Table 9.10.3.5-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration termination.

Table 9.10.3.5-1: SEALDD policy configuration delete request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |

9.10.3.6	SEALDD policy configuration delete response

Table 9.10.3.6-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration termination.

Table 9.10.3.6-1: SEALDD policy configuration delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

9.10.4	APIs

9.10.4.1	General

Table 9.10.4.1-1 illustrates the APIs exposed by SEALDD server for SEALDD policy configuration.

Table 9.10.4.1-1: List of SEALDD server APIs for policy configuration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_PolicyConfiguration | Request | Request/Response | VAL server |
|  | Update |  |  |
|  | Delete |  |  |

9.10.4.2	Sdd_PolicyConfiguration operation

API operation name: Sdd_PolicyConfiguration_Request

Description: The consumer requests for one time for SEALDD policy configuration.

Inputs: See clause 9.10.3.1.

Outputs: See clause 9.10.3.2.

See clause 9.10.2.1 for details of usage of this operation.

9.10.4.3	Sdd_PolicyConfiguration update operation

API operation name: Sdd_PolicyConfiguration_Update

Description: The consumer requests for one time for SEALDD policy configuration update.

Inputs: See clause 9.10.3.3.

Outputs: See clause 9.10.3.4.

See clause 9.10.2.2 for details of usage of this operation.

9.10.4.4	Sdd_PolicyConfiguration termination operation

API operation name: Sdd_PolicyConfiguration_Delete

Description: The consumer requests for one time for SEALDD policy configuration termination.

Inputs: See clause 9.10.3.5.

Outputs: See clause 9.10.3.6.

See clause 9.10.2.3 for details of usage of this operation.