3GPP TSG-SA WG6 Meeting #56	S6-232463

Gothenburg, Sweden, 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0367 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revoke the floor by the authorized user in multi-talker case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Base on the SA1 requirement “[R-6.2.3.7.2-003] The MCPTT Service shall determine which Participant(s) are allowed to transmit to all other Participant(s) in an MCPTT Group configured for multi-talker control.” multipatipants are allowed to talk simultaneously. However, the authorized user may needs to “mute” certain participants due to e.g., too much background noise which imapct the other listeners of the group communication. How to stop the talker from transmitting to all participants is not support yet in TS 23.379. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure to allow the authorized user to revoke the floor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No means to stop the talker from transmitting to all the participants in a group communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)10.9.1.2.aa, (new)10.9.1.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

##### 10.9.1.2.aa	Multi-talker floor revoked

Table 10.9.1.2.aa-1 describes the information flow multi-talker floor revoked, from the floor participant to floor control server, which is used to revoke the floor from its current holder (the floor participant who was granted the floor).

Table 10.9.1.2.aa-1: Multi-talker floor revoked

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the user who requests to revoke the floor from another talker |
| MCPTT ID | M | Revoked party identity |
| Functional alias | O | Functional alias of the requester |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

# /************************ First change ******************************/

##### 10.9.1.3.x	Multi-talker floor revoke by the authorized user

Figure 10.9.1.3.x-1 shows the high level procedure that the floor granted to a multi-talker user is revoked by the authroized user. Only two UEs involved in the session are shown on the figure for the simplicity.

Pre-condition:

1.	The participant B is granted the floor and transmitting the voice media to the group.

Floor participant A

(Authorized user)

Floor control server

Floor participant B

(one of the multi-talkers)

1. Multi-talker Floor revoked

3. Floor revoked

2. Authorization check

MCPTT user at floor participant B is transmitting voice media to the group

4. Notification to the user

Figure 10.9.1.x.1-1: Floor revoke in multi-talker case

1.	The authorized user at floor participant A sends floor revoked message to revoke the floor of floor participant B. The identity of the floor participant B whose floor is to be revoked is included.

2.	The floor control server checks the floor participant A is authorized to perform the floor revoke.

3.	The floor control server sends the floor revoked message towards the floor participant B.

4.	The floor participant B notifies the MCPTT server about the floor is revoked.