3GPP TSG-SA WG6 Meeting #56	S6-232474

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0451 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for alignment with CAPIF deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Deployments in relation with CAPIF are specified in clause A.5. The deployment enables CAPIF-1/CAPIF-1e to be supported by EDGE-3 where EAS acts as API invoker. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the normative descriptions considering the deployment options in clause A.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CAPIF deployments specified in clause A.5 will not be implementable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.5.4, 6.5.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.3.2	Edge Enabler Server (EES)

EES provides supporting functions needed for EASs and EEC.

Functionalities of EES are:

a)	provisioning of configuration information to EEC, enabling exchange of application data traffic with the EAS;

b)	providing API invoker and API exposing functions as specified in 3GPP TS 23.222 [6]. May provide CAPIF core functions as specified in 3GPP TS 23.222 [6] to support deployments specified in clause A.5;

c)	interacting with 3GPP Core Network for accessing the capabilities of network functions either directly (e.g. via PCF) or indirectly (i.e. SCEF/NEF/SCEF+NEF);

d)	exposing events related to ACT;

e)	EEC context transfer between EESs;

f)	supporting external exposure of 3GPP network and service capabilities to the EAS(s) over EDGE-3;

g)	registration functions (i.e., registration, update, and de-registration) for the EEC(s) and the EAS(s);

h)	triggering the EAS instantiation on demand; and

i)	supporting ACR related operations (e.g. ACR launching, ACR information notification, EELManagedACR).

* * * Next Change * * * *

### 6.5.4	EDGE-3

EDGE-3 reference point enables interactions between the EES and the EASs. It supports:

a)	registration of EASs with availability information (e.g. time constraints, location constraints);

b)	de-registration of EASs from the EES;

c)	discovery of T-EAS information to support ACT;

d)	providing access to network capability information (e.g. location information);

e)	requesting the setup of a data session between AC and EAS with a specific QoS and receiving QoS related information;

f)	service continuity procedures (e.g. ACR status); and

g)	supporting interactions related to CAPIF-2/CAPIF-2e between API invoker and AEF as specified in 3GPP TS 23.222 [6]. May support CAPIF-1/CAPIF-1e between API invoker and CCF as specified in 3GPP TS 23.222 [6] to support deployments specified in clause A.5.

NOTE:	Optimized distribution of events across the EDGE-3 interface is out of scope of this specification.

* * * Next Change * * * *

### 6.5.10	EDGE-9

EDGE-9 reference point enables interactions between two EESs. EDGE-9 reference point may be provided between EES within different EDN (Figure 6.5.10-1) and within the same EDN (Figure 6.5.10-2).

Figure 6.5.10-1: Inter-EDN EDGE-9

Figure 6.5.10-2: Intra-EDN EDGE-9

EDGE-9 supports:

a)	discovery of T-EAS information to support ACR;

b)	EEC context relocation procedures;

c)	transparent transfer of the application context during EELManagedACR; and

d)	supporting interactions related to CAPIF-6/CAPIF-6e between CCFs as specified in 3GPP TS 23.222 [6] to support deployments specified in clause A.5.

* * * End of Changes * * * *