3GPP TSG-SA WG6 Meeting #56	S6-232489

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-231821)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0317 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Information flows and procedures to support ad hoc group standalone short data service using signalling control plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4FRMCS, MC_AHGC |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ad hoc group support was provided for the MCData communication which uses the session for communication and Ad hoc group support for standalone short data service using signalling control plane was not introduced. This CR is trying to introduce the Ad hoc group support for standalone short data service using signalling control plane. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the new clauses to provide the information flows and procedures for the Ad hoc group standalone short data service using signalling control plane involving single and multiple MCData systems. The procedure supports participant list determined by the MCData servers or provided by the MCData client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ad hoc group support for standalone short data service using signalling control plane procedures will be missing in single and multiple MCData systems where participants list determined by the MCData servers or provided by the MCData client. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New (7.17.A, 7.17.A.1, 7.17.A.1.1, 7.17.A.1.2, 7.17.A.1.3, 7.17.A.1.4, 7.17.A.1.5, 7.17.A.1.6, 7.17.A.2, 7.17.A.2.1, 7.17.A.3, 7.17.A.3.1, 7.17.A.3.2, 7.17.A.4, 7.17.A.4.1, 7.17.A.4.2) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.17.A	Ad hoc group standalone short data service using signalling control plane

#### 7.17.A.1	Information flows

##### 7.17.A.1.1	Ad hoc group standalone data transfer request (MCData client – MCData server)

Table 7.17.A.1.1-1 describes the information flow for the Ad hoc group standalone data transfer request sent from the MCData client to the MCData server.

Table 7.17.A.1.1-1: Ad hoc group standalone data transfer request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the request |
| Functional alias | O | The associated functional alias of the MCData user sending request. |
| Encryption supported | M | Indicates whether this ad hoc group data transfer support requires end-to-end encryption |
| MCData ID list (see NOTE 1, NOTE 2) | O | MCData IDs for the ad hoc group standalone short data service |
| Criteria for determining the participants (see NOTE 2) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the list of MCData users e.g., it can be a location based criteria to SDS data transfer in a particular area |
| Emergency indicator (see NOTE 3) | O | Indicates that the data transfer request is for emergency ad hoc group standalone short data service |
| Imminent peril indicator (see NOTE 3) | O | Indicates that the data transfer request is for imminent peril ad hoc group standalone short data service |
| Application identifier (see NOTE 4) | O | Identifies the application for which the data payload is intended (e.g. text string, port address, URI) |
| NOTE 1: This element is included only when the the ad hoc group standalone short data service initiating client sends the list of MCData users. NOTE 2: Only one of these information elements is present. NOTE 3: If used, only one of these information elements shall be present. NOTE 4: The application identifier shall be included only if the data transfer is for application consumption. |  |  |

##### 7.17.A.1.2	Ad hoc group standalone data transfer response (MCData server – MCData client)

Table 7.17.A.1.2-1 describes the information flow Ad hoc group standalone data transfer response from the MCData server to the MCData client. This response to provide the server assigned MCData ad hoc group ID and preconfigured group identity of preconfigured group from which the configurations to be used (e.g. security related information).

Table 7.17.A.1.2-1 Ad hoc group standalone data transfer response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending the request` |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group standalone short data service which is genarted and assigned by the MCData server. |
| Preconfigured MCData group ID (see NOTE) | O | Group identity whose configuration is to be applied for ad hoc group standalone short data service. |
| Result | M | Result of the ad hoc group standalone data transfer (success or failure) |
| NOTE: If end-to-end enrycption is required, then this element is included. |  |  |

##### 7.17.A.1.1	Ad hoc group standalone data request (MCData client – MCData server)

Table 7.17.A.1.1-1 describes the information flow for the Ad hoc group standalone data request sent from the MCData client to the MCData server.

Table 7.17.A.1.1-1: Ad hoc group standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID (see NOTE 1) | M | The MCData group ID to be associated with the ad hoc group standalone short data service |
| Preconfigured MCData group ID (see NOTE 2) | O | Group identity whose configuration is to be applied for ad hoc group standalone short data service. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Emergency indicator (see NOTE 3) | O | Indicates that the data request is for emergency ad hoc group standalone short data service |
| Imminent peril indicator (see NOTE 3) | O | Indicates that the data request is for imminent peril ad hoc group standalone short data service |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| MCData ID list (see NOTE 4) | O | The specified MCData users who should send a disposition notification message. |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE 5) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Payload | M | SDS content |
| NOTE 1: The MCData ad hoc group ID is determined prior to by using the Ad hoc group standalone data transfer request. NOTE 2: If end-to-end enrycption is required, then this element is included and the value is determined prior to by using the Ad hoc group standalone data transfer request. NOTE 3: If used, only one of these information elements shall be present. NOTE 4: If Disposition Type IE is not present, this IE shall not be present. If Disposition Type IE is present but this IE is not, which indicates that all receivers shall respond with disposition notification message. NOTE 5: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

##### 7.17.A.1.2	Ad hoc group standalone data request (MCData server – MCData client)

Table 7.17.A.1.2-1 describes the information flow for the Ad hoc group standalone data request sent from the MCData server to the MCData client.

Table 7.17.A.1.2-1: Ad hoc group standalone data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID associated with ad hoc group standalone short data service |
| Preconfigured MCData group ID (see NOTE 1) | O | Group identity whose configuration is to be applied for this ad hoc group standalone short data service. |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Emergency indicator (see NOTE 2) | O | Indicates that the data request is for emergency ad hoc group standalone short data service |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the data request is for imminent peril ad hoc group standalone short data service |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| Payload | M | SDS content |
| Time to live | O | Indicates how long the ad hoc group is persisted and members can respond to the data transfer. |
| NOTE 1: If end-to-end enrycption is required, then this element is included. NOTE 2: If used, only one of these information elements shall be present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the payload is for application consumption. |  |  |

##### 7.17.A.1.4	Ad hoc group standalone data response (MCData server – MCData client)

Table 7.17.A.1.4-1 describes the information flow Ad hoc group standalone data response from the MCData server to the MCData client.

Table 7.17.A.1.4-1 Ad hoc group standalone data response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group standalone short data service |
| Time to live | O | Indicates how long the ad hoc group is persisted and members can respond to the data transfer. |
| Result | M | Result of the ad hoc group standalone short data service (success or failure) |

##### 7.17.A.1.5	Ad hoc group standalone data get userlist (MCData server – MCData server)

Table 7.17.A.1.5-1 describes the information flow Ad hoc group standalone data get userlist from one MCData server to another MCData server.

Table 7.17.A.1.5-1: Ad hoc group standalone data get userlist

| Information element | Status | Description |
| --- | --- | --- |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group standalone short data service |
| Criteria for determining the participants | M | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the list of MCData users e.g., it can be a location based criteria to SDS data transfer in a particular area |

##### 7.17.A.1.6	Ad hoc group standalone data get userlist response (MCData server – MCData server)

Table 7.17.A.1.6-1 describes the information flow Ad hoc group standalone data get userlist response from one MCData server to another MCData server.

Table 7.17.A.1.6-1: Ad hoc group standalone data get userlist response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group standalone short data service |
| MCData ID list | M | List of MCData IDs meeting the criteria specified in the Ad hoc group standalone data get userlist |

* * * Next Change * * * *

#### 7.17.A.2	Ad hoc group standalone short data service using signalling control plane common procedure

##### 7.17.A.2.1	Procedure to determine the ad hoc group and preconfigured group for end-to-end encryption

The procedure in figure 7.17.A.2.1-1 describes about determining the MCData ad hoc group ID and preconfigured group identity of preconfigured group from which the configurations to be used (e.g. security related information).

Pre-conditions:

1.	The MCData users belong to primary MCData system and partner MCData system and already registered for receiving MCData service.

2.	The authorized MCData user at MCData client 1 wants to invite MCData users for the ad hoc group standalone short data service either by providing the list of MCData users in the request or by providing the certain criteria in the request to determine the MCData users who are satisying certain criteria.

NOTE 1:	Selection of MCData IDs of the ad hoc group members receiving the SDS data can be manual or from the user profile configuration data or by any other means. This is left for the implementation.

3.	Number of ad hoc group members receiving the SDS data should be within the configured limit.

Figure 7.17.A.2.1-1: Ad hoc group standalone data transfer request to determine ad hoc group and preconfigured group for end-to-end encryption

1.	The user at MCData client 1 wants initiate an SDS data transfer to multiple MCData users with/without end-to-end encryption supported, and either by providing the list of MCData users in the request or by providing the certain criteria in the request.

2.	The MCData client 1 sends an Ad hoc group standalone data transfer request towards the MCData server of the primary MCData system. The Ad hoc group standalone data transfer request contains either the criteria to be applied by the MCData server of the primary MCData system for determining the list of MCData users or the list of MCData users as selected by the user at MCData client 1. The request may also contain the indicator for data transfer is for emergency, imminent peril and the application identifier for which the data transfer is intended.

3.	If the ad hoc group communication is supported, the MCData server of the primary MCData system checks whether the MCData user at MCData client 1 is authorized to initiate a Ad hoc group standalone data transfer request. If not authorized, the MCData server rejects the Ad hoc group standalone data transfer request as specified in the step 9. If the request contains the list of MCData users as selected by the user at MCData client 1, the MCData server of the primary MCData system validates whether the number of MCData users provided in the request are within the configured limit. If not within the configured limit, the MCData server rejects the Ad hoc group standalone data transfer request as specified in the step 9.

4.	If the request contains the criteria, the MCData server of primary MCData system determines the list of MCData users for ad hoc group standalone short data service from the primary MCData system and determines the partner MCData system to be involved in the ad hoc group standalone short data service based on the criteria and according to local policy if required. This information element to determine the ad hoc group member contains the criteria, indicator identifying pre-defined criteria, or a combination of both.

NOTE 1:	The values of criteria, indicator identifying pre-defined criteria and determining the list of participants using these values by MCData server is left to implementation.

5.	The MCData server of primary MCData system involves the partner MCData system based on the agreement, based on the criteria for determining the list of MCData users, and according to local policy if required. It sends the ad hoc group standalone data get userlist request to the MCData server of partner MCData system. This request carries the criteria received in the Ad hoc group standalone data transfer request.

6.	The MCData server of partner MCData system determines the list of MCData users satisfying the criteria and sends the response containing the list of MCData users satisfying the criteria. The MCData server of partner MCData system may apply local policies if any while determining the participants satisfying the criteria. The determination of list of MCData users is performed only once.

7.	The MCData server of primary MCData system compiles the list of MCData users for ad hoc group standalone short data service from primary MCData system and from partner MCData system, if requested.

NOTE 2:	The step 4 to step 7 are applicable if the request contains the criteria for determining the ad hoc group members from primary system and may be from partner system.

8.	The MCData server of the primary MCData system forms the ad hoc group by using MCData users' information received in the Ad hoc group standalone data transfer request and list of MCData users to be part of ad hoc group. The MCData server further determines the preconfigured group to be used for the configuration (e.g. security related information) of the ad hoc group if end-to-end encryption is required. The MCData server assigns an MCData ad hoc group ID for the newly formed ad hoc group and optionally assign the time to live value. The ad hoc group members can communicate in the ad hoc group until the time to live value is expired. The MCData server ensure the list of MCData users determined in the step 7 are within the configured limit.

9.	The MCData server of the primary MCData system send the Ad hoc group standalone data transfer response message to MCData client 1 containing the below:

i.	The MCData ad hoc group ID (only included when the Ad hoc group standalone data transfer request is succesful);

ii.	The MCData group ID of the pre-configured group whose configuration is to be applied for this Ad hoc group standalone short data service if end-to-end encryption is required (only included when the Ad hoc group standalone data transfer request is succesful); and

iii.	Result of whether the Ad hoc group standalone data transfer request successful or failure.

If the Ad hoc group standalone data transfer request not authorized or result not succesful, the MCData client 1 shall not proceed with remaining steps for the data transfer as specified in the clause 7.17.A.3 and 7.17.A.4.

#### 7.17.A.3	Ad hoc group standalone short data service using signalling control plane involving single MCData system

##### 7.17.A.3.1	General

The initiation of Ad hoc group standalone short data service using signalling control plane results in ad hoc group members from single MCData system receiving the SDS data. The SDS payload data size is assumed to be below the configured maximum payload data size for SDS over signalling control plane.

##### 7.17.A.3.2	Procedure

The procedure in figure 7.17.A.3.2-1 describes the case where an MCData user is initiating ad hoc group standalone short data service using signalling control plane with or without disposition request.

Pre-conditions:

1.	MCData users on MCData clients 1 to n belong to the same MCData system and are already registered for receiving MCData service.

2.	The authorized MCData user at MCData client 1 wants to invite MCData users of the ad hoc group for the ad hoc group standalone short data service.

3.	Number of ad hoc group members receiving the SDS data is within the configured limit.

4.	The preconfigured group identity and preconfigured group configuration (e.g. security related information) to be used for an ad hoc group have been preconfigured in the MCData client and other MCData users of ad hoc group have also received the relevant security related information.

Figure 7.17.A.3.2-1: Ad hoc group standalone short data service using MCData ad hoc group

1.	The user at MCData client 1 wants initiates an SDS data transfer to multiple MCData users from primary MCData system.

2.	The MCData client 1 shall determine the preconfigured group if end-to-end encryption is required and an ad hoc group by executing the procedures as described in the clause 7.17.A.2.

3.	MCData client 1 sends an Ad hoc group standalone data request towards the MCData server. The request shall contain the MCData ad hoc group ID as determined in the step 2, the conversation identifier for message thread indication, may include additional implementation specific information in the application metadata container, may contain disposition request if indicated by the user at MCData client 1 and may include associated functional alias of the user at MCData client 1. If end-to-end encryption is required, the request shall contain the Preconfigured MCData group ID as determined in the step 2. If the end-to-end encryption is not required then include the payload without encryption and if required then include the payload with encryption based on the Preconfigured MCData group ID as determined in the step 2.

If the MCData user at MCData client 1 initiates an emergency ad hoc group standalone short data service or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the Ad hoc group standalone data request shall contain an emergency indicator;

ii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled; and

iii)	once an emergency SDS data transfer has been initiated, the ad hoc group is considered to be in an in-progress emergency state until SDS data transfer is completed and the time to live value of ad hoc group expires.

If the MCData user at MCData client 1 initiates an imminent peril ad hoc group standalone short data service:

i)	the Ad hoc group standalone data request shall contain imminent peril indicator; and

ii)	once an emergency SDS data transfer has been initiated, the ad hoc group is considered to be in an in-progress imminent peril state until SDS data transfer is completed and the time to live value of ad hoc group expires.

4.	If the ad hoc group communication is supported, the MCData server checks whether the MCData user at MCData client 1 is authorized to send Ad hoc group standalone data request. If not authorized, the MCData server rejects the Ad hoc group standalone data request as specified in the step 9. The MCData server checks whether any policy is to be asserted to limit certain types of message or content to certain members, for example, to location or user privilege. The MCData server also verifies whether the provided functional alias can be used and has been activated for the user.

5.	The MCData server considers the the ad hoc group memebers as implicitly affiliated to the ad hoc group.

i)	If an emergency indicator is present in the received Ad hoc group standalone data request and if the ad hoc group is not in the in-progress emergency state, the ad hoc group is considered to be in the in-progress emergency state until SDS data transfer is completed and the time to live value of ad hoc group expires; and

ii)	If an imminent peril indicator is present in the received Ad hoc group standalone data request and if the ad hoc group is not in the in-progress imminent peril state, the ad hoc group is considered to be in the in-progress imminent peril state until SDS data transfer is completed and the time to live value of ad hoc group expires; and

6.	The MCData server initiates a Ad hoc group standalone data request towards each of the target MCData users. While sending the Ad hoc group standalone data request, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients (e.g. MCData ID list). The request should contain the ad hoc group ID, pre-configured group if end-to-end encryption is required and time to live value for the ad hoc group.

7.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData clients 2 to n may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData clients 2 to n shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

8.	The receiving MCData clients send Ad hoc group standalone data responses to the MCData server.

9.	The MCData server sends the Ad hoc group standalone data response to MCData client 1 through the signalling path to inform about the result of SDS data transfer including any error (e.g. the Ad hoc group standalone data request authorization failure). The response should contain the ad hoc group ID, pre-configured group if end-to-end encryption is required and time to live value for the ad hoc group. The primary MCData server removes the ad hoc group information from the dynamic data once the time to live value is expired and thus the ad hoc group ceases to exist.

10.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report as described in the step 6 to step 9 of clause 7.4.2.5.2.

#### 7.17.A.4	Ad hoc group standalone short data service using signalling control plane involving multiple MCData system

##### 7.17.A.4.1	General

The initiation of Ad hoc group standalone short data service using signalling control plane results in ad hoc group members from multiple MCData system receiving the SDS data. The SDS payload data size is assumed to be below the configured maximum payload data size for SDS over signalling control plane.

##### 7.17.A.4.2	Procedure

The procedure in figure 7.17.A.4.2-1 describes the case where an MCData user is initiating ad hoc group standalone short data service using signalling control plane with or without disposition request.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MCData systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MCData system share their users' information to the primary MCData system.

2.	MCData users on MCData clients 1 to M belong to the primary MCData system and MCData users on MCData clients 1 to N belong to the partner MCData system and are already registered for receiving MCData service.

3.	The authorized MCData user at MCData client 1 wants to invite MCData users at MCData client 2 to M and MCData client 1 to N from ad hoc group for the ad hoc group standalone short data service.

4.	The MCData server of the primary MCData system is where the authorized MCData user/dispatcher creates the ad hoc group.

5.	Number of ad hoc group members receiving the SDS data is within configured limit.

6.	The preconfigured group identity and preconfigured group configuration (e.g. security related information) to be used for an ad hoc group have been preconfigured in the MCData client and other MCData users of ad hoc group have also received the relevant security related information.

Figure 7.17.A.4.2-1: Ad hoc group standalone short data service using MCData ad hoc group

1.	The user at MCData client 1 wants initiates an SDS data transfer to multiple MCData users from primary and partner MCData systems.

2-5.	Same steps as described in the steps 2 to 5 of clause 7.17.A.3.2.

6-6a.	The MCData server of the primary MCData system initiates an Ad hoc group standalone data request towards each of the target MCData users in the primary MCData system and the partner MCData system. While sending the Ad hoc group standalone data requests, the MCData server of the primary MCData system shall remove the information elements that are not required to be conveyed to the target MCData clients (e.g. MCData ID list). The request should contain the ad hoc group ID, pre-configured group if end-to-end encryption is required and time to live value for the ad hoc group.

7-7a.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData clients 2 to M and MCData clients 1 to N may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData clients 2 to M and MCData clients 1 to N shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData clients 2 to M and MCData clients 1 to N which is addressed to a known local non-MCData application that is not yet running shall cause the clients 2 to M and MCData clients 1 to N to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

8-8a.	The receiving MCData clients send Ad hoc group standalone data responses to the respective MCData server.

9.	The MCData server of the primary MCData system sends the Ad hoc group standalone data response to MCData client 1 through the signalling path to inform about the result of SDS data transfer including any error (e.g. the Ad hoc group standalone data request authorization failure). The response should contain the ad hoc group ID, pre-configured group if end-to-end encryption is required and time to live value for the ad hoc group. The primary MCData server removes the ad hoc group information from the dynamic data once the time to live value is expired and thus the ad hoc group ceases to exist.

10.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report as described in the step 6 to step 9 of clause 7.4.2.5.2.

* * * End of Changes * * * *