3GPP TSG-SA WG6 Meeting #56	S6-232490

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

Source:	Huawei, HiSilicon, China Mobile

Title:	Pseudo-CR on key issue of IMS call with Data Channel capability

Spec:	3GPP TR 23.700-92 v0.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

The FS_eMMTelAPP SID includes the following objective:

Study the application layer APIs which integrated with the underlying 3GPP core network architectures to allow exposure of relevant capabilities and other core network interactions to application providers/Vertical service provider.

2. Reason for Change

SA2 already finished their Rel-18 study on IMS Enhancement to support Data Channel, the IMS Architecture supporting Data Channel is listed in Annex AC of 3GPP TS 23.228 and Telco network capability APIs are listed in Annex AA.2.4 and Annex AA.2.5.

For SA6 it's very meaningful to hide telco complexity making APIs easy to consume for vertical customers and offer interactive features to vertical customers.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.0.0.

* * * First Change * * * *

## 6.1	Key Issue #X: IMS call with Data Channel capability

### 6.1.1	Description

In TS 23.228 [x], UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel. from the application providers perspective of view, there are two cases bellow:

Case 1: Take the Contact Center as an example, the Contact Center needs to initiate an IMS call with the data channel, then the Contact Center invokes the capability exposed by the eMMTel service enabler layer to establish an IMS audio/video call with the data channel.

Case2: Also take the Contact Center as an example, the customer has established an IMS audio/video call with Contact Center through the eMMTel service enabler layer, during the IMS audio/video call, Contact Center needs to add an interactive channel, then the Contact Center invokes the capability exposed by the eMMTel service enabler layer to establish a data channel for the IMS audio/video call.

And, IMS audio/video call as the basis capability of IMS Data Channel is not fully exposed in 3GPP as there was only audio related capability in 3GPP TS 23.198 [y].

* * * Next Change * * * *

6.1.2	Identified requirements

So, the Key Issue is to study the IMS call with Data Channel capability including:

-	Study, and identify how the eMMTel service enabler layer provides an IMS call.

-	Study, and identify how the eMMTel service enabler layer provides an IMS call with data channel capability and adds data channel capability to an existing IMS call.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[y]	3GPP TS 23.198: "Open Service Access (OSA)".

* * * End of Changes * * * *