3GPP TSG-SA WG6 Meeting #56	S6-232493

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-231348)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0361 | rev | 5 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the target KMS URI for a migrated MC service user |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During #52 meeting, CR S6-223567 was agreed, which was proposed to introduce a generic procedure for a private communication towards a migrated MC service user at a partner MC system. A private call redirection message is introduced to provide the necessary information of the called migrated MC service user. A private call redirection message is sent from the MC service server to the calling MC service user which includes the necessary information (The MC service ID of migrated MC service user obtained from the partner MC system) so that the calling party can reinitiate a private call towards the corresponding MC system. The other users profile configuration (e.g. calling user’s user profile configuration document) in the existing specification does not provide any information about the other migrated users related configuration (such as migrated system assigned MC service ID and associated KMS URI). So the calling MC service user`s user profile is not pre-configured with the KMS URI of the migrated MC service user information. Following EN is added to study further on determining how the KMS URI of migrated MC service user is learnt by the calling party so that end to end encryption can be achieved. Editor's note: It is FFS whether the KMS URI of the migrated MC service user is preconfigured in the calling MC service user`s user profile. The TS 33.180 (clause D.2.7) provides a way for determining the KMS URI of the MC service user using KMS Lookup request. This introduces an additional step as a part of call setup procedure which will increase further a call setup time. This CR is trying to propose the optimized solution for above Editor’s note and which uses the information which is already available with MC service server without needing an additional procedure or overhead or complexity to return the KMS URI in private call redirect response. This KMS URI information is may included in private call redirect response. If this information element is not present, then the KMS URI associated with the MC service ID of the migrated MC service user can be obtained using the procedures defined in 3GPP TS 33.180 [19]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The information elements in Table 10.16.5.2-1 updated to include the KMS URI associated to the MC service ID of the private call target user assigned at the partner MC system. In addition, procedure updated to return the KMS URI in the Private call redirection message if the end-to-end encryption is required. Removed the Editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | End-to-end encryption is not possible with migrated MC service user. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.16.5, 10.16.5.2, and 10.16.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev5: Re-submitted Rev4: Re-submitted Rev3: - 10.16.5.2 Private call redirection – Updated the NOTE to include the SA3 procedure along with proposed way of returning KMS URI. - 10.16.5.3 Procedure – Simplified the texts to convey that KMS URI is provided. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.16.5	Generic private call procedure towards a migrated MC service user

#### 10.16.5.2	Private call redirection

Table 10.16.5.2-1 describes the information flow of a private call redirection, which is sent from the MC service server to an MC service client initiating a private call towards a migrated MC service user.

Table 10.16.5.2-1: Private call redirection

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user initiating a private call, i.e., calling party. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user (i.e., called party), which the MC service user has obtained from its primary MC system before migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user, which the MC service user has obtained from its migrated MC system after Migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| KMS URI (NOTE) | O | The KMS URI associated with the MC service ID of the migrated MC service user. |
| Redirection reason | O | The MC service server informs the calling party that the target MC service user has migrated. |
| NOTE: If this information element is not present, then the KMS URI associated with the MC service ID of the migrated MC service user can be obtained using the procedures defined in 3GPP TS 33.180 [19]. |  |  |

* * * Next Change * * * *

#### 10.16.5.3	Procedure

Figure 10.16.5.4-1 presents a generic private communication procedure from MC service user 1 towards a migrated MC service user 2 where the MC system A is the primary MC system of MC service user 2 before migration, and the MC system B is the MC system that the MC service user 2 has migrated

Pre-conditions:

-	MC system A and MC system B are interconnected.

-	MC system A is the primary MC system of MC service user 2 before migration. MC system B is the MC system that MC service user 2 has migrated.

-	The MC service server A is aware that MC service user 2 has migrated and is informed of its MC service ID provided by MC system B, as described in clause 10.6.3.3.

Figure 10.16.5.3-1: MC private call towards a migrated MC service user.

1.	The MC service client 1 initiates a private call request towards MC service user 2 (MC service client 2) who has migrated to MC system B. The private call request includes among others the MC service ID of MC service user 2, which is provided by its primary MC system, as the target, i.e., called party. The private call request is described in clause 10.7.2.1.1 in 3GPP TS 23.379 [16], in clause 7.2.2.2.1 in 3GPP TS 23.281 [12], or the corresponding procedures in 3GPP TS 23.282 [13].

2.	MC service server A checks that MC service user 2 has migrated to MC system B with a new MC service ID assigned by MC system B.

3.	The MC service server A sends a private call redirection towards the MC service client 1, to inform MC service user 1 that MC service user 2 has migrated and its new MC service ID of MC service user 2 assigned by the migrated MC system. The MC service server A may include the KMS URI associated with the MC service ID of the migrated MC service user in a private call redirection. The MC service client 1 releases the private call request at step 1.

4.	The MC service client 1 initiates a private call towards MC service user 2, including the MC service ID of MC service user 2 obtained from MC system. The initiated private call can either be an MCPTT private call, an MCVideo private call, or a corresponding one-to-one MCData communication, and shall perform the procedures as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], or 3GPP TS 23.282 [13], respectively.

NOTE:	If end-to-end encryption is required, the migrated MC service user 2 is assumed to have the necessary security information needed to establish a protected call, as defined in 3GPP TS 33.180 [25].

* * * End of Changes * * * *