3GPP TSG-SA WG6 Meeting #56	S6-232503

Gothenburg, Sweden 21st – 25th August 2023

Source:	Qualcomm

Title:		Discussion on latency requirement of first packet(s) transmission for mission critical service

Agenda Item:	4.2

Contact:	Nishant Gupta <gnishant@qti.qualcomm.com>

Abstract: SA6, through an LS to SA2 in S6-231018, highlighted a potential issue of packet loss when using MBS for mission critical services. SA2 has discussed the issue and asked SA6, through their reply LS in S6-232240/S2-2307982, if there is a latency requirement of first packet(s) transmission for mission critical service. No latency requirement of first packet(s) transmission is specified for mission critical service, however service KPIs for access time, end-to-end access time and mouth-to-ear latency are defined. This discussion paper highlights these aspects and proposes to provide a reply to SA2.

Potential Issue as described in SA6 LS to SA2 (S6-231018):

According to the LS sent to SA2 in S6-231018, packet loss could happen in multicast MBS delivery, when the MBS session is used after a period of long inactivity – resulting in some UE’s going into idle (or inactive) RRC mode.

Reply from SA2 asking for more information (S6-232240/S2-2307982):

Reply LS from SA2 (S6-232240/S2-2307982) indicates that SA2 discussed the LS from SA6 and contemplated on multiple options to address SA6’s concerns, including use of MB-UPF buffering DL MBS data.

MB-UPF buffering the DL MBS data, while waiting for all the concerned UEs to become active has latency implications on delivery of the first downlink packet.

Concerned about this delay due to buffering, SA2 enquires from SA6 the KPI/latency requirements for transmission of the first downlink packet(s).

MC KPI requirements:

Observation 1: No KPI requirement is defined for the latency or transmission delay aspects of the first downlink packet(s).

However, following 3 service KPIs are defined in 3GPP TS 22.179:

-	KPI 1: [R-6.15.3.2-012] For group calls where no acknowledgement is requested from affiliated MCPTT group members, the MCPTT Service shall provide an MCPTT Access time (KPI 1) less than 300 ms for 95% of all MCPTT Request.

-	KPI 2: [R-6.15.3.2-014] For group calls where automatic acknowledgement is requested from the UEs of the affiliated MCPTT group members, the MCPTT Service shall provide an End-to-end MCPTT Access time (KPI 2) less than 1000 ms for users under coverage of the same network.

-	KPI 3: [R-6.15.3.2-015] The MCPTT Service shall provide a Mouth-to-ear latency (KPI 3) that is less than 300 ms for 95% of all voice bursts.

Observation 2: Considering these KPIs it can be inferred that a delay of up to 1000ms (KPI 2) for call setup i.e., time between PTT Request and PTT Request grant, is acceptable. Additionally, between the PTT Request grant and playback of voice at the receiving terminal (KPI 3), the delay shall be less than 300ms for 95% of all the requests.

When using multicast, additional delay for setup is expected:

Reply LS from SA2 (S6-232240/S2-2307982) also highlights that an additional delay is expected to allow battery saving techniques when using MBS, as highlighted by NOTE 8 in Table 5.7.4-1 (Standardized 5QI to QoS characteristics mapping) of 3GPP TS 23.501:

NOTE 8:	In both RRC_IDLE and RRC_CONNECTED mode, the PDB requirement for these 5QIs (QCI 65 and 69) can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques.

Observation 3: In order to permit reasonable battery saving (DRX) techniques, PDB requirements for the first downlink packet(s) for mission critical related 5QIs (QCI 65 and 69) can be relaxed.

Summary and proposal:

Observation 1: No KPI requirement is defined for the latency or transmission delay aspects of the first downlink packet.

Observation 2: Considering these KPIs it can be inferred that a delay of up to 1000ms (KPI 2) for call setup i.e., time between PTT Request and PTT Request grant, is acceptable. Additionally, between the PTT Request grant and playback of voice at the receiving terminal (KPI 3), the delay shall be less than 300ms for 95% of all the requests.

Observation 3: In order to permit reasonable battery saving (DRX) techniques, PDB requirements for the first downlink packet(s) for mission critical related 5QIs (QCI 65 and 69) can be relaxed.

Proposal: It is proposed to reply to SA2 with the above observations. A draft proposal is available in S6-232504.