3GPP TSG-SA WG6 Meeting #56	S6-232504

Gothenburg, Sweden 21st – 25th August 2023

Title:	Reply LS on addressing packet loss during multicast MBS delivery

Response to:	S6-232240/S2-2307982 on addressing packet loss during multicast MBS delivery from SA2

Release:	Rel-17

Work Item:	5MBS, MCOver5MBS

Source:	SA6

To:	SA2

Cc:	RAN2, RAN3

Contact person:	Nishant Gupta

gnishant@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

SA6 thanks SA2 for the reply LS and the question on latency requirement of first packet(s) transmission for mission critical service.

SA6 would like to inform SA2 that no latency requirement is defined for first downlink packet(s) for mission critical services. However, following service KPI requirements are defined in 3GPP TS 22.179:

-	Access time (KPI 1) – time taken between transmission of a PTT Request by the transmitting user and reception of a PTT Request grant, for calls where no acknowledgement from the receiving end is required  - shall be less than 300ms for 95% of all MCPTT Requests; and

-	End-to-end Access time (KPI 2) – For group calls where automatic acknowledgement is requested from the UEs of the affiliated MCPTT group members, the MCPTT Service shall provide an End-to-end MCPTT Access time (KPI 2) less than 1000ms for users under coverage of the same network.

-	Mouth-to-ear latency (KPI 3) – time between an utterance by the transmitting user after PTT Request grant is received, and the playback of the utterance at the receiving user's speaker – shall be less than 300ms for 95% of all voice bursts.

Considering these KPIs, a delay of up to 1000ms (KPI 2) for call setup i.e., time between PTT Request and PTT Request grant, is acceptable. Additionally, between the PTT Request grant and playback of voice at the receiving terminal (KPI 3), the delay shall be less than 300ms for 95% of all the requests.

Further, SA6 also acknowledges that use of MBS may require additional setup time, considering the potential delay for first downlink packet(s) to permit reasonable battery saving (DRX) techniques, as described in NOTE 8 in Table 5.7.4-1 of 3GPP TS 23.501, and highlighted by the reply LS from SA2. SA6 also acknowledges that overall latency is dependent on UE state and how that is managed is up to configurations in the UE, CN and RAN.

SA6 requests SA2 to take the above feedback into account.

# 2	Actions

To SA2:

ACTION: 	SA2 kindly asks RAN2 to take the above feedback into account.

# 3	Dates of next TSG SA WG 6 meetings

SA6#57                 9th October – 13th October 2023 	Xiamen, China

SA6#58                 13th November – 17th November 2023 	Chicago, US