3GPP TSG-SA WG6 Meeting #56	S6-232510

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0079 | rev | 0 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-triggered dynamic VRU zone configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XApp2_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Added Procedure on UE-triggered dynamic VRU zone configuration from TR 23.700-64 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Procedure on UE-triggered dynamic VRU zone configuration from TR 23.700-64 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution remains unsupported in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.21.1; New: 9.21.2.x1; 9.21.2.x2; 9.21.2.x3; 9.21.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.21.1	General

This capability supports the configuration and provisioning of the VRU high risk zone at the VAE layer based on a request from the V2X application specific server (VASS). UE-initiated triggering of configuration and provisioning of  protection zones is also supported at the VAE layer using the VRU zone concept, with the VRU zone configuration being dynamic and specific to the protected UE.

* * * Next Change * * * *

### 9.21.2	Information flows

#### 9.21.2.x1	Protection zone management request

Table 9.21.2.x1-1 describes information elements for the Protection zone management request from the VAE client to the VAE server.

Table 9.21.2.x1-1: Protection zone management request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor UE ID | M | The identifier of the VAL UE for which the protection zone is applicable |
| VRU zone information | M | This information identifies, based on the VRUP scenario, which type of UEs (V2X UEs, pedestrian UEs) are to be considered. |
| VRU application requirements | M | This includes requirements include the types of supported messages and requirements (e.g. requirements for VAM messages), and application QoS requirement dedicated for the VRU zone (e.g. URLLC like). |
| V2P communication parameters | M | This includes V2P communication parameters such as V2X identifiers, radio communication parameters, supported transmission modes (unicast, groupcast, broadcast) within the protection zone, communication schedule, and QoS parameters. |
| Area of Interest | M | The geographical or service area for which the request applies |
| Time validity | M | The time validity of the request |

#### 9.21.2.x2	Protection zone management response

Table 9.21.2.x2-1 describes information elements for the VRU zone management subscription response from the VAE server to VAE client.

Table 9.21.2.x2-1: Protection zone management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the VRU zone management subscription request (positive or negative acknowledgement) |
| VRU zone ID | M | An identifier of the VRU zone assigned by the VAE layer. |

#### 9.21.2.x3	Protection zone management notification

Table 9.21.2.x3-1 describes information elements for the Protection zone management notification from the VAE server to the VASS .

Table 9.21.2.x3-1: Protection zone management notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor UE ID | M | The identifier of the VAL UE for which the protection zone is applicable |
| VRU zone ID | M | An identifier of the VRU zone which may be provided by the requestor or assigned by the VAE layer. |
| VRU zone information | M | This information identifies, based on the VRUP scenario, which type of UEs (V2X UEs, pedestrian UEs) are to be considered. |
| VRU zone area | M | The geographical (coordinates) or topological area for the zone (cell IDs, TAs) |
| VRU zone configuration parameters | M | These parameters include: - The list of value-add VAE services required within the zone (e.g. location tracking, V2X message bundling), - the supported transmission modes (unicast, groupcast, broadcast) within the applications within the zone, - the allowed interface selection within the zone (Uu, PC5), the use of application relaying or not |
| Area of Interest | M | The geographical or service area for which the request applies |
| V2P communication parameters | M | This includes V2P communication parameters such as V2X identifiers, radio communication parameters, supported transmission modes (unicast, groupcast, broadcast) within the protection zone, communication schedule, and QoS parameters. |
| VRU zone adaptation assistance info | O | If the VRU zone is dynamic, this IE provides assistance info on the configuration adaptation triggers or to provide the planning (e.g. based on a school bus route) to allow the network to adapt the configuration. |
| Time validity | M | The time validity of the request |

* * * Next Change * * * *

### 9.21.x4	 Procedure on UE-triggered dynamic VRU zone configuration

VRU-specific protection zones are supported in the VAE layer via dynamic VRU zones configured based on UE-initiated requests and VASS configurations. The procedure in figure 9.21.x4-1 complements the VASS Server-triggered procedure in clause 9.21.3.

Pre-conditions:

- V2X application specific server or client has subscribed to VAE layer to provide support for V2P communication.

- The VAE server is preconfigured with a policy allowing support of VRU-specific protection zone configuration and support of UE/VRU-initiated requests.

Figure 9.21.x4-1: UE-triggered dynamic VRU-specific (i.e. protection) zone configuration

- a.  Optionally, an application client on a pedestrian/ VRU-candidate UE makes a request to the VAE client to create a protection zone, i.e. a dynamic VRU zone specific to the UE.

b.  The VAE client sends a request to a VAE server to create a protection zone. The request includes V2X Service ID, UE ID, application ID, UE location and location reporting configuration, destination location, request for dynamic VRU zone, V2P communication configuration, user consent for using user application context information, etc.

2.	The VAE server sends a notification to a VASS for the creation of the protection zone for the candidate protected VRU  and includes the information provided by the requesting UE.

3.	The VASS authorizes the request and determines whether a protection zone based on information received from the VAE server is to be created. The VASS provides the VAE server with the protection zone and V2P communication configuration.

4.	The VASS determines a dynamic VRU zone configuration corresponding to the protection zone request and configures the VAE server accordingly. Steps 1-3 of clause 9.21.3 are executed for the dynamic VRU zone determined.

5.	The VAE server configures CN and SEAL LMS subscriptions to obtain UE location from the 5G system for the protected UE and for vehicular UEs around the protected zone. Sources of UE mobility information may come from SEAL LMS servers, EES, an analytics function (e.g. from ADAES or NWDAF), and the 5G core network. For V2X UEs location obtained from the location management server, the procedures specified in 3GPP TS 23.434 (clause 9.3.12) apply.

- a. The VAE Server sends a response to the VAE client that the protection zone has been created.

b.  Optionally, the VAE client sends a response to a triggering application client on the protected UE, confirming the creation of the protection zone.

7.	The VAE server uses notifications from the 5G network and SEAL LMS to update the dynamic VRU-specific zone e.g. based on updated location information of the protected UE .

8.	The VAE server sends notification messages including the dynamic VRU zone configuration parameters to other VRUs (pedestrians or V2X-UEs at similar risk) within the area that will be covered by the dynamic VRU-specific zone. The VAE server also sends vehicular UEs V2P communication parameters to enable communication between the vehicular UE and the protected UE.

9. a.The VAE server alerts the VAE client when V2X UEs posing risk to the protected VRU are expected to move into the dynamic VRU-specific zone area in a given time horizon.

b. The VAE server alerts VASS when V2X UEs posing risk to the protected VRU are expected to move into the dynamic VRU-specific zone area.

* * * End of Changes * * * *