3GPP TSG-SA6 Meeting #56	S6-232511

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0466 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on proxy bundle type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Uangel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in the clause 7.2.10, one EAS (i.e., proxy EAS) coordinates with the other EASs of the EAS bundle to provide services to the AC. An EAS is defined to provide server functions that AC can connect to and they are not capable of consuming services provided by the other EASs. While it is possible for the proxy EAS to consume the EAS Service APIs exposed by the other EASs as described in the clause 6.7.1, their operations such as API publication or discovery are based on CAPIF so that the detailed operations such as EAS discovery or ACR for proxy bundle type cannot be handled by EEL. Thus, it is proposed to clarify the detailed operations (i.e., EAS interactions, ACR) for proxy bundle type in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - to clarify how to interact in the proxy bundle type with CAPIF and the clause 6.7.1. - to modify the title of the ACR operations to cover direct bundle type only - to add an editor’s note that ACR for proxy bundle type is FFS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EEL operations for proxy bundle type does not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.10, 8.8.1.6, 8.8.2.8, 8.8.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

7.2.10	EAS bundle information

The EAS bundle information includes EAS bundle type, a list of EASIDs or a bundle ID and EAS bundle requirements. The EAS bundle information also includes main EASID. EAS bundle ID establishes an association between the EASs. When included in the EAS profile, EAS bundle ID denotes the bundle to which the EAS belongs. When included in the AC profile EAS bundle ID is used to perform different Edge Enabler Layer operations, such as EAS discovery. Edge Enabler Layer handles the EASs belonging to the same bundle as required by related EAS bundle requirements as described in clause 8.2.10.

NOTE 1 :	Both, EAS bundle ID and EAS bundle requirements, are provided by the ASP.

NOTE 2:	Bundle ID is necessary when the affinity between bundled EASs is strong (e.g., co-deployment and co-migration is essential), and the related ASPs which provide the AC and bundled EASs established the bundle. List of EASIDs is required when the affinity between the bundled EASs is weak (e.g., co-deployment and co-migration is only "nice to have").

NOTE 3:	Following types of EAS bundles are considered in this release:

-	Direct bundle, where AC interacts with multiple EASs of the EAS bundle directly with no coordination between the EASs; and

-	Proxy bundle, where the AC interacts with one EAS of the EAS bundle which in turn coordinates with other EASs of the EAS bundle to provide services to the AC by consuming EAS Service APIs via CAPIF as described in the clause 6.7.1.

NEXT CHANGE

8.8.1.6	Service continuity for EAS bundle

This clause describes solution of relocating EASs in a bundle together instead of individual relocation for AC-EAS sessions one by one. To avoid ACR being triggered for each EAS in a bundle with different initiators (e.g. EAS 1 and EAS 2 in a bundle trigger ACR simultaneously), a main EAS may be used and a main EES is used correspondingly. The main EAS or EES is responsible for ACR detection and initiation in the network side. The main EAS information is sent to EEL and the main EES is the EES registering the main EAS.

NOTE 1:	ASP can have requirement of the dependencies between bundled EAS(s) when provisioning them for any deployment scenario, and indicate whether the affinity between them as strong (co-deployment is essential) or weak (co-deployment is only "nice to have").

NOTE 2:	It is possible that some EASs in a bundle do not need relocation because the UE can still be served by these EASs. A deployment example is both EASs providing services covering the whole city and EASs providing services covering city district are serving the AC as an EAS bundle, and when UE moves from one district to another district in the city, only EASs serving the district from where UE is moving out need relocation.

NOTE 3:	In the proxy type of bundle, the main EAS is the connecting EAS serving the AC.

NOTE 4:	In current release of the specification, the main EAS is selected by ASP.

Editor’s note: ACR for proxy bundle type is FFS.

NEXT CHANGE

8.8.2.8	ACR for direct EAS bundle, executed by S-EAS

In this scenario, the main S-EAS executes and triggers necessary ACR for all AC-EAS service session(s) in a bundle.

This scenario description is same as described for figure 8.8.2.4-1 except for the following clarifications:

Pre-condition:

1.	The main S-EAS may depend on the receipt of ACR management events from its S-EES, e.g. "user plane path change" events or "ACR monitoring" events as described in clause 8.6.3, to detect the need for an ACR. The main S-EAS may also depend on the receipt of UE location notification from its S-EES as described in clause 8.6.2.2.3, to detect the need for an ACR. For the following procedure it is assumed that the main S-EAS has subscribed to continuously receive the respective events; and

2.	The EEC has subscribed to receive ACR information notifications for target information notification events and ACR complete events from S-EESs serving the EAS bundle, as described in clause 8.8.3.5.2.

Figure 8.8.2.8-1: S-EAS executed ACR for EAS bundle

Phase I: ACR Detection

1.	ACR is detected, the procedure is same as step 1 of clause 8.8.2.4.

Phase II: ACR Decision

2.	The main S-EAS performs ACR decision for bundled EASs.

NOTE 1:	The main S-EAS is aware of all bundle EASs service area and/or DNAIs so it can decide whether to perform ACR for the corresponding EASs.

Phase III:	ACR Execution

3.	The main S-EAS discovers from all candidate T-EAS(s) in EAS bundle. If the affinity is set to strong, then T-EASs are from the same EDN. The main S-EAS may apply AF traffic influcne for all bundled T-EAS(s).

NOTE:	T-EAS(s) should be within the same EDN as preferred if EAS affinity is preferred, or different EDNs if no T-EAS(s) in same EDN available, if the EAS affinity is set to weak.

4.	The procedure is same as step 4 to step 6 of clause 8.8.2.4. The main S-EAS sends selected T-EAS declaration message to the main S-EES with the selected T-EASs, the main S-EES sends selected T-EAS(s) to the EEC.

5.	The main S-EES informs associated S-EESs with bundled T-EASs. Then ACT starts between the S-EAS(s) and T-EAS(s) in a bundle requiring service continuity.

Phase IV:	Post-ACR Clean up

6.	During post-ACR, all S-EASs and T-EASs send ACR status update to S-EES and T-EES, respectively, as described in step 8 and step 9 of clause 8.8.2.4. The EEC collects ACR complete notifications from all S-EESs and ACR for EAS bundle is completed.

NEXT CHANGE

8.8.2.9	ACR for direct EAS bundle, executed by S-EES

In this scenario, a main S-EES (serving a main S-EAS) executes and triggers necessary ACR for AC-EAS service session(s) in a bundle.

This scenario description is same as described for figure 8.8.2.5-1 except for the following clarifications:

Pre-condition:

1.	The AC at the UE already has a connection with S-EASs in a bundle;

2.	The EEC subscribes to receive ACR information notifications for target information notification events and ACR complete events from S-EESs serving the EAS bundle, as described in clause 8.8.3.5.2;

3.	The main S-EAS may subscribe to receive ACR management notifications for "ACR facilitation" events to the main S-EES, in order to enable ACR detection at the main S-EES.

Figure 8.8.2.9-1: S-EES executed ACR for EAS bundle

Phase I: ACR Detection

1.	The main S-EES detects the need for ACR, the procedure is same as step 2 of clause 8.8.2.5.

Phase II: ACR Decision

2.	The main S-EES performs ACR decision for bundled EASs.

NOTE 1:	The main S-EES is aware of all bundle EASs service area and/or DNAIs so it can decide whether to perform ACR for the corresponding EASs.

Phase III:	ACR Execution

3.	The main S-EES discovers all candidate T-EAS(s) in EAS bundle. If the affinity is set to strong, then T-EASs are from the same EDN.

NOTE 2:	T-EAS(s) should be within the same EDN as preferred if EAS affinity is preferred, or different EDNs if no T-EAS(s) in same EDN available, if the EAS affinity is set to weak.

4.	The procedure is same as step 5b to step 10 of clause 8.8.2.5. The main S-EES sends selected T-EAS(s) to the EEC, triggers application traffic influence for T-EAS(s) and notifies the main S-EAS with selected T-EAS of the same EAS service. The main S-EES may notify more S-EAS(s) with selected T-EAS of the corresponding EAS service.

5.	The main S-EES performs ACR launching procedure (as described in clause 8.8.3.4) with the ACR action indicating ACR initiation and the corresponding ACR initiation data to the associated S-EES(s).

6.	The associated S-EES(s) notifies the corresponding bundled S-EAS(s) and ACT starts between the S-EAS(s) and T-EAS(s) in a bundle requiring service continuity.

Phase IV:	Post-ACR Clean up

7.	During post-ACR, all S-EASs and T-EASs send ACR status update to S-EES and T-EES, respectively, as described in step 12 and step 13 of clause 8.8.2.5. The EEC collects ACR complete notifications from all S-EESs and ACR for EAS bundle is completed.

END OF CHANGES