3GPP TSG-SA WG6 Meeting #56	S6-232650

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232344)

Source:	Samsung

Title:	Pseudo-CR on Mapping roles between EDGEAPP architecture and GSMA OPG architecture

Spec:	3GPP TR 23.958

Agenda item:	7.9

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR introduces mapping of roles between EDGEAPP architecture and GSMA OPG architecture.

2. Reason for Change

In S6-232233, GSMA OPG has asked 3GPP to provide their view on mapping, Quote:

“GSMA OPG kindly asks 3GPP SA6 to provide their view on architecture mapping between EGDEAPP and OP”

It is required for 3GPP SA6 to provide mapping of roles as defined by OP to 3GPP defined EDGEAPP architecture entites.

Added new clause to specify the roles by GSMA OPG reference architecture and provided possible mapping with EDGEAPP architecture

Further, Edge Cloud and Edge Cloud Resource are defined as follows in GSMA PRD:

| Edge Cloud (EC) | Cloud-like capabilities located at the network edge including, from the Application Provider's perspective, access to elastically allocated compute, data storage and network resources. Edge Clouds are targeted mainly at Edge-Enhanced Applications and Edge-Native Applications. In the context of this document, the Edge Cloud is managed by an Operator Platform's Service Resource Manager Role. The phrase "located at the infrastructure edge" is not intended to define where an Operator deploys its Edge Cloud. The Edge Cloud is expected to be closer (for example, latency, geolocation, etc.) to the Application Clients than today's centralised data centres, but not on the User Equipment, and could be in the last mile network. Note 1: This definition is based on that in "Open glossary of edge computing", v2.0 [3]. |
| --- | --- |
| Edge Cloud Resources | In the context of this document, resources of the Edge Cloud Service that are managed by the Service Resource Manager Role. |

As specified, The Edge Cloud is expected to be closer (for example, latency, geolocation, etc.) to the Application Clients than today's centralised data centres, but not on the User Equipment, and could be in the last mile network. So, Edge Cloud can be mapped to EDN.

Further, Edge Could Resources can be mapped to list of EESs and list of EASs supported within EDN.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958 (v0.6.0).

* * * First Change * * * *

## 6.x	Mapping roles between EDGEAPP architecture and GSMA OPG architecture

GSMA Operator Platform Group has published their edge requirements in Operator Platform Telco Edge Requirements [3].

Clause 3 of Operator Platform Telco Edge Requirements [3] provides the OP Roles and Interfaces Reference Architecture. Clause 3.2 of Operator Platform Telco Edge Requirements [3] defines roles along with their key functions.

The three main roles in the GSMA OP architecture and their mapping with EDGEAPP architecture entity is as follows:

1)	Capabilities Exposure Role: It enables an Application Provider to operate their applications. Table 6.x-1 shows the scenarios enabled by Capabilities exposure role as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table 6.x-1: Capabilities exposure role

| Scenario | Description | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| Edge Cloud Infrastructure Endpoint Exposure | The Application Provider uses an authenticated and authorized endpoint to carry out scenarios involving application instances on edge clouds; | EES (as per clause 6.3.2 and clause 8.5) |
| Application Onboarding | The Application Provider uses the NBI to provide application images and metadata to the OP Federation Broker/Manager Role | See NOTE. |
| Application Metadata/Manifest Submission | The Application Provider uses the NBI and the metadata model to submit application metadata to the OP and follows defined procedures to extend the metadata model specification | See NOTE. |
| Application CI/CD Management DevOps | The Application Provider integrates the CI/CD framework used to create an application with the OP via NBI APIs (which implies an integration between a CI/CD framework and Application Onboarding and Lifecycle Management) | See NOTE. |
| Application Lifecycle Management | The Application Provider observes and changes the operational state of application instances, including the geographical/network extent of the OP on which application instances may run; | See NOTE. |
| Application Resource Consumption Monitoring | The Application Provider observes resource consumption of application instances, using the resource data model | See NOTE. |
| Edge Cloud Resource Catalogue exposure | The Application Provider inventories edge cloud resources nominally available to application instances | See NOTE. |
| Network Capabilities exposure | The Application Provider inventories network capabilities, like Network Analytics, nominally available to application instances | EES (as per clause 8.7.3) |
| NOTE: The scenario may be within scope of 3GPP SA5. |  |  |

2)	Service Resource Manager Role: The Service Resource Manager role in the OP is responsible for managing Cloud and Network resources from the Edge Cloud(s) via the SBI and UNI interfaces. Table 6.x-2 shows typical scenarios enabled by the Service Resource Manager role towards the different interfaces as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table 6.x-2: Service Resource Manager Role

| Interface | Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| SBI | Inventory, Allocation and Monitoring of Compute resources from Edge Cloud Infrastructure via the Southbound Interface – Cloud Resources (SBI-CR); | See NOTE. |
|  | Orchestration of Application instances on the Edge Cloud Infrastructure via the SBI-CR interface | See NOTE. |
|  | Cloud resource reservation managed by the OP | See NOTE. |
|  | Configuring UE traffic management policies to accomplish the application's requirements, , or the UE's IP address shall be maintained | See NOTE. |
|  | Exposure of usage and monitoring information to operator's charging engine via the Southbound Interface – Charging functions (SBI-CHF) to enable operators to charge for the OP's services. | See NOTE. |
| Southbound Interface – Network Resources (SBI-NR) | Fetch Cloudlet locations based on the mobile network data-plane breakout location | See NOTE. |
|  | Subscribe and receive notifications on UE Mobility events from the network to assist applications | EES and ECS (clause 8.10) |
|  | Configure traffic steering in the Mobile Network towards Applications orchestrated in Edge Clouds | EES (related to traffic influence) |
|  | Receive statistics/analytics, e.g. to influence Application placement or mobility decisions |  |
|  | Receive information related to the network capabilities, such as QoS, policy, network information, etc | EES and ECS (clause 8.10) |
| UNI | Application Instantiation/Termination, e.g. based on triggers from the UNI | EES (clause 8.12) |
|  | Application Endpoint exposure towards User Clients (UC) via the UNI | EES (clause 8.4.2 and clause 8.5) |
|  | Application Placement decisions, e.g. based on measurements/triggers from the UNI | See NOTE. |
| NOTE: The scenario may be within scope of 3GPP SA5. |  |  |

3)	Federation Broker and Federation Manager Roles: The Federation Broker and Manager Roles in the OP are responsible for interfacing with other OPs via the East-West Bound Interface. Table 6.x-3 shows typical scenarios enabled by the Federation Broker and Federation Manager Roles as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table 6.x-3: Federation Broker and Federation Manager Roles

| Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- |
| Federation Interconnection Management |  |
| Edge Cloud Resource Exposure and Monitoring towards partner OPs | ECS (Edge Cloud Resource Exposure, clause 8.17.2.4) |
| Network and Analytics Capabilities Exposure towards partner OPs | See NOTE. |
| Application Images and Application metadata transfer towards partner OPs | See NOTE. |
| Application Instantiation/Termination towards partner OPs | See NOTE. |
| Application Monitoring towards partner OPs | See NOTE. |
| Edge Cloud Resource Catalogue exposure | EES (Exposing EAS profile) |
| Service Availability in visited networks | ECS (clause 8.17.2.4) |
| NOTE: The scenario may be within scope of 3GPP SA5. |  |

Based on the mapping tables in this clause, EES and ECS of 3GPP EDGEAPP architecture can be mapped to take responsibilities of the some roles as defined in GSMA OPG reference architecture.

* * * End of Change * * * *