3GPP TSG-SA WG6 Meeting #56	S6-232666

# Goteborg, Sweden, 21st August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0227 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service area triggering criteria in the SS_LocationReporting API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The received LS S6-231661 asking SA6 to clarify the VAL service area based triggering criteria. This CR proposes to specify the VAL service area based triggering criteria in explicit form to address the questions in the received LS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the VAL service area based triggering criteria in the SS_LocationReporting API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of the VAL service area functionality cannot be completed in the stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1, 9.3.2.4, 9.3.3.4, 9.3.13. 9.3.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR addresses LS S6-231661. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

The SS_LocationReporting API does not specify clearly the triggering criteria for the location reporting based on the VAL service area.

This CR and the corresponding LS reply propose to specify a flexible VAL service area triggering criteria that supports both periodic and event-triggered location reporting.

We would like to discuss the following key aspects of the VAL service area triggering criteria:

- the triggering criteria enable the reporting inside and outside the locations defined in the VAL service area (the possibility of the UE location monitoring inside and outside the locations defined in the VAL service area dramatically improves the application usability, i.e., the application does not need to create and synchronize any changes between definition two VAL service areas that represent the locations inside and outside);

- the triggering criteria support a custom time range for the location reporting, e.g., the reporting shall occur each working day during the working hours.

- the triggering criteria support a configurable granularity of the location change conditions, e.g., cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates.

The VAL service area based triggering criteria with the key aspects listed above enable a wide variety of the use cases of the SS_LocationReporting API. For example:

- tracking assert in case of theft or inproper use: the factory owner would like to track the GPS coordinates (aspect 3) of a factory loader outside (aspect 1) the factory area in non-working hours (e.g., after 5:00 PM during working days and weekends) (aspect 2).

- the HR system would like to receive an indication if a worker is outside of the factory area (aspect 1) at 8:00 AM (aspect 2).

- the security team wants to receive a periodical report of the VAL user/UE location that is inside the forbidden areas identified by the VAL service area.

23.273 18.2.0 defines the reporting indication with event reporting allows the reporting outside the geographical location, i.e., clause 5.1 states, the event report allowed area can also be used differently if an area usage indication is provided together with the power saving area by the UE. A reporting indication that is determined by GMLC based on the area usage indication means when the UE detects the triggered or periodic event happens, if it is outside the event report allowed area, the UE is allowed to generate and send the event report to network to reduce UE power consumption.

To avoid non-efficient energy consumption at VAL UE side, an addititonal indication of the time range is added to the location reporting configuration response, i.e., the VAL UE/VAL Group shall report the location during the given time periods to reduce energy consumption and prolong the productive time of the VAL UE.

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Requested location access type (see NOTE 2) | O (see NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested positioning method | O (see NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report as specified in table 9.3.2.4-2. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE 1: If none of the information element is present, this represents a cancellation for location reporting. NOTE 2: The non-3GPP access as defined in 23.273[50] and TS 29.572 [51] is out of scope of the present specification. |  |  |

* * * Next change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested. This information element may be represented as VAL service area ID(s) when this information flow from VAL server to the location management server. |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE as specified in table 9.3.2.4-2. |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

Table 9.3.2.4-2: Location reporting Triggering criteria

| Information element | Status | Description |
| --- | --- | --- |
| Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event-triggered. |
| Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| Location change condition | O (see NOTE) | It indicates the location change condition when the reporting shall occur, e.g., VAL user or VAL UE changes a cell, NodeB, TA, RA, WLAN access network, civic address, GPS coordinates, SAI, ECGI, RAT, VAL service area. If the Frequency of reporting is event-triggered, this IE shall be provided. |
| Inside or outside indication | O (see NOTE) | It indicates the condition when the reporting shall occur, i.e., the VAL user or VAL UE is inside or outside the locations defined by the VAL service area ID(s). |
| Reporing schedule | O | It indicates the requested reporting schedule, e.g., days of the week and/or time period for the location reporting. |
| NOTE: These information elements may be provided when this information flow is from the VAL serverto the location management server. |  |  |

* * * Next Change * * * *

#### 9.3.3.4	Update Location reporting configuration

The location management server may update the location reporting configuration information and inform the location management client of the update ones at any time by triggering the location reporting configuration update procedure.

Figure 9.3.3.4-1 illustrates the procedure for location reporting configuration update.

Figure 9.3.3.4-1: Location reporting configuration update procedure

1.	Based on the local configurations or the location information request from other entities (e.g., another location management client, VAL server) or if the VAL server updates the geographical location information of the existing VAL service area ID and there is triggering criteria corresponding to the updated VAL service area ID, the location management server decides to update the location reporting configuration and informs the updated information to the location management client.

2.	The location management server sends location reporting configuration update request message to the location management client(s) which may contain the updated location reporting configuration, e.g. the event triggers, the minimum time between consecutive reports, the requested location access type and positioning methods when VAL UE fetches the UE location report. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time. If the location reporting trigger from VAL server contains the triggering criteria based on the VAL service area ID, then the location management server converts those triggers into geographical area based triggers while sending it to the location management client.

NOTE:	Different location management clients may be given different location reporting criteria.

3.	The location management client(s) reply with location reporting configuration update response to the location management server with the update result.

4. The location management client stores or updates the new location reporting configuration received in step 2.

* * * Next Change * * * *

### 9.3.13	VAL Service Area configuration

#### 9.3.13.1	General

The VAL service area refers to the geographical area served by a VAL server. The VAL server may have more than one VAL service area defined. These VAL service areas are configured by the VAL server to the location management server and and are assigned with the unique identifier. When the VAL service area configurations in place the VAL server may use the VAL service area ID as part of the location information in the SEAL APIs requiring location information. These configurations can be updated by the VAL server in-order to modify the geographical area associated with the VAL service area ID and when is updated, the SEAL internally manage the updates without requiring the VAL server to re-invoke the APIs where the updated VAL service area ID are being used.

The VAL server may configure the location reporting trigger with the triggering criteria based on the VAL service area ID. These triggers could be to receive the location report from the VAL client when it enters a particular VAL service area or when it exits a particular location identified in the location change conditions. Since the VAL service area ID is not configured at the VAL client, the location management server shall convert the VAL service area ID based triggers to the corresponding mapped geographical area while configuring the location management client for location reporting.

* * * End of changes * * * *