3GPP TSG-SA WG6 Meeting #56	S6-232545

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-232265)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0143 | rev | 1 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to clause 8.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.1.4 mentions UE-2a and UE-2b but there is no figure that shows where UE-2a and UE-2b fit in the MSGin5G architecture. In a previous meeting the architecture was simplified and a Proxy UE no longer exists. Clause 8.2.8 shows UE-3 as the Gateway UE and not UE-1. Table 8.1.4-1 mentions the maximum time allowed for processing the bulk registration request, but the MSGin5G Server is also involved in this processing. A requirement cannot be imposed on both entities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace "UE-2a and UE-2b"with UE-2. Replace "Proxy UE" by "Gateway UE". Replace "UE-1" by "UE-3". Table 8.1.4-1 specifies the maximum wait time for the Gateway UE before the bulk registration message is forwarded to the MSGin5G Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of entities that don't exist in the architecture may lead to wrong implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The first paragraph was in NO style and has been corrected to Normal style, but this doesn't show up with revision marks. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.1.4	MSGin5G UE bulk configuration over MSGin5G-6 reference point

When multiple instances of constrained device MSGin5G UE-2, which support an MSGin5G Client, get the MSGin5G Service configuration information via MSGin5G UE-3, the MSGin5G UE-3 may decide to use bulk configuration procedure specified in this clause based on service policy.

Pre-conditions:

1.	MSGin5G UE-2 has discovered and selected MSGin5G UE-3 as the MSGin5G Gateway UE as specified in clause 8.2.8 and connected to the serving network via MSGin5G UE-3 successfully.

2.	The MSGin5G UE-3 has been configured by using the procedure specified in clause 8.1.2 of the present document.

The MSGin5G UE-2 sends the Get VAL UE configuration request as specified in clause 8.1.2 of the present document to MSGin5G UE-3. If the MSGin5G UE-3 decides to use bulk configuration based on the service policy, the MSGin5G UE-3 sends a Get VAL UE configuration response including the information elements as listed in table 8.1.4-1 as a part of VAL UE configuration data to MSGin5G UE-2 before bulk configuration is performed. This Get VAL UE configuration response is used to inform each instance of MSGin5G UE-2 that their Get VAL UE configuration requests will be handled later.

NOTE:	How to decide that bulk configuration is used is implementation specific and out of scope.

Table 8.1.4-1: Information in the Get VAL UE configuration response to constrained device before bulk configuration is performed

| Information element | Status | Description |
| --- | --- | --- |
| Maximum configuration time | M | The maximum wait time for the bulk configuration request to be sent to the MSGin5G Server. |

In the MSGin5G UE bulk configuration procedure, the MSGin5G UE-3 acts as Configuration management client specified in clause 8.1.2 of the present document. In addition to the information elements listed in table 8.1.2-1 of the present document and clause 11.3.2.1 of 3GPP TS 23.434 [5], the Get VAL UE configuration request used for bulk configuration also includes the information elements as listed in table 8.1.4-2.

Table 8.1.4-2: additional Information in the Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Bulk configuration flag | M | Indicates this request is an enhanced Get VAL UE configuration request used for MSGin5G UE bulk configuration |
| List of MSGin5G UE IDs | M | List of MSGin5G UE ID of MSGin5G UE-bs which needed to be configured in this bulk configuration request |

In addition to the information elements listed in table 8.1.2-2 of the present document, the Get VAL UE configuration response used for bulk configuration also includes the information elements as listed in table 8.1.4-3 as a part of VAL UE configuration data as specified in clause 11.3.2.2 of 3GPP TS 23.434 [5].

Table 8.1.4-3: additional Information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| List of MSGin5G UE configuration information | M | Each element in this list contains information as specified in Table 8.1.2-2. The configuration information included in this IE is used to complete the configuration of the constrained devices, e.g. MSGin5G UE-2a and UE-2b. |

Editor's note:	The enhancement of VAL UE configuration request and response are needed to be captured by eSEAL2 WID and are FFS.

Upon receiving the Get VAL UE configuration response used for bulk configuration, the MSGin5G UE-includes the MSGin5G UE configuration information in the List of MSGin5G UE configuration information into multiple individual Get VAL UE configuration responses and sends them to the corresponding MSGin5G UE-2s.