3GPP TSG-SA WG6 Meeting #56	S6-232562

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232465)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0168 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the Message Aggregation at MSGin5G Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The inbound and outbound procedure at MSGin5G Server are revised to messages origination and messages termination procedures. When the Application-to-Point messages are needed to be aggregated at MSGin5G Server, the aggregation procedure is needed to be handled before the MSGin5G Server try to deliver the aggregated message. Therefore, the Message Aggregation is needed to be preformed at originating MSGin5G Server. This CR is proposed to clarify the this issue. Some corrections are also proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the Message Aggregation is needed to be preformed at originating MSGin5G Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the Message Aggregation is preformed is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: text related to clause 8.4.3 in CR S6-232261 is merged into this revision. The step 4c related to broadcast message is updated. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4.3	Message aggregation at MSGin5G Server

Figure 8.4.3-1 shows the procedure for the MSGin5G Server aggregating a set of Application-to-Point messages, set of group messages for the same MSGin5G group, or  a set of messages delivered based on Messaging Topic for the same Messaging Topic, each carrying a small amount data, targeted towards the same target MSGin5G UE(s).

NOTE 1:	Aggregation of multiple messages can also be done at the Application Server; in this case it is implementation specific and out of the scope of the current specification.

Figure 8.4.3-1: MSGin5G Server aggregates messages towards target MSGin5G UE

Figure 8.4.3-2 shows the procedure for MSGin5G Server aggregating a set of Application-to-Point messages, or a set of group messages for the same MSGin5G group, or a set of message delivered based on Messaging Topic for the same Messaging Topic, each carrying a small amount data targeted towards the same target Non-MSGin5G UE(s).

Figure 8.4.3-2: MSGin5G Server aggregates messages towards target Non-MSGin5G UE

Figure 8.4.3-3 shows the procedure for MSGin5G Server aggregating Broadcast messages, each carrying a small amount data targeted towards the target Broadcast Area.

Figure 8.4.3-3: MSGin5G Server delivers aggregated message towards Non-MSGin5G UEs in the Broadcast Area

The following procedure applies to the above figures 8.4.3-1, 8.4.3-2 and 8.4.3-3 with the exception that step 4a only applies to figure 8.4.3-1, step 4b only applies to figure 8.4.3-2 and step 4c only applies to figure 8.4.3-3.

1.	The Application Server initiates to send a set of Application-to-Point messages or a set of group messages for the same MSGin5G group or a set of messages delivered based on Messaging Topic for the same Messaging Topic and sends the request to MSGin5G Server. The MSGin5G Server executes the MSGin5G messages origination procedure for each message as specified in clause 8.3.2.

2.	The MSGin5G Server checks the message data size and the priority level of each message to determine if these messages, targeted to the same service end-point and of the same message type (i.e., either a Application-to-Point, a set of group messages for the same MSGin5G group or a set of messages delivered based on Messaging Topic for the same Messaging Topic), can be aggregated. For example, the MSGin5G Server finds that the messages have small payload size when compared to the maximum segment size that can be transmitted over available transport and are not high priority messages (i.e. the value of Priority type included in the message is not "High"), which could be sent as per scheduling policy towards a selected target.

NOTE 2:	MSGin5G Server decides to continue aggregating messages until optimal use of segment size before sending message towards MSGin5G Client 1.

3.	The MSGin5G Server aggregates multiple MSGin5G message requests intended for the target UE.The Aggregated message request contain the information elements as defined in Table 8.4.3-1 and Table 8.4.2-2. The MSGin5G Server delivers the Aggregated message request by using the MSGin5G messages termination procedure specified in clause 8.3.3.

Table 8.4.3-1: Aggregated message request (MSGin5G Server to MSGin5G Client)

| Information element | Status | Description |
| --- | --- | --- |
| Originating AS Service ID | M | The service identity of the sending Application Server. |
| Recipient UE Service ID | O | The service identity of the receiving MSGin5G Client. This IE in Mandatory for all type of message request except the broadcast message. |
| Group Service ID (see NOTE) | O | The service identifier of the target MSGin5G Group. This IE should be present if the aggregated message is a set of group messages for the same MSGin5G group. |
| Messaging Topic (see NOTE) | O | Indicates which Messaging Topic this message is related to. This IE should be present if the aggregated message is a set of message delivered based on Messaging Topic for the same Messaging Topic |
| Message ID | M | Unique identifier of this message |
| Number of individual messages | M | Indicates total number of messages which are aggregated into single message |
| List of Individual messages | M | Each element in this list contains information as specified in Table 8.3.3-1 |
| NOTE: Only one of these IEs shall be included to represent the type of message request. |  |  |

NOTE 3:	Total size of Aggregated message request is less than or equal to maximum segment size allowed to transmit over available transport.

4a.	If the recipient of the aggregated message is MSGin5G UE, the MSGin5G Client 1 residing on the MSGin5G UE splits the received Aggregated message request into multiple individual MSGin5G message requests. The content of each MSGin5G message is delivered to the recipient Application Client(s).

NOTE 4:	The delivery between MSGin5G Client and Application Client is out of scope of the present document.

4b.	If the recipient of the aggregated message is the Message Gateway on behalf of Non-MSGin5G UE, the Message Gateway splits the received Aggregated message request into multiple individual MSGin5G message requests. The content of each MSGin5G message is delivered to the Non-MSGin5G UE via Non-MSGin5G message delivery.

NOTE 5:	The Non-MSGin5Gmessage delivery is out of scope of this document.

NOTE 6:	The MSGin5G Server may aggregate messages towards the target UE if it receives messages from multiple MSGin5G UEs (instead of Application Server).

4c.	If the recipient of the aggregated message is a Broadcast Area, the Broadcast Message Gateway split the received aggregated message request into multiple individual MSGin5G message requests. The Broadcast Message Gateway broadcasts each individual MSGin5G message request to the MSGin5G UEs in the Broadcast Area, or broadcasts the content of each individual MSGin5G message request to the non-MSGin5G UEs in the Broadcast Area.via CBCF. The procedures on  CBCF are specified in 3GPP TS 23.041 [14].