3GPP TSG-SA WG6 Meeting #56	S6-232587

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232357)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0321 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on temporary and pre-arranged MCData Broadcast Groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, BDBOS, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In reviewing MCPTT support for pre-arranged broadcast groups and how authorize transmit privileges it was noticed that MCData does not have this support. In TS22.282 there is a requirement [R-6.2.2.1-001] - The MCData service shall determine which Participant(s) are allowed to transmit data. And TS 22.280 [R-5.2.1-001] The MCX Service shall support Broadcast Group Communications within that MCX Service from authorized MCX Service Group Members as determined by the MCX Service Administrator. This is currently not supported |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table A.3-1 to include support for 3GPP TS22.280 [R-6.6.3-002] and A.4.2 in TS23.282 to support pre-arranged broadcast groups to support TS22.282 [R-6.2.2.1-001] and TS 22.280 [R-5.2.1-001]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-1 Requirement is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial Revision |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# A.3	MCData user profile configuration data

The general aspects of MC service user profile configuration data are specified in 3GPP TS 23.280 [5]. The MCData user profile configuration data is stored in the MCData user database. The MCData server obtains the MCData user profile configuration data from the MCData user database (MCData-2).

Tables A.3-1 and A.3-2 contain the MCData user profile configuration required to support the use of on-network MCData service. Tables A.3-1 and A.3-3 contain the MCData user profile configuration required to support the use of off-network MCData service. Data in table A.3-1 and A.3-3 can be configured offline using the CSC-11 reference point.

Table A.3-1: MCData user profile configuration data (on and off network)

| Reference | Parameter description | MCData UE | MCData Server | Configuration management server | MCData user database |
| --- | --- | --- | --- | --- | --- |
| Subclause 8.1.2 of 3GPP TS 23.280 [5] | MCData identity (MCData ID) | Y | Y | Y | Y |
| 3GPP TS 33.180 [13] | KMSUri for security domain of MCData ID (see NOTE 1) | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [5] | Pre-selected MCData user profile indication (see NOTE 2) | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [5] | MCData user profile index | Y | Y | Y | Y |
| Subclause 5.2.4 of 3GPP TS 23.280 [5] | MCData user profile name | Y | Y | Y | Y |
| [R-5.17-007], [R-6.13.4-002] of 3GPP TS 22.280 [2] | User profile status (enabled/disabled) |  | Y | Y | Y |
| [R-5.7-001], [R-6.9-003] of 3GPP TS 22.280 [2] | Authorised to create and delete aliases of an MCData user and its associated user profiles. |  |  | Y | Y |
| [R-5.7-002], [R-6.9-003] of 3GPP TS 22.280 [2] | Alphanumeric aliases of user | Y | Y | Y | Y |
| [R-5.1.1-005], [R-5.9-001] of 3GPP TS 22.280 [2] | Participant type of the user | Y | Y | Y | Y |
| [R-5.1.8-006], [R-5.3-002], [R-5.9-001], [R-5.16.2-001], [R-5.16.2-002] of 3GPP TS 22.280 [2] | User's Mission Critical Organization (i.e. which organization a user belongs to) | Y | Y | Y | Y |
| [R-5.2.2-003] and [R6.6.3-002] of 3GPP TS 22.280 [2] | Authorisation to create a group-broadcast group (see NOTE 8) |  |  | Y | Y |
| [R-5.2.2-003] and [R6.6.3-002] of 3GPP TS 22.280 [2] | Authorisation to create a user-broadcast group (see NOTE 8) |  |  | Y | Y |
| [R-5.6.2.4.1-002] of 3GPP TS 22.280 [2] | Authorised to activate MCData emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-013] of 3GPP TS 22.280 [17] | Automatically trigger a MCData emergency communication after initiating the MCData emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-004] [R-5.6.2.4.1-008] [R-5.6.2.4.1-012] of 3GPP TS 22.280 [2] | Group used on initiation of an MCData emergency group communication (see NOTE 3) |  |  |  |  |
| [R-5.6.2.4.1-004], [R-5.6.2.4.1-008], [R-5.6.2.4.1-012] of 3GPP TS 22.280 [17] | Recipient for an MCData emergency private communication (see NOTE 3) |  |  |  |  |
|  | > MCData ID | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCData ID (see NOTE 1) | Y | Y | Y | Y |
| [R-5.6.2.4.2-002] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCData emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-002] of 3GPP TS 22.280 [2] | Authorised to activate an MCData ad hoc group emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-006] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCData ad hoc group emergency alert | Y | Y | Y | Y |
| [R-6.15.6.2-007] of 3GPP TS 22.280 [2] | Authorised to set up an MCData group communication using the ad hoc group used for the alert | Y | Y | Y | Y |
|  | Authorised to receive the participants information of an MCData ad hoc group emergency alert | Y | Y | Y | Y |
| [R-6.1.1.2-005], [R-6.1.1.2-006], [R-6.1.1.2-007] of 3GPP TS 22.282 [3] | Individual conversation hang time | Y | Y | Y | Y |
|  | One-to-one communication |  |  |  |  |
| [R-6.3.1.2-007] of 3GPP TS 22.282 [3] and 3GPP TS 33.180 [13] | > List of MCData users this MCData user is authorized to initiate a one-to-one communication |  |  |  |  |
|  | >> MCData ID | Y | N | Y | Y |
|  | >> Discovery Group ID | Y | N | Y | Y |
|  | >> User info ID (as specified in 3GPP TS 23.303 [7]) | Y | N | Y | Y |
|  | >> KMSUri for security domain of MCData ID (see NOTE 1) | Y | Y | Y | Y |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | Authorised to make one-to-one communications towards users not included in "list of MCData user(s) this MCData user is authorized to initiate a one-to-one communication" | Y | Y | Y | Y |
|  | File distribution |  |  |  |  |
| [R-5.3.2-010] of 3GPP TS 22.282 [3] and 3GPP TS 33.180 [13] | > List of MCData users this MCData user is allowed to cancel distribution of files being sent or waiting to be sent |  |  |  |  |
|  | >> MCData ID | Y | Y | Y | Y |
|  | >> KMSUri for security domain of MCData ID (see NOTE 1) | Y | Y | Y | Y |
|  | Transmission and reception control |  |  |  |  |
| [R-6.2.2.1-001] of 3GPP TS 22.282 [3] | > Whether the MCData user is permitted to transmit data | Y | Y | Y | Y |
| [R-6.2.3-005] of 3GPP TS 22.282 [3] | > Maximum amount of data that the MCData user can transmit in a single request during one-to-one communication | Y | Y | Y | Y |
| [R-6.2.3-005] and [R-6.3.1.2-008] of 3GPP TS 22.282 [3] | > Maximum amount of time that the MCData user can transmit in a single request during one-to-one communication | Y | Y | Y | Y |
| [R-6.2.3-001] of 3GPP TS 22.282 [3] | > List of MCData users this MCData user is allowed to request the release of an ongoing transmission that this MCData user is participating in |  |  |  |  |
|  | >> MCData ID | Y | Y | Y | Y |
| [R-5.1.7-002] and [R-6.8.7.2-007] and [R-6.8.7.2-008] of 3GPP TS 22.280 [2] | Priority of the user (see NOTE 4) |  | Y | Y | Y |
|  | Lossless communication for private communication | Y | Y | Y | Y |
|  | Store communication in Message Store (see NOTE 5) | Y | Y | Y | Y |
|  | Store private communication in Message Store (see NOTE 6) | Y | Y | Y | Y |
| [R-6.12-003] of 3GPP TS 22.280 [17] | Authorised to restrict the dissemination of the location information | Y | Y | Y | Y |
| Subclause 10.9 of 3GPP TS 23.280 [5] | Authorised to request location information of another user in the primary MCData system (see NOTE 7) | Y | Y | Y | Y |
| Subclause 10.9 of 3GPP TS 23.280 [5] | List of partner MCData systems for which user is authorised to request location information for another user |  |  |  |  |
|  | > Identity of partner MCData system | Y | Y | Y | Y |
| NOTE 1: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [5]. NOTE 2: As specified in 3GPP TS 23.280 [5], for each MCData user's set of MCData user profiles, only one MCData user profile shall be indicated as being the pre-selected MCData user profile. NOTE 3: This parameter is used for the emergency communication and also used as a target of the emergency alert request. At most one of them is configured; i.e. emergency communication will go to either a group or a user. If both are not configured the MCData user's currently selected group will be used. NOTE 4: The use of the parameter is left to implementation. NOTE 5: This is the top-level control parameter to determine whether MCData communications will be stored or not. When this parameter is set; the second level control parameter is used to determine whether a specific MCData communication (private or which group) will be stored and MCData user can request for all or selected of his/her MCData communication shall be stored in the MCData message store or not. NOTE 6: This is the second level control parameter to determine whether a private communication will be stored when the Store communication in Message Store top level control parameter is set. NOTE 7 : Further differentiation on authorisation for requesting location information based on detailed characterstics (e.g. MC organization, MC service ID, functional alias) is left to implementation. NOTE 8: This parameter applies to temporary broadcast groups built from regrouping mechanism. This authorisation automatically sets the originator of the temporary group as the only transmitting party. |  |  |  |  |  |

* * * Second Change * * * *

# A.4	MCData related Group configuration data

The general aspects of group configuration are specified in 3GPP TS 23.280 [5].

Parameters specified in table A.4-1 are child parameters of the "MCData configuration" parameter specified in table A.4-1 in 3GPP TS 23.280 [5]. Parameters specified in table A.4-2 are child parameters of the "MCData configuration" parameter specified in table A.4-2 in 3GPP TS 23.280 [5]. Parameters specified in table A.4-3 are child parameters of the "MCData configuration" parameter specified in table A.4-3 in 3GPP TS 23.280 [5].

Table A.4-1: Group configuration data (on and off network)

| Reference | Parameter description | MCData UE | MCData Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Media confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Transmission control confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Group media protection security material (see NOTE) | Y | N | Y |
| Subclause 5 | >> MCData sub-services and features enabled for the group |  |  |  |
|  | >>> Short data service enabled | Y | Y | Y |
|  | >>> File distribution enabled | Y | Y | Y |
|  | >>> IP connectivity enabled | Y | Y | Y |
|  | >>> Conversation management enabled | Y | Y | Y |
|  | >>> Transmission control enabled | Y | Y | Y |
|  | >>> Reception control enabled | Y | Y | Y |
|  | >>> Enhanced status enabled | Y | Y | Y |
|  | >> Enhanced status |  |  |  |
| [R-6.1.3.2-002] of 3GPP TS 22.282 [3] | >>> List of operational status values | Y | N | Y |
| [R-6.1.1.2-011] of 3GPP TS 22.282 [2] | >> Lossless communication | Y | Y | Y |
| [R-6.1.1.2-007] of 3GPP TS 22.282 [5] | >> Conversation hang time | Y | Y | Y |
| NOTE: Security mechanisms are specified in 3GPP TS 33.180 [13]. |  |  |  |  |

Table A.4-2: Group configuration data (on network)

| Reference | Parameter description | MCData UE | MCData Server | Group management server |  |
| --- | --- | --- | --- | --- | --- |
| [R-6.4.5-001], [R-6.4.5-003] of 3GPP TS 22.280 [2] | >> Authorisation of a user to request a list of affiliated members of a group | Y | Y | Y |  |
| [R-5.1.7-002], [R-6.2.2-001], [R-6.6.2.2-006], [R-6.8.7.2-003] of 3GPP TS 22.280 [2] | >> Priority of the group | N | Y | Y |  |
| Subclause 6.2.2 of 3GPP TS 22.282 [3] | >> Transmission and reception control |  |  |  |  |
|  | >>> Maximum data size for SDS | Y | Y | Y |  |
|  | >>> Maximum data size for FD | Y | Y | Y |  |
|  | >>> Maximum data size for auto-receive | N | Y | Y |  |
| 3GPP TS 23.283 [18] | >> Indication whether use of LMR E2EE is permitted on the MCData group | Y | N | Y |  |
| 3GPP TS 23.283 [18] | >> LMR specific identity for MCData group (see NOTE 1) | Y | N | Y |  |
| 3GPP TS 23.283 [18] | >> Group to key binding (see NOTE 1) | Y | N | Y |  |
| [R-6.2.2.1-001] of 3GPP TS 22.282 [3], [R-5.2.1-001] of 3GPP TS22.280 [2] | >> List of group members which are allowed to transmit data (see NOTE 2) |  |  |  |
|  | >>> MCData ID | Y | Y | Y |
| NOTE 1: This is an LMR specific parameter with no meaning within MC services. NOTE 2: This parameter sets the authorization to transmit data in pre-arranged groups used in broadcast calls. |  |  |  |  |  |

* * * End of Change * * * *