3GPP TSG-SA WG6 Meeting #56	S6-232589

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232413)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0417 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resources management procedures update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The messages and requests included in resources management related procedures are not consistent among each other. Having different messages which are the same and have the same purpose may cause confusion. Furthermore, in several locations the established session is referred to as a “call”, which is not entirely correct. A session is established which can be used for a call (communication) based on the service selected. Furthermore, there are request messages are not written correctly, and the reference to 3GPP TS 23.203 is fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - correcting the reference to 3GPP TS 23.203 to be [8] instead of [4]. - remove “call” in “MC call/session…” where not correct. - “request” is added to “session establishment”. - replace “call control, bearer modification request” with “session update request”. - enhance the session release part requested by the MC service server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current messages and procedures related to resources management are not consistent and may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11.2, 10.11.2a, 10.11.3, 10.11.5, 10.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - change “call/session” in clause to “session” - change “imminent threat” to “imminent-peril”. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.11.2	Request for unicast resources at session establishment

The procedure defined in this subclause specifies how network resources are requested at session establishment. If concurrent sessions are used the MC service server may utilize the capability of resource sharing specified in 3GPP TS 23.203 [8]. The request for resources is sent to the PCRF on the Rx reference point and includes media type, bandwidth, priority, application identifier and resource sharing information.

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 10.11.2-1 are the signalling procedures for the requesting resource at session establishment.

Figure 10.11.2-1: Resource request at session establishment

1.	MC service client sends a session establishment request.

2.	MC service server receives evaluates the need network resources and use of media resource sharing.

3.	MC service server send a session progress request containing request for resources.

4.	PCC procedures (as defined in 3GPP TS 23.203 [8]) initiated from SIP core local inbound/outbound proxy over Rx.

5.	The SIP core local inbound / outbound proxy forwards the call control protocol request to the MC service client.

6.	The MC service client acknowledges the session progress request with an OK message.

7.	The SIP core local inbound / outbound proxy forwards the OK message to the MC service server.

8.	The MC service session is established and resources have been allocated.

# * * * * Second change * * * *

### 10.11.2a	Request for unicast resources at session establishment from MC service server

#### 10.11.2a.1	General

The procedure defined in this clause specifies how network resources are requested at session establishment from the MC service server. The request for resources is sent to the PCRF via the Rx reference point from the MC service server and includes media type, bandwidth, priority, application identifier and resource sharing information. If concurrent sessions are used, the MC service server may utilize the capability of resource sharing specified in 3GPP TS 23.203 [8].

For the request of network resources by the MC service server via the Rx reference point, the MC service client provides to the MC service server the final access resource details (e.g. IP addresses and ports) of the MC service client and the media anchoring points.

This procedure is generic to any type of session establishment with the MC service server requesting network resources.

#### 10.11.2a.2	Procedure

Figure 10.11.2a.2-1 describes the procedure for the request of resources at session establishment from the MC service server.

Figure 10.11.2a.2-1: Resource request at session establishment from the MC service server

1.	The MC service client sends a session establishment request. The request includes, apart from the SDP offer, access resource details, e.g. IP addresses and ports of the MC service client related to the media session.

2.	The MC service server evaluates the need of network resources and use of media resource sharing.

3.	The MC service server sends a session progress request to the SIP core.

NOTE:	The session progress request does not include a request for network resources to be performed by the SIP core.

4.	The SIP core local inbound / outbound proxy forwards the session progress request to the MC service client.

5.	The MC service client acknowledges the session establishment to the MC service server. This message contains the final negotiated media access parameters, e.g. IP addresses and ports related to the media anchoring points received in the SDP answer from the SIP core.

6.	The MC service server sends a request for media resources to the PCRF over Rx (as defined in 3GPP TS 23.203 [8]).

7.	The MC service session is established, and resources have been allocated.

# * * * * Third change * * * *

### 10.11.3	Request for modification of unicast resources

To modify unicast media bearers, the MC service server shall send a resource modification request containing the parameters to be modified, using the call control protocol via the SIP core to the UE.

Possible scenarios when this procedure may be used are:

-	Modify the allocation and retention priority for unicast resources;

-	Release and resume resources in-between MC service calls when using the chat model; or

-	Release and resume resources when a UE is able to receive the MC service call over multicast transmission

Releasing resources for the media plane should give the option to allow the SIP session to either be torn down or continue.

Procedures in figure 10.11.3-1 are the signalling procedures for the modification of a unicast:

Pre-conditions:

-	An MC service  session is already in progress;

Figure 10.11.3-1: Bearer modification request

1.	MC service server decides to modify the parameters of a unicast bearer (e.g. a request to upgrade the existing MC service call to an MC service emergency or imminent-peril call).

2.	MC service server sends a session update which includes a resource modification request containing the modified parameters of the unicast bearer.

3.	PCC procedures (as defined in 3GPP TS 23.203 [8]) initiated from SIP core local inbound/outbound proxy over Rx.

4.	The SIP core local inbound / outbound proxy forwards the session update request to the MC service client.

5.	The MC service client acknowledges the call control protocol request with an OK message.

6.	The SIP core local inbound / outbound proxy forwards the OK message to the MC service server.

7.	The MC service session continues with the modified unicast resources.

NOTE 1:	If the MC service call is transferred to multicast transmission, the unicast resources could be temporarily released.

NOTE 2:	If multiple audio streams are sent to the UE, additional bearer resources could be required during an established session. Pre-allocation of additional bearer resources already at session establishment could be useful.

# * * * * Fourth change * * * *

### 10.11.5	Request for resources with shared priority

#### 10.11.5.1	General

An MC service server that supports simultaneous sessions may need to share a common priority on the EPS bearer for several MC service group calls that may use different priorities on the application level. This is achieved by including information for priority sharing on the request of resources over Rx reference point to PCRF. All sessions associated with the same priority sharing information will be handled by one EPS bearer, which will have a priority based on the highest requested priority among the sessions.

The use of the procedure defined in subclause 10.11.5.2 is dependent on operator policy.

#### 10.11.5.2	Procedure

Pre-conditions:

-	All previous resource requests from the MC service have included a priority sharing information.

Figure 10.11.5.2-1: Resource request including priority sharing information

1.	The MC service server decides based on a request from the MC service client that the priority of an ongoing call must be adjusted.

2.	The MC service server requests a session update to the SIP core. This request will contain information of priority sharing.

3.	PCC update procedures (as defined in 3GPP TS 23.203 [8]) initiated from SIP core local inbound/outbound proxy over Rx. PCC updates the bearer priority for the bearer that contains the SDFs associated with the same priority sharing information. The priority is set to highest priority (lowest ARP) among those SDFs. No additional bearer is created. Also, the default bearer priority is updated accordingly.

4.	The session update is forwarded to the MC service client.

NOTE 1:	The procedure defined above requires a PCC enhancement and is subject to implementation in EPC and IMS and can therefore only be used if supported by EPC and IMS.

NOTE 2:	If the EPC and IMS does not support the priority sharing from MC system, no shared priority treatment applies, and a new EPS bearer will be setup based on QCI/ARP combination.

# * * * * Fifth change * * * *

### 10.11.6	Request for media resources from MC service server

#### 10.11.6.1	General

The procedure in this sub clause specify how request for resource for floor control (or transmission control in MCVideo and MCData) and for the media plane can be handled independently. This procedure utilizes the Rx reference point both between from the MC service server to the PCRF and from the SIP core to the PCRF.

Resource for the transmission control is requested at session establishment, in this case the IMS standard procedures as specified in 3GPP TS 23.228 [9] are used. The session description shall in this procedure include bandwidth information applicable for the transmission control traffic requirement. At group call setup the request for resources for the media plane is triggered. This request is sent directly from the MC service server to the PCRF.

The procedure is optional and is suitable when the procedures for pre-established sessions are used, it may also be used to setup and tear down the media plane used by between consecutive group calls in one communication session using the chat call model.

#### 10.11.6.2	Procedure

The figure below illustrates the procedure for resource reservation

Figure 10.11.6.2-1: Request for resources for transmission control and media plane

1.	The MC service client sends a request for group affiliation.

2.	The MC service client sends a request to the MC service server for establishment of a communication session.

3.	The MC service server answer the session establishment request and adjust the bandwidth information in the session description. The requested bandwidth shall be minimized to cover the bandwidth requirements for floor control signaling (or transmission control for MCVideo or MCData).

4.	The SIP core request resources towards the PCRF according to the session establishment request.

5.	The session establishment request is completed and a response is sent towards the MC service client.

6.	The MC service client sends a cell setup message according to existing procedures.

7.	The MC service server sends a session establishment request for resources for the media plane to PCRF, and the media plane is by that established. This request includes media description relevant for the media plane.

8.	Group call is ongoing on the group communication session.

9.	The MC service serve sends a release of media resources to PCRF, and the media plane is by that terminated.

10.	The PCRF performs session termination procedure according to 3GPP TS 23.203 [8].

11. The PCRF sends a session release response back to the MC service server.

NOTE 1:	The resources for transmission control are retained.

NOTE 2:	Step 6-9 can be repeated several times within the life cycle of one communication session.

# * * * * End of changes * * * *