3GPP TSG-SA WG6 Meeting #56	S6-232591

Gothenburg, Sweden, 21st – 25th August 2023	(revision of S6-232463)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0367 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revoke the floor by the authorized user in multi-talker case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Base on the SA1 requirement “[R-6.2.3.7.2-003] The MCPTT Service shall determine which Participant(s) are allowed to transmit to all other Participant(s) in an MCPTT Group configured for multi-talker control.” multipatipants are allowed to talk simultaneously. However, the authorized user may needs to “mute” certain participants due to e.g., too much background noise which imapct the other listeners of the group communication. How to stop the talker from transmitting to all participants is not support yet in TS 23.379. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure to allow the authorized user to revoke the floor. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No means to stop the talker from transmitting to all the participants in a group communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)10.9.1.2.aa, (new)10.9.1.3.x, 10.9.1.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

10.9.1.2.aa	floor revoke request

Table 10.9.1.2.aa-1 describes the information flow floor revoke request, from the floor participant to floor control server, which is used to revoke the floor from its current holder (the floor participant who was granted the floor).

Table 10.9.1.2.aa-1: Floor revoke request

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the user who requests to revoke the floor from another talker |
| MCPTT ID | M | Revoked party identity |
| Functional alias | O | Functional alias of the requester |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

# /************************ Next change ******************************/

10.9.1.2.8	Floor idle

Table 10.9.1.2.8-1 describes the information flow floor idle, from the floor control server to the floor participant, which is used to indicate that the floor is available. This information flows is sent in unicast or broadcast.

Table 10.9.1.2.8-1: Floor idle

| Information element | Status | Description |
| --- | --- | --- |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the floor participant is required |

# /************************ Next change ******************************/

##### 10.9.1.3.x	Floor revoke by the authorized user

Figure 10.9.1.3.x-1 shows the high level procedure that the floor granted to a talker user is revoked by the authroized user. Only two UEs involved in the session are shown on the figure for the simplicity.

Pre-condition:

1.	The participant B is granted the floor and transmitting the voice media to the group.

2.	For multi-talker case, there may be other talkers in the group communication.

Floor participant A

(Authorized user)

Floor control server

Floor participant B

(talker)

1. Floor revoke request

3a. Floor revoked

2. Authorization check

MCPTT user at floor participant B is transmitting voice media to the group

3b. Notification to the user

4b. Notification to the user

Floor participant C

(listener)

4a. Floor idle

4a. Floor idle

4b. Notification to the user

Figure 10.9.1.x.1-1: Floor revoke by authorized user

1.	The authorized user at floor participant A sends floor revoked message to revoke the floor of floor participant B. The identity of the floor participant B whose floor is to be revoked is included.

2.	The floor control server checks the floor participant A is authorized to perform the floor revoke.

3.	The floor control server sends the floor revoked message towards the floor participant B. and the floor participant B notifies the MCPTT server about the floor is revoked.

4.	The floor control server sends the floor idle to the other participants, and the participants may notify the MCPTT users accordingly.