3GPP TSG-SA WG6 Meeting #56	S6-232593

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232289)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 23.280 | CR | 0410 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Functional model for sharing administrative configuration between interconnected MC systems |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCShAC |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR provides the functional model for ‘sharing administrative configuration between interconnected MC systems’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subclauses describing the functional model, changes to clause A.3 and required authorization for clients in this model are added to TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functional model is the basis of sharing administrative configuration between interconnected MC systems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New chapter, A.3, New chapter |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## X.2	Functional Model

### X.2.1	General

To allow required exchange of configuration data with a partner MC System, an ACM server and an ACM client are added to the functional model of the MC system. The transfer of administrative configuration between two MC systems uses the interconnection interface and reference points as specified in TS 23.280.

### X.2.2	Functional model

Figure X.2.2-1 shows the generic ACMX functional model. It comprises ACM clients connected to ACM servers at each MC system. Primary and Partner MC system’s ACM functional entities are connected through ACM servers by each side.

- NOTE1:	An ACM client can either be an an integral functional entity of the configuration management client (refer to Figure 7.4.3.1-1: Functional model for application plane for an MC system) or an functional entity of its own

- NOTE2: An ACM server can either be an integral functional entity of the configuration management server (refer to Figure 7.4.3.1-1: Functional model for application plane for an MC system) or an functional entity of its own.

Each exchange of administrative configuration data will be triggered by an ACM client sending a request to its ACM server. The server in return checks the client’s authorization and then forwards the request for exchange to the partner MC system’s ACM server. This receiving ACM server checks the requesting client’s authorization and starts processing its request. Based on service agreements negotiated beforehand for these MC systems, the ACM server will process the request either by its own or with decision taking participating ACM client. The result of the requested exchange of administrative configuration data will be returned back by the partner MC system’s ACM server to primary MC system’s ACM server. This server then sends the result to the originating ACM client.

MC service users (e.g dispatcher) need dedicated authorization for using ACM clients. The user profiles of such MC service users shall reflect the necessary permissions and priviliges for requesting exchange of administrative configuration data at partner MC system.

Figure X.2.2-1 Generic ACMX functional model

The ACM client has the following tasks:

- Requesting exchange of administrative configuration data at partner MC system by sending this request via its connected ACM server to an addressed ACM client in the partner MC system.

- Decision taking on received requests from partner MC system. The client may accept or reject the request and will instruct its attached ACM server to handle the received requests accordingly.

The ACM server has the following tasks:

- Receiving request for exchange of administrative configuration data from its ACM clients

- Checking authorizations of its ACM clients having sent a request.

- Forwarding received requests of ACM clients to the ACM server in the partner MC system.

- Receiving requests from a partner MC system’s ACM server and checking the sender’s authorization.

- Storing the received request

- Processing the request on its own based on pre-defined automated rules agreed between interconnected MC systems or forwarding the request to an available decision taking ACM client and executing the ACM client’s instruction.

- Forwarding the result of the request back to the ACM server of the originating MC system.

- Forwarding received result of an sent request from partner MC system to the originating ACM client

### X.2.3	ACM client authorization

Figure X.2.1-1 Generic ACMC functional model shows both ACM client as authorized users respectively as administrators.

Before an ACM client can start using ACMX two topics have to be addressed:

- The ACM client’s MC service ID shall have user profile configuration data assigned which gives evidence on required permissions and privileges. This user profile configuration can be found in Table A.X in Annex A.X

- The ACM server will check the ACM client’s authorisation prior to any exchange with partner MC system’s ACM server.

* * * Second Change * * * *

# A.3	MC service user profile configuration data

The MC service user profile configuration data is stored in the MC service user database. The configuration management server is used to configure the MC service user profile configuration data to the MC service user database (CSC-13) and MC service UE (CSC-4).

MC service user profile configuration data can be configured offline using the CSC-11 reference point.

MC service user profile configuration data is specific to each MC service and is specified as follows:

-	MCPTT user profile configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo user profile configuration data is specified in 3GPP TS 23.281 [12];

-	MCData user profile configuration data is specified in 3GPP TS 23.282 [13]; and

-	Location user profile configuration data is specified in clause A.8.

-	ACM Client user profile configuration data is specified in clause A.X

* * * Third Change * * * *

# A.X	ACM client user profile configuration data

The ACM client user profile configuration data is stored at the MC system’s ACM server. The authorisations of requesting ACM clients will be verified based on valid entries in Table A.X.

Table A.X: MC service user profile configurations for authorisation of ACMX (on-network)

| Reference | Parameter description | ACMC | ACMS |
| --- | --- | --- | --- |
| Subclause 5.16.4 of 3GPP TS 22.280 [2] | Authorised to perform ACM transactions | Y | Y |
| Subclause 5.16.4 of 3GPP TS 22.280 [2] | List of partner MC systems for which user is authorised to perform ACM transactions |  |  |
|  | > Identity of partner MC system | Y | Y |

* * * End of Change * * * *