3GPP TSG-SA WG6 Meeting #56	S6-232597

Goteborg, Sweden 21st – 25th August 2023	(revision of S6-232355)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0320 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify list of participants by changing the criteria during an ongoing MCData ad hoc group communication (single MC system) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current standard does not allow to update the participants list of an ongoing communication by changing the criteria, only adding and removing of specific participants is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The modify ad hoc group communication participants request and modify ad hoc group call participants response messages are updated. The procedure on modification of ad hoc group communication participants by the initiator is enhanced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of ad hoc group call participants cannot be modified during an ongoing communication by changing the criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 7.17.2.20, (new) 7.17.2.21, (new) 7.17.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.17.2.20	Modify ad hoc group data session criteria request (MCData client – MCData server)

Table 7.17.2.20-1 describes the information flow Modify ad hoc group data session criteria request from the MCData client to the MCData server.

Table 7.17.2.20-1: Modify ad hoc group data session criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the MCData user who is requesting for modification of ad hoc group data session participants |
| Functional alias | O | The associated functional alias of the MCData user who is requesting for modification of ad hoc group data session participants |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 7.17.2.21	Modify ad hoc group data session criteria response (MCData server – MCData client)

Table 7.17.2.21-1 describes the information flow Modify ad hoc group data session criteria response from the MCData server to the MCData client.

Table 7.17.2.21-1 Modify Ad hoc group data session criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the MCData user who is requesting for modification of ad hoc group data session participants |
| Functional alias | O | The associated functional alias of the MCData user who is requesting for modification of ad hoc group data session participants |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group data communication participants request (success or failure) |

# * * * * Second change * * * *

##### 7.17.3.1.6	Modification of ad hoc group data communication criteria by the initiator

Figure 7.17.3.1.6-1 below illustrates the modification of ad hoc group data communication criteria procedure by the initiator of the ad hoc group data communication.

Pre-conditions:

1.	The MCData client 1 is the initiator of the ad hoc group data communication.

2.	The MCData users on MCData client 1, MCData client 3 to MCData client n are on an ongoing ad hoc group data communication.

3.	MCdata user 1 requests to modify the criteria for determining the list of participants. MCData client 3 does not meet the modified criteria while MCData client 2 meets the modified criteria.

Figure 7.17.3.1.6-1: Modification of ad hoc group data communication criteria by the initiator

1.	The MCData client 1 sends the modify ad hoc group data session criteria request to the MCData server which contains an updated criteria to determine the list of participants.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group data communication. The MCData server determines the list of ad hoc group call participants based on the criteria provided and indentifies that MCData client 3 is to be removed from and MCData client 2 is to be added to the group.

3.	The MCData server sends modify ad hoc group data session criteria response to the MCData client 1.

4.	The MCData server sends the ad hoc group data session leave request to the MCData client 3 in order to remove it from the on-going ad hoc group data communication.

5.	The MCData client 3 notifies the user of the ad hoc group data session leave request.

6.	The MCData client 3 sends the ad hoc group data session leave response to the MCData server.

7.	The MCData server removes the MCData client 3 from the ongoing ad hoc group data communication.

8.	The MCData server sends the ad hoc group data session request towards MCData client 2.

NOTE 1:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCData client 2 notifies the user about the incoming ad hoc group data communication.

10.	The MCData client 2 accepts the ad hoc group data session request and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

11.	The MCData server may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

12.	The MCData server may notify the participants about the change in the participants list of on-going ad hoc group data communication.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *