3GPP TSG-SA WG6 Meeting #56	S6-232598

Goteborg, Sweden 21st – 25th August 2023	(revision of S6-232356)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0413 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify list of participants by changing the criteria during an ongoing ad hoc group emergency alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Kontron Transportation France, UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current standard does not allow to update the participants list of an ongoing emergency alert by changing the criteria. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Message flows and procedures are added to allow the change of the criteria of an ongoing adhoc group emergency alert. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of ad hoc group emergency alert participants cannot be modified during an ongoing alert by changing the criteria. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.10.3.2.12, (new) 10.10.3.2.13, (new) 10.10.3.3.5, (new) 10.10.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.10.3.2.12	Ad hoc group emergency alert modify request

Table 10.10.3.2.12-1 describes the information flow ad hoc group emergency alert modify request from the MC service client to the MC service server and from the MC service server to the MC service server.

Table 10.10.3.2.12-1 Ad hoc group emergency alert modify request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| Ad hoc group ID | M | The group ID used for the ad hoc group alert |
| Location information | O | The alerting MC service client's location |
| Criteria to determine the list of participants | M | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MC service server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |
| Additional information | O | The alerting party may provide additional information related to the alert, e.g., medical help needed or reason for the alert |

##### 10.10.3.2.13	Ad hoc group emergency alert modify response

Table 10.10.3.2.13-1 describes the information flow ad hoc group emergency alert modify response from the MC service server to the MC service client and from the MC service server to the MC service server.

Table 10.10.3.2.13-1 Ad hoc group emergency alert modify response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| MCPTT ad hoc group ID | M | The MCPTT group ID of ad hoc group whose criterion for determining the participants is modified |
| Result | M | Indicates the success or failure for the result |

# * * * * Second change * * * *

##### 10.10.3.3.5	Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert

During an ongoing ad hoc group emergency alert, the alerting user modifies the criteria for determining the list of participants of the ad hoc group emergency alert. The MC service server checks whether additional MC service users meet the modified conditions or whether other MC service users don’t meet the conditions any longer.

As illustrated in Figure 10.10.3.3.5-1 MC service client 2 is added to the ad hoc group as the MC service client 2 meets the modified criteria for receiving an ongoing ad hoc group emergency alert, whereas MC service client 3 is removed from the ad hoc group as the MC service client 3 does not meet the modified criteria for receiving the ongoing ad hoc group emergency alert.

Pre-conditions:

1.	An ad hoc group emergency alert has been initiated by MC service client 1 and the related ad hoc group exists.

2.	The MC service user on MC service client 1 is authorized to modify the criteria for determining the list of participants by the MC service server.

Figure 10.10.3.3.5-1: Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert

1.	The MC service user at the MC service client 1 wishes to modify the criteria for determining the list of ad hoc group emergency alert participants.

2.	The MC service server receives the ad hoc group emergency alert modify request.

3.	The MC service server checks whether MC service client 1 is authorized to modify the criteria which determines the list of ad hoc group participants.

4.	The MC service server analyses the modified criteria and identifies that MC service client 3 does not meet the modified criteria while MC service client 2 meets the modified criteria. The ad hoc group is updated with the determined user list as a member of the group.

4a.	MC service server sends an ad hoc group emergency alert request towards MC service client 2.

4b.	The MC service user 2 is notified of entering an ongoing ad hoc group emergency alert.

4c.	MC service client 2 sends an ad hoc group emergency alert response towards MC service server.

4d.	MC service server sends an ad hoc group emergency alert cancel request towards MC service client 3.

4e.	The MC service user 3 is notified of leaving an ongoing ad hoc group emergency alert.

4f.	MC service client 3 sends an ad hoc group emergency alert response towards MC service server.

5.	The MC service server sends an ad hoc group emergency alert modify response back to MC service client 1, which indicates whether the modify request was successful.

6.	The MC service server adds MC service client 2 to the ad hoc group emergency alert and removes MC service client 3 from the ad hoc group emergency alert.

NOTE:	If the ad hoc group emergency alert is associated with an ad hoc group call and the change of criteria caused the modification of the ad hoc group emergency alert participant list then the ongoing ad hoc group call is modified accordingly.

# * * * * Third change * * * *

##### 10.10.3.4.4	Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert

Figure 10.10.3.4.4-1 illustrates when the alerting user modifies the criteria for determining the list of participants of an ongoing ad hoc group emergency alert between multiple MC systems. This includes a scenario of MC service users entering and leaving an ongoing ad hoc emergency alert.

Preconditions:

-	An ad hoc group emergency alert has been initiated by MC service user 1 in both MC system A and MC system B, and a related ad hoc group exists.

-	The MC service server A and MC service server B are aware of the criteria sent by MC service client 1 related to the ongoing ad hoc group emergency alert.

-	MC system A and MC system B are interconnected.

Figure 10.10.3.4.4-1: Modifying the criteria for determining the participants during an ongoing ad hoc group emergency alert between multiple MC systems

- An ad hoc group emergency alert has been established based on a set of criteria sent by MC service client 1 upon initiating the ad hoc emergency alert.

2.	The MC service user at the MC service client 1 wishes to modify the criteria for determining the list of ad hoc group emergency alert participants.

3a/b.	The MC service server A receives the ad hoc group emergency alert modify request and forwards the request to MC service server B at MC system B.

4.	MC service server B detects that MC service client 3 meets the criteria for the ongoing ad hoc group emergency alert initiated at MC system A.

4a.	MC service server B sends an ad hoc group emergency add user notification message towards MC service server A.

4b.	MC service server A sends an ad hoc group emergency alert request towards MC service client 3.

4c.	The MC service user 3 is notified of entering an ongoing ad hoc group emergency alert.

4d.	MC service client 3 sends an ad hoc group emergency alert response towards MC service server A.

4e.	MC service server B sends an ad hoc group emergency remove user notification message towards MC service server A.

4f.	MC service server A sends an ad hoc group emergency alert cancel request towards MC service client 4.

4g.	The MC service user 4 is notified of leaving an ongoing ad hoc group emergency alert.

4h.	MC service client 4 sends an ad hoc group emergency alert response towards MC service server A.

5a/b.	The MC service server B sends an ad hoc group emergency alert modify response back to MC service client 1 in MC system A, which indicates whether the modify request was successful.

6.	The MC service server A adds MC service client 3 to the ad hoc group emergency alert and removes MC service client 4 from the ad hoc group emergency alert.

NOTE:	If the ad hoc group emergency alert is associated with an ad hoc group call and the change of criteria caused the modification of the ad hoc group emergency alert participant list then the ongoing ad hoc group call is modified accordingly.

# * * * * End of changes * * * *