3GPP TSG-SA WG6 Meeting #56	S6-232639

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232348)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0432 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | APIs for EDGE-5 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Removal of EN in Clause 8.14.4 Editor's note: API details are FFS. It is requried to define APIs for EDGE-5 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added APIs for all procedrues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EDGE-5 interface procedures will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, 8.14.1, 8.14.4, 8.14.4.1 (new), 8.14.4.2 (new), 8.14.4.2.1 (new), 8.14.4.2.2 (new), 8.14.4.2.3 (new), 8.14.4.3 (new), 8.14.4.3.1 (new), 8.14.4.4 (new), 8.14.4.4.1 (new), 8.14.4.5 (new), 8.14.4.5.1 (new), 8.14.4.5.2 (new), 8.14.4.5.3 (new), 8.14.4.5.4 (new), 8.14.4.6 (new), 8.14.4.6.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES | 8.8.3.3 |

Table 6.7.2-2 summarizes the APIs exposed the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TargetEASDiscovery | EAS, EES | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EAS | 8.8.3.6 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES | 8.9.4.3 |
| Eees_SelectedTargetEAS | EAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS | 8.8.3.8 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-x summarizes the APIs exposed the EEC.

Table 6.7.2-x: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

* * * Next Change * * * *

### 8.14.1	General

EEC exposes EDGE-5 APIs corresponding to EEC's capabilities, for the AC to request EEC's services for edge enablement. Using these APIs, ACs request the EEC for EEL services. EDGE-5 APIs include one-time request/response operations for EAS discovery, retrieval of UE ID and ACR operations. Additionally, the AC can request for an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

Editor's note:	[SA3] How to ensure of user's authorization/consent as well as AC’s authorization in invoking functions exposed by EEC (to AC) which in turn relies on functions exposed by the network (e.g. Location) via EES/NEF is SA3’s responsibility.

* * * Next Change * * * *

### 8.14.4	APIs

* * * Next Change * * * *

#### 8.14.4.1	General

Table 8.14.4.1-1 illustrates the API exposed by EEC.

Table 8.14.4.1-1: EEC API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eeec_ACRegistration | Request | Request/Response | AC |
|  | Update |  |  |
|  | Deregister |  |  |
| Eeec_EASDiscovery | Request | Request/Response | AC |
| Eeec_ACRTrigger | Request | Request/Response | AC |
| Eeec_Services | Subscribe | Subscribe/Notify | AC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eeec_UEId | Request | Request/Response | AC |

#### 8.14.4.2	Eeec_ACRegistration API

##### 8.14.4.2.1	Eeec_ACRegistration_Request operation

API operation name: Eeec_ACRegistration_Request

Description: The consumer requests to register the AC on the EEC.

Inputs: See clause 8.14.3.2.

Outputs: See clause 8.14.3.3.

See clause 8.14.2.2.2 for details of usage of this operation.

##### 8.14.4.2.2	Eeec_ACRegistration_Update operation

API operation name: Eeec_ACRegistration_Update

Description: The consumer requests to update the registered information of the AC on the EEC.

Inputs: See clause 8.14.3.4.

Outputs: See clause 8.14.3.5.

See clause 8.14.2.2.3 for details of usage of this operation.

##### 8.14.4.2.3	Eeec_ACRegistration_Deregister operation

API operation name: Eeec_ACRegistration_Deregister

Description: The consumer requests to de-register the AC from the EEC.

Inputs: See clause 8.14.3.6.

Outputs: See clause 8.14.3.7.

See clause 8.14.2.2.4 for details of usage of this operation.

#### 8.14.4.3	Eeec_EASDiscovery API

##### 8.14.4.3.1	Eeec_EASDiscovery_Request operation

API operation name: Eeec_EASDiscovery_Request

Description: The consumer requests for one time EAS discovery information.

Inputs: See clause 8.14.3.8.

Outputs: See clause 8.14.3.9.

See clause 8.14.2.3 for details of usage of this operation.

#### 8.14.4.4	Eeec_ACRTrigger API

##### 8.14.4.4.1	Eeec_ACRTrigger_Request operation

API operation name: Eeec_EASDiscovery_Request

Description: The consumer requests to trigger ACR.

Inputs: See clause 8.14.3.10.

Outputs: See clause 8.14.3.11.

See clause 8.14.2.4 for details of usage of this operation.

#### 8.14.4.5	Eeec_Services API

##### 8.14.4.5.1	Eeec_Services_Subscribe operation

API operation name: Eeec_Services_Subscribe

Description: The consumer subscribes for EEC Services related events.

Inputs: See clause 8.14.3.12.

Outputs: See clause 8.14.3.13.

See clause 8.14.2.5.2 for details of usage of this operation.

##### 8.14.4.5.2	Eeec_Services_Notify operation

API operation name: Eeec_Services_Notify

Description: The consumer is notified about EEC Services related events.

Inputs: See clause 8.14.3.14.

Outputs: None.

See clause 8.14.2.5.3 for details of usage of this operation.

##### 8.14.4.5.3	Eeec_Services_UpdateSubscription operation

API operation name: Eeec_Services_UpdateSubscription

Description: The consumer updates an existing subscription for EEC Services related events.

Inputs: See clause 8.14.3.15.

Outputs: See clause 8.14.3.16.

See clause 8.14.2.5.4 for details of usage of this operation.

##### 8.14.4.5.4	Eeec_Services_Unsubscribe operation

API operation name: Eeec_Services_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed EEC Services related events.

Inputs: See clause 8.14.3.17.

Outputs See clause 8.14.3.18.

See clause 8.14.2.5.5 for details of usage of this operation.

#### 8.14.4.6	Eeec_UEId API

##### 8.14.4.6.1	Eeec_UEId_Request operation

API operation name: Eeec_UEId_Request

Description: The consumer requests for EAS specific UE ID.

Inputs: See clause 8.14.3.19.

Outputs: See clause 8.14.3.20.

See clause 8.14.2.6 for details of usage of this operation.

* * * End of Change * * * *