3GPP TSG-SA WG6 Meeting #56	S6-232651

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232387)

Source:	Nokia, Nokia Shanghai Bell, Intel

Title:	General content regarding alignment between EDGEAPP and GSMA OP

Spec:	3GPP TR 23.958 v0.5.0

Agenda item:	7.9

Document for:	Agreement

Contact:	Diego Preciado (diego.preciado_rojas@nokia.com)

1. Introduction

GSMA Operator Platform (OP) provides a common solution for operators for exposing network and edge capabilities to enterprise customers and application developers. Through the introduction of a multi-layer architecture with multi-role separation of the OP functionalities, it is aimed to achieve interoperability among several OPs and developers. Additionally, through the introduction of the federation concept, the OP allows an application developer to connect to a single platform (the leading OP), and get access to the capabilities/resources exposed for the partners OP(s), i.e. the other members of the federation, enabling in that manner several service scenarios (e.g. Edge Node Sharing).

2. Reason for Change

To provide updated references relevant to GSMA OP. To add of new relevant abbreviations. To provide a consistent description of the content in the main OP Permanent Reference Documents, as well as a gentle introduction to Section 6.1. related to the aligment between EDGEAPP and GSMA OP.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958.

* * * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 5.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.02-v5.0-Operator-Platform-Requirements-and-Architecture.pdf.

[4]	ETSI GS MEC 003 (V3.1.1), "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	ETSI GS MEC 011 (V3.1.1), “Multi-access Edge Computing (MEC); Edge Platform Application Enablement”.

[6]	ETSI GS MEC 040 (V3.1.1), “Multi-access Edge Computing (MEC); Federation enablement APIs”.

[7]	ETSI GS MEC 021 V2.2.1, "Multi-access Edge Computing (MEC); Application Mobility Service API".

[8]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[9]	ETSI GS MEC 010-2 (V2.2.1), "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[10]	3GPP TS 23.222: “Common API Framework for 3GPP Northbound APIs”.

[11]	ETSI GR MEC 031 (V3.1.1), “Multi-access Edge Computing (MEC); MEC 5G Integration”.

[12]	GSMA PRD OPG.03 - “Southbound Interface Network Resources APIs Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.03-v3.0-Southbound-Interface-Network-Resources-APIs.pdf.

[13]	GSMA PRD OPG.04 - “East-Westbound Interface APIs Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.04-v3.0-GSMA-Operator-Platform-Group-East-Westbound-Interface-APIs-Version-3.0.pdf.

[14]	GSMA PRD OPG.05 - “User-Network Interface APIs Version 1.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/03/GSMA-Operator-Platform-Group-User-Network-Interface-APIs-v1.pdf.

* * * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

API	Application Program Interface

APF	API Publishing Function

AS	Application Server

CAPIF	Common API Framework

CCF	CAPIF Core Function

CER	Capabilities Exposure Role

EAS	Edge Application Server

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EEL	Edge Enabler layer

EES	Edge Enabler Server

ETSI	European Telecommunications Standards Institute

MEC	Multi-access Edge Computing

MEP	MEC Platform

NaaS	Network-as-a-Service

NSaaS	Network Slice-as-as-Service

OP	Operator Platform

OPG	Operator Platform Group

SBI-NR	Southbound Interface-Network Resources

SNMP	Stand-Alone Non-Public Network

SRM	Service Resource Manager

UC	User Client

UNI	User-to-Network Interface

* * * * Next Change * * * *

# 4	Related work in other SDOs

This section includes the existing/on-going work in GSMA/ETSI MEC which is relevant to the alignment work with EDGEAPP e.g., about OP roles, existing mapping/alignment aspects captured by GSMA OP/ETSI MEC,

## 4.1	General

## 4.2	GSMA OPG

The Operator Platform (OP), as defined by GSMA OPG in GSMA PRD OPG.02 [3], guides the industry to define a common solution for exposing network capabilities and edge compute resources to enterprise customers and developers allowing monetization of those capabilities. The technical requirements, functional blocks and interfaces characteristics of such a generic platform to facilitate access to the Edge Cloud capability of an operator or federation of operators and their partners are provided in GSMA PRD OPG.02 [3]. GSMA PRD OPG.02 [3] also provides mapping of these requirements and architectures to the specifications from certain SDOs.

The integration of OP with the mobile network takes place using the Southbound Interface-Network Resources (SBI-NR) through the set of APIs specified in GSMA PRD OPG.03 [12] which is linked to the corresponding 3GPP-defined APIs. GSMA PRD OPG.04 [13] specifies the set of APIs for the interactions/operations over the East/West Bound Interface (E/WBI), which enables an OP to share network and edge capabilities with other OP(s). The E/WBI interactions enable the management of a federation (i.e. creation/update/removal), management of the application onboarding in the federation, edge node discovery, etc. The APIs for the interactions between OP and the User Client (UC) are provided in GSMA PRD OPG.05 [14]. The interactions between OP and the User Client (UC) are supported by the User-to-Network Interface (UNI), allowing the UC to discover the existence of an Edge Cloud service, to register to the OP’s Service Resource Manager (SRM), to trigger the selection of a cloudlet (a point of presence for the Edge Cloud offering a set of resources at a particular location as specified in GSMA PRD OPG.02 [3]) by the OP, to trigger the instantiation of an application instance on the selected cloudlet.

* * * * Next Change * * * *

# 6	Alignment of EDGEAPP with GSMA OP

This section captures the alignment of EDGEAPP with GSMA OP as per TS 23.558 which mainly provides any mapping guidance and interpretations of different EDGEAPP schemas, IEs, APIs etc. with GSMA requirements e.g. mapping of data models, application profiles etc.

## 6.1 General

Clause 6 describes the following alignment between 3GPP EDGEAPP and GSMA OP:

-	Relationship between 3GPP EDGEAPP architecture and GSMA OP architecture is described in clause 6.2.

## 6.2 Relationship between EDGEAPP architecture and GSMA OPG reference architecture

Figure 6.2-1 illustrates the relationship between EDGEAPP architecture and GSMA OPG reference architecture [3].

Figure 6.2-1: Relationship between EDGEAPP architecture and GSMA OPG reference architecture

EDGE-1 and EDGE-4 reference points can support similar function(s) as OP's User-Network interface (UNI), providing the Edge Enabler Client (corresponding to Edge/User Client in OP) with the information required to access the edge services. EDGE-1/EDGE-4 neither impact nor overlap with other existing 3GPP interfaces between the UE and the network, catering to the OP's requirements on UNI.

EDGE-2 and EDGE-8 reference points can support similar function(s) as OP's Southbound interface (SBI), through which the edge enabler layer (corresponding to the operator platform) access the 3GPP network capabilities and services (e.g. SCEF/NEF). Specifically, EDGE-2 and EDGE-8 cater to the requirements of the SBI-Network Resource interface. ECSP management system as specified in 3GPP TS 28.538 [8] caters to the requirements of OP's SBI-Cloud Resource interface.

EDGE-3 reference point can support similar function(s) as OP's Northbound interface (NBI), exposing the capabilities of Edge Enabler Server to the Edge Application Servers (EAS) hosted on the edge. OP's NBI also expands capabilities exposure to ASPs, for example to on-board applications to be deployed as EASs based on specific criteria.

EDGE-9 reference point can support similar function(s) as OP's East/Westbound interface (E/WBI), allowing the edge enabler layer to interact within and beyond its domains e.g., between operator platforms. OP's E/WBI focuses on use cases like user and application roaming or resource sharing across domains.