3GPP TSG-SA WG6 Meeting #56	S6-232664

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232472)

Source:	Nokia, Nokia Shanghai Bell

Title:	Editorial correction to clause 7

Spec:	3GPP TR 23.958 v0.5.0

Agenda item:	7.9

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Editorial correction to clause 7.

2. Reason for Change

Clause 7.2 is a hanging paragraph and should be fixed.

The layout of the figure in clause 7.2 cannot be modified. Hence, changed the figure in clause 7.2 to a visio figure.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958v0.5.0.

* * * First Change * * * *

# 7	Deployment Considerations

## 7.1	General

Application Server

(EAS and/or

MEC app instance)

Edge platform

EDGE-3

/ Mp1

EDGE-9 / Mp3

Application Server

(EAS and/or

MEC app instance)

Edge platform

EDGE-3

/ Mp1

CAPIF-1(e) & CAPIF-2(e)

CAPIF-1(e) & CAPIF-2(e)

CAPIF-6(e)

Figure 7.1-1 shows the EDGEAPP and ETSI MEC aligned Application Server and platform deployment. The depicted Edge platform consists of functionalities provided by both EES and MEC platform. The EES and MEC platform are aligned in the implementation of the Edge platform. The depicted Application Server implements the functionalities of an Edge Application Server or MEC application instance or both.

Figure 7.1-1: EDGEAPP and ETSI MEC aligned Application Server and platform deployment.

EDGE-3 is provided to Application Servers presenting themselves to the Edge platform as Edge Application Servers. Mp1 is provided for Application Servers presenting themselves to the Edge platform as MEC application instances.

An Edge platform adopting the CAPIF framework could provide the capabilities offered by EDGE-3 and Mp1 through utilisation of CAPIF-1 (/CAPIF-1e) and CAPIF-2 (/CAPIF-2e) to provide a unified service. For instance, a mapping of the MEC service management API to the 3GPP CAPIF API is presented in Annex B of ETSI GS MEC 011 [5].

EDGE-9 and Mp3 provide services required for interconnectivity between Edge platforms. EDGE-9 provides the interconnectivity required by Edge Enabler Servers, whilst Mp3 provides the interconnectivity required by MEC platforms.

An Edge platform adopting the CAPIF framework could provide the capabilities offered by EDGE-9 and Mp3 through utilisation of CAPIF-6 (/CAPIF-6e) to provide a unified service.

NOTE: APIs supported over Mp3 have not currently been specified by ETSI MEC.

Management aspects relating to application server and platform management are captured in 3GPP TS 28.538 [2] (EDGEAPP entity specific) and ETSI GS MEC 010-2 [9] (MEC entity specific), where the commonality is that both specify ETSI NFV MANO for performing lifecycle management functions. Management related interfaces are not depicted in Figure 7.1-1.

### 7.2 Deployment option of EDGEAPP and ETSI MEC using CAPIF

3GPP provides a framework for supporting common capabilities for northbound APIs (e.g., onboarding, authentication, authorization, discovery, auditing, etc.) through the so-called common API framework (CAPIF) [10]. The functional model of CAPIF enables an API invoker to access (and invoke) service APIs and supports API exposing functions in publishing the API towards the API invokers. To enable the common supporting capabilities among API invokers and API providers, the CCF is introduced as the functional entity in charge of, among other tasks, authenticating an API invoker, providing authorization for the API invoker prior to accessing the service API, publishing, storing, and supporting the discovery of service APIs information, etc.

As from Annex A in TS 23.558 [2], an EAS may assume the role of:

- API invoker: for accessing northbound APIs exposed by SCEF/NEF or consuming service APIs offered by other EASs or EES, or

- API provider: exposing its service APIs for consumption by other API invokers,

whereas the EES may act like AEF and CCF (the latter being dispensable if, e.g., a centralized instance of CCF is available, see Clause A.5.3 in TS 23.558 [2]).

Additionally, as from Annex B in ETSI GR MEC 031 [11], a MEC App may assume the role of:

- API invoker: for accessing northbound APIs exposed by SCEF/NEF or consuming MEC services advertised by the MEP, or

- API provider: registering (publishing) its service APIs and exposing them for consumption by other API invokers,

whereas the MEP may assume the role of API provider, CCF, and API invoker. A MEC service produced by a MEC App or MEP can be mapped into the API provider domain in CAPIF.

Since the different functional entities of both EDGEAPP and ETSI MEC may assume several roles, the number of scenario combinations is high, which becomes worse if several trust domains are considered. Therefore, rather than providing an exhaustive study of possible CAPIF-based deployments, a generic scenario is presented in Figure 7.2-1, in which it is assumed that:

- MEC platform and EDGEAPP co-exist in the network within the same PLMN trust domain,

- API invokers want to access MEC services provided by MEP or MEC Apps, or API invokers want to consume EDGEAPP services provided by EASs or EES,

- REST-HTTP transport is used

NOTE 1:	Enhancements of the functional entities might be needed to fully support the functionalities of the CAPIF functional entities, namely CCF, API provider domain functions and API invoker

NOTE 2:	Extension of the reference point in both platforms might be needed to support the functionalities of the CAPIF-related reference points (see reference points in Figure 7.2-1)

Figure 7.2-1: Deployment option of EDGEAPP and ETSI MEC using CAPIF

On the left-hand side of Figure 7.2-1, EES implements CCF 1 to manage the Service APIs exposed by EAS and EES, EAS implements the API Provider domain functions to expose EAS Service APIs to EDGEAPP EASs (and possibly ETSI MEC Applications acting as API invokers). EES implements API Provider domain functions to expose EES Service APIs (e.g., UE location API, AC information exposure API, etc.) to API invokers (e.g., EASs, ETSI MEC Applications).

On the right-hand side of Figure 7.2-1, MEP implements an instance of CCF (i.e., CCF 2) to manage the MEC Services exposed by MEP and MEC App. Both MEP and MEC App implement API Provider domain functions to expose MEC Services via CAPIF-2.

CCF 1 and CCF 2 interact via CAPIF-6 reference point, which supports publishing the service APIs information and discovering the service APIs information on both platforms. The CCF 1 and the CCF 2 publish the service API provided by its connected API exposing function(s) and obtains the service API information provided by other CCF.

An API invoker (e.g., EAS or MEC App) connected to the any of the CCF can discover and invoke service APIs provided by the API exposing function connected to the othe CCF.

Editor’s note:	Deployment options including both trusted and untrusted domains is FFS.