3GPP TSG-SA WG6 Meeting #56	S6-232669

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232506)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0231 | rev | 001 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NRM BDT configuration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 14.3.13 on Background Data Transfer requires completion by defining the corresponding request/response and APIs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 14.3.2: added definition of BDT configuration request and BDT configuration response In clause 14.4.2: added definition of BDT configuration operation and BDT configuration notification In clause 14.3.13 a remaining EN is deleted and references to the new APIs have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of BDT in SEAL on behalf of VAL applications is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.13; New: 14.3.2.y1; 14.3.2.y2; 14.4.2.z1; 14.4.2.z2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1 (S6-232669) cover page corrections. In Table 14.3.2.y1-1: BDT configuration request the IE “number of UEs” and associated NOTE 1 were deleted BDT negotiation notification was introduced in 14.3.2.y3. The figures in clauses 14.3.13.2 and 14.3.13.3 have been updated to better reflect the flow naming |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.y1	BDT configuration request

Table 14.3.y1-1 describes the information flow for the BDT configuration request from the VAL server to the NRM Server.

Table 14.3.2.y1-1: BDT configuration request

| Information element | Monitoring | Description |
| --- | --- | --- |
| VAL service ID | M | Identity of the VAL service for which the configuration is requested. |
| List of VAL UE IDs ) | O | List of VAL UE IDs for which the transfer policy applies or a VAL group ID. |
| Volume per UE | O | Expected data volume for the background data transfer. |
| Desired time window | O | Desired time window for the background data transfer. |
| Desired area information | O | Desired geographical area for the background data transfer |
| Policy Selection Guidance | O | List that includes guidance to the NRM in selecting from multiple transfer policies provided by underlying network. Possible values include: ”lowest cost”, “highest throughput given maximum cost of X”, etc. If not included, the NRM may choose from among multiple transfer policies, depending on local and ASP-provided policies. |

#### 14.3.2.y2	BDT configuration response

Table 14.3.2.y2-1 describes the information flow for BDT configuration response from the NRM server to the VAL server.

Table 14.3.2.y2-1: BDT configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the UE-to-UE resource coordination response operation. |
| Policy reference ID | O | Indicates the Background data transfer policy ID |
| Granted time window | O | Granted time window for the background data transfer. |

#### 14.3.2.y3	BDT negotiation notification

Table 14.3.2.y3-1 describes the information flow for BDT negotiation notification from the NRM server to the VAL server.

Table 14.3.2.y3-1: BDT negotiation notification

| Information element | Status | Description |
| --- | --- | --- |
| Granted time window | M | Granted time window for the background data transfer. |
| Policy reference ID | O | Indicates the Background data transfer policy ID |

* * * Next Change * * * *

### 14.3.13	Background Data Transfer configuration

#### 14.3.13.1	General

The Background Data Transfer (BDT) feature requires an initial step in which policies are requested and negotiated. BDT Policy requests to the 3GPP network are based on an expected time window and UE set, with additional optional information e.g., expected data volume per UE. The UE set may be indicated as an expected number of UEs, a group ID or geographical area.

The feature allows for the server involved to negotiate the policies proposed by the network. It also allows the server to enable notifications to be sent, should network conditions affect future BDT policies.

Based on the BDT policies obtained using the procedures detailed in this clause, a VAL server can initiate a data transfer to the client at the negotiated time and with the negotiated charging rates. The data transfer between the  VAL Server and the VAL Client is performed without NRM Server enablement. Service layer functionality for the purpose of facilitating the data transfer with the negotiated policy is not in scope of this specification.

#### 14.3.13.2	Request and Select Background Data Transfer Policy

Figure 14.3.13.2-1 depicts a general procedure for the request and configuration of traffic policies for BDT initiated by a request from a VAL Server.

Figure 14.3.13.2-1: General Procedure for configuration of Background Data Transfer

1.	A VAL Server requests the NRM Server to negotiate with the 3GPP network a background data transfer policy using the BDT_Configuration_request (see clause 14.4.2.z1).

The request includes expected data volume, expected number of UEs, expected time window for the background data transfer. The request may also include group ID, geographic information for the UEs, a request expiration time, guidance for policy selection. If guidance for policy selection is not included, the  VAL Server indicates if the NRM Server may choose independently from among multiple transfer policies.

2.	Based on the request expiration time and Service Provider policies, NRM Server may determine to delay interactions with the 3GPP network in order to negotiate on behalf of multiple VAL Servers.

The NRM Server performs the resource management of background data transfer procedure described in 3GPP TS 23.502 [11] clause 4.16.7.2. The procedure requires that expected data volume, expected number of UEs, and expected time window are provided by the NRM Server. If the NRM Server determined to negotiate on behalf of multiple VAL Servers, the parameters included reflects a superset of the individual VAL Server requests.

NOTE 1:	The NRM Server determines to negotiate on behalf of multiple VAL Servers based on implementation options and local policies. For example, if the request expiration time and expected time window are sufficiently large and, respectively, far away in time, the NRM Server may be allowed to delay the negotiations with the 3GPP network in case another request is received, targeting the same group of UEs. If another request is received with expected time windows sufficiently close and if the guidance for policy selection allows, a single policy/time window may be negotiated instead. This allows the UE group to wake up only once for multiple background data transfers.

The 3GPP network determines one or more applicable transfer policies based on the requesting Background Data Transfer parameters. A list of transfer policies is provided to the NRM Server. Each transfer policy includes mandatory Reference ID, charging rating group reference and allocated time window and optional maximum UL and DL bandwidth. The NRM Server uses ASP policies and the transfer selection guidance (if available) to select a policy. The NRM Server informs the 3GPP Network of the selected transfer policy.

NOTE 2:	Based on 3GPP TS 23.503 [12]  clause 6.1.2.4. it is assumed that the NRM server is configured to understand the charging rating group reference based on agreements with the operator.

NOTE 3:	The NRM server sets the warning notification based on local policies.

3.	The NRM Server responds to the VAL Server, providing the Reference ID and allocated time window of the background data transfer policy.

#### 14.3.13.3	Reselect Background Data Transfer Policy

Figure 14.3.13.3-1 depicts a general procedure for reselecting BDT policies after BDT warning.

Figure 14.3.13.3-1: General Procedure for reselecting BDT policies after warning

1.	The 3GPP Network, via NEF, sends the BDT warning (BDT Policy negotiate) notification to the NRM server. The notification includes the affected BDT policy Reference ID and list of candidate BDT policies.

Each of the BDT policies in the candidate BDT list includes mandatory Reference ID, charging rating group reference and time window, as well as optional maximum UL and DL bandwidth.

2.	The NRM Server checks the new BDT policies included in the candidate list of the BDT warning notification. The NRM Server determines whether the notification affects multiple VAL Servers or not. The NRM Server uses ASP policies and the transfer selection guidance (if available) provided with the initial VAL Server request to select a policy.

The NRM Server informs the 3GPP Network of the selected transfer policy or that no new policy has been selected by using steps 11-16 of the procedure for BDT warning notification in 3GPP TS 23.502 [11] clause 4.16.7.3.

3	The NRM Server ends a new response to the VAL Server via the BDT_Negotiation_notification (see clause 14.4.2.z3), providing information about the new policy, or that no policy is available. If a new BDT policy is available, the information provided to the VAL Server includes the ID of the applicable policy and the time window.

* * * Next Change * * * *

#### 14.4.2.z1	BDT_Configuration_request

API operation name: BDT_Configuration_request

Description: Requesting Background data transfer configuration.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.y1

Outputs: See subclause 14.3.2.y2

#### 14.4.2.z2	BDT_Negotiation_notification

API operation name: BDT_Negotiation_notification

Description: Forwards BDT negotiation notification.

Known Consumers: VAL server.

Inputs: None

Outputs: See subclause 14.3.2.y3

* * * End of Changes * * * *