3GPP TSG-SA WG6 Meeting #56	S6-232670

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232364)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0001 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing cause field and transfer of SEALDD server policy information in context transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The response message indicates the success or failure but does not indicate the reason for the failure. The context transfer procedure described in clause 9.6.2.1 does not mention the transfer of SEALDD server policy. This change mentions explicitly the need to transfer the policy during context transfer as it is also a part of SEALDD-S context. It also introduces the cause field to understand the failure in the response for the consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a cause field to indicate the reason for the failure result and transfer of SEALDD server policy during the SEALDD context transfer procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consumer cannot understand the failed response and during transfer, the new SEALDD server will not have the SEALDD server policies from the OLD SEALDD server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.1, 9.6.3.1, 9.2.3.2, 9.2.3.4, 9.3.3.4, 9.3.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Added VAL server configured SEALDD policy transfer from the OLD SEALDD server to NEW SEALDD server via Pull operation. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.6.2.1	SEALDD context transfer

Figure 9.6.2.1-1 describes SEALDD context transfer procedure in pull (step 1a and 2a) or push (step 1b and 2b) operation.

Pre-conditions:

1.	For pull operation, the old SEALDD server endpoint is available in the new SEALDD server

2.	For push operation, the new SEALDD server endpoint is available in the old SEALDD server.

Figure 9.6.2.1-1: SEALDD context transfer

The transferred SEALDD context includes the service subscription information created upon VAL server's interaction for requesting SEALDD transmission service (e.g. step 1 and 2 in Figure 9.2.2.1-1), and also the SEALDD client and SEALDD server communication tunnel management information (e.g. UE IP address) which is created upon SEALDD client's interaction for requesting data transmission (e.g. step 5 and 6 in Figure 9.2.2.1-1). If new SEALDD server supports transportation layer service continuity, additional transport layer context (e.g. TCP/TLS/QUIC) is transferred from old SEALDD server to new SEALDD server. If the new SEALDD Server is not provisioned with the VAL server configured SEALDD policy, then it can pull from the Old SEALDD server via the pull operation. The push operation does not support the transfer of the VAL server configured SEALDD policy between Old and New SEALDD server.

The new SEALDD server, after receiving the SEALDD context from the old SEALDD server, allocates IP address and port for SEALDD-Uu user plane communication. The new SEALDD server also sends back to the old SEALDD server with the allocated endpoint information If operation is push. Then the old SEALDD server can request 5GC to perform IP replacement procedure, as defined in clause 6.3.3 of 3GPP 23.548 [8]. The request includes the traffic descriptor of old SEALDD server (i.e., SEALDD-UU address/port), the traffic descriptor of new SEALDD server (i.e., SEALDD-UU address/port) and/or the target DNAI.

Optionally, after receiving the SEALDD context with the SEALDD-UU endpoint from the old SEALDD server, the new SEALDD server can request 5GC to perform IP replacement procedure.

NOTE:	The TCP/TLS/QUIC context transfer is required to support the SEALDD server relocation with IP replacement procedure, as described in clause 9.6.2.2.

* * * Next change * * * *

#### 9.6.3.3	SEALDD context pull request

Table 9.6.3.3-1 describes the information flow from the new SEALDD server to the old SEALDD server to pull the SEALDD context.

Table 9.6.3.3-1: SEALDD context pull request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identifies the requestor (i.e. new SEALDD server). |
| SEALDD policy indication | O | Indicates the need to transfer the VAL server configured SEALDD policy. |

#### 9.6.3.4	SEALDD context pull response

Table 9.6.3.4-1 describes the information flow from the new SEALDD server to the old SEALDD server for responding to the SEALDD context pull request.

Table 9.6.3.4-1: SEALDD context pull response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-Uu Context | M | Identifies the context related to SEALDD-Uu connection, which is created upon SEALDD connection establishment. |
| SEALDD-S Context | M | Identifies the context related to SEALDD-S subscription. |
| Transport layer context | O | Identifies the context related to Transport layer (e.g. TCP/TLS/QUIC) for SEALDD-Uu user plane communication. |
| SEALDD policy | O | Indicates the VAL server configured SEALDD policy. |

* * * Next change * * * *

#### 9.2.3.2	SEALDD enabled regular transmission response

Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the regular application transmission.

Table 9.2.3.2-1: SEALDD enabled regular transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information Data transmission connection information | O | Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * Next change * * * *

#### 9.2.3.4	SEALDD regular transmission connection establishment response

Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the regular SEALDD connection establishment.

Table 9.2.3.4-1: SEALDD regular transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side (for client side initiated request) or the SEALDD client side (for server side initiated request) used to establish SEALDD connection. |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O See NOTE 2 | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure |  |  |

* * * Next change * * * *

#### 9.3.3.4	SEALDD URLLC transmission connection establishment response

Table 9.3.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the URLLC transmission connection establishment.

Table 9.3.3.4-1: SEALDD URLLC transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD server side used to establish redundant SEALDD connection. |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure |  |  |

* * * Next change * * * *

#### 9.3.3.6	SEALDD URLLC transmission connection update response

Table 9.3.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the redundant transmission connection update.

Table 9.3.3.6-1: SEALDD URLLC transmission connection update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the SEALDD connection. |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure |  |  |

* * * End of changes * * * *