3GPP TSG-SA WG6 Meeting #56	S6-232671

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232367)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0002 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD client initiated connection release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SEALDD client can initiate the connection release for various reasons like airplane mode, device power off, or client-initiated etc. To handle such triggers, there is no mechanism to release the SEALDD connection in the existing clause 9.2. This proposal addresses the gap of client initiated connection release procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing SEALDD client-initiated connection release procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD client connection resources cannot be released. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.x(new), 9.2.3.5, 9.2.3.6, 9.2.4.1, 9.2.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - SEALDD server releases the connection established by SEALDD client or SEALDD server - Figure 9.2.2.x-1- Information flow alignment with the IE table name Table 9.2.3.5-1, Table 9.2.3.6-1 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.2.x	SEALDD client initiated connection release

Figure 9.2.2.x-1 illustrates the procedure for SEALDD client initiated connection release procedure from the SEALDD client to the SEALDD server.

Figure 9.2.2.x-1: SEALDD client initiated connection release

1.  The SEALDD client sends the SEALDD connection release request to the SEALDD server to release the established connection.

2.  The SEALDD server releases the SEALDD-UU data transmission connection (which was established by SEALDD client or SEALDD server) and sends the response in the SEALDD connection release response message. Upon receiving the acknowledgement, the SEALDD client can release the connection resources.

* * * Next change * * * *

#### 9.2.3.5	SEALDD regular data transmission connection release request

Table 9.2.3.5-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the SEALDD connection release.

Table 9.2.3.5-1: SEALDD regular data transmission connection release request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD flow ID | M | Identifies the SEALDD flow. |

#### 9.2.3.6	SEALDD regular data transmission connection release response

Table 9.2.3.6-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding the SEALDD connection release request.

Table 9.2.3.6-1: SEALDD regular data transmission connection release response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the operation. |

* * * Next change * * * *

#### 9.2.4.1	General

Table 9.2.4.1-1 illustrates the APIs exposed by SEALDD server for regular connection establishment.

Table 9.2.4.1-1: List of SEALDD server APIs for data distribution

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_RegularTransmission | Request | Request/Response | VAL server |
| Sdd_RegularTransmissionConnection | Establish | Request/Response | SEALDD client |
|  | Release | Request/Response | SEALDD client |
| Sdd_ConnectionStatusEvent | Subscribe | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |

* * * Next change * * * *

#### 9.2.4.3	Sdd_RegularTransmissionConnection_Establish operation

API operation name: Sdd_RegularTransmissionConnection_Establish

Description: The consumer requests for one time for SEALDD enabled regular data connection establishment.

Inputs: See clause 9.2.3.3.

Outputs: See clause 9.2.3.4.

See clause 9.2.2.1 and 9.2.2.2 for details of usage of this operation.

* * * Next change * * * *

#### 9.2.4.x	Sdd_RegularTransmissionConnection_Release operation

API operation name: Sdd_RegularTransmissionConnection_Release

Description: The consumer requests to release the SEALDD connection resources.

Inputs: See clause 9.2.3.5.

Outputs: See clause 9.2.3.6.

See clause 9.2.2.x for details of usage of this operation.

* * * End of changes * * * *