3GPP TSG-SA WG6 Meeting #56	S6-232672

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232368)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0003 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the missing SEALDD policy delete procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SEALDD policy configuration procedure lacks a delete procedure. The VAL server can request the SEALDD server to delete the policy for various reasons, such as the VAL server no longer wanting to use the SEALDD service or removing the old policy for installing new or updated policies. The proposed procedure enables the VAL server to delete the configured policy. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix missing SEALDD policy delete procedure in clause 9.10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VAL server cannot remove the configured policy in the SEALDD server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.2.x(new), 9.10.3.x1(new), 9.10.3.x2(new), 9.10.4.1,9.10.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Added Huawei in the source to merge since we have a overlap |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.10.2.x	SEALDD policy configuration delete

Figure 9.10.2.x-1 illustrates the procedure for SEALDD policy configuration delete from the VAL server used for SEALDD policy configuration to the SEALDD server.

Figure 9.10.2.x-1: SEALDD policy configuration delete

1.  The VAL server used for SEALDD policy configuration determines that the existing SEALDD policy needs to be deleted, the VAL server sends the SEALDD policy configuration delete request to the SEALDD server.

2. The SEALDD server performs authorization check to verify whether the VAL server can be accepted/authorized to delete the SEALDD policy configuration.

3.  Upon successful authorization, the SEALDD server deletes the SEALDD policy configuration and replies to the VAL server with the SEALDD policy configuration update response.

* * * Next Change * * * *

#### 9.10.3.x1	SEALDD policy configuration delete request

Table 9.10.3.x1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration delete.

Table 9.10.3.x1-1: SEALDD policy configuration delete request

| Information element | Status | Description |
| --- | --- | --- |
| Configuration ID | M | Identifier of the SEALDD policy configuration. |

* * * Next Change * * * *

#### 9.10.3.x2	SEALDD policy configuration delete response

Table 9.10.3.x2-1 describes the information flow from the SEALDD server to the VAL server for responding to the SEALDD policy configuration delete.

Table 9.10.3.x2-1: SEALDD policy configuration delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

* * * Next Change * * * *

#### 9.10.4.1	General

Table 9.10.4.1-1 illustrates the APIs exposed by SEALDD server for SEALDD policy configuration.

Table 9.10.4.1-1: List of SEALDD server APIs for policy configuration

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_PolicyConfiguration | Request | Request/Response | VAL server |
|  | Update |  |  |
|  | Delete |  |  |

* * * Next Change * * * *

#### 9.10.4.x	Sdd_PolicyConfiguration delete operation

API operation name: Sdd_PolicyConfiguration_Delete

Description: The consumer requests for one time for SEALDD policy configuration deletion.

Inputs: See clause 9.10.3.x1.

Outputs: See clause 9.10.3.x2.

See clause 9.10.2.x for details of the usage of this operation.

* * * End of changes * * * *