SA WG6 Meeting #56	S6-232679

Goteborg, Sweden, August 21 – 25, 2023	(revision of S6-232320)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0001 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing information elements to information flow of PINE join into PIN request/response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Figure 8.5.8.2.1-1: PIN client requests to join into a PIN, that in step 3, the PEMC can indicate to PIN client of the lifetime of the PIN. But in the Table 8.5.8.3.3-1: PIN Management PINE join into PIN response, the life time of PIN is missing. Also, in step 1 of Figure 8.5.8.2.4-1, that the PIN Management PINE join into PIN request contains the PEMC identity. But in the Table 8.5.8.3.2-1, the PEMC ID is also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the life time of PIN into the Table 8.5.8.3.3-1: PIN Management PINE join into PIN response. Add the PEMC ID into the Table 8.5.8.3.2-1: PIN Management PINE join into PIN request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relevant information elements in PIN Management PINE join into PIN request/response are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.3.2, 8.5.8.3.3, 8.5.8.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.8.3.2	PIN Management PINE join into PIN request

Table 8.5.8.3.2-1 describes information elements in the PIN Management PINE join into PIN request from the PINE to PEMC, from PINE to PEGC, from PEGC to PEMC, from PIN server to PEMC.

Table 8.5.8.3.2-1: PIN Management PINE join into PIN request

| Information element | Status | Description |
| --- | --- | --- |
| PIN ID | M | Identifier of the PIN that wants to join in. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN client ID | M | The PIN client ID of PINE |
| PEMC ID | O | Identifier of the PEMC that PEGC should send request to. |
| PIN client profile(s) | O | Profiles of PIN clients. The PIN client profiles are further described in Table 8.2.2.3. |
| PIN server endpoint information | O | Includes URI(s), FQDN(s), IP address(es)) of PIN server. |

* * * Start of Change 2 * * *

8.5.8.3.3	PIN Management PINE join into PIN response

Table 8.5.8.3.3-1 describes information elements in the PIN Management PINE join into PIN response from the PEMC to PINE, from PEGC to PINE, from PEMC to PEGC, from PEMC to PIN server.

Table 8.5.8.3.3-1: PIN Management PINE join into PIN response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (see NOTE) | Indicates that the PIN Management PINE join into PIN request was successful. |
| > Updated PIN client profile | O | PIN client profile information updated by the PEMC (e.g., default and backup PEGCs assigned to PINE). |
| > Heartbeat Timer | M | Heartbeat timer value assigned to PINE |
| > Lifetime of the PIN | M | Indicates the lifetime of PIN. |
| > Identifier of PEGCs | O | Indicates the PINE identifier authorized to be the PEGCs of this PIN. |
| >> PEGC address | O | Assigned IP address or port number of PEGC |
| > PEGC information | O | Includes the PEGC information for example, |
| >> Access control information | O | Includes: user name, account, SSID, BSSID. All the information is used by PIN elements in PIN to access 5G or access other application outside of PIN |
| Failure response | O (see NOTE) | Indicates that the PIN Management PINE join into PIN request failed. |
| > Cause | M | Provides the cause for PIN Management PINE join into PIN request failure. |
| NOTE: At least one of the IE shall be present. |  |  |

* * * Start of Change 3 * * *

8.5.8.2.4	PINE join into PIN via PEGC

The PINE joins the PIN via the PEGC is depicted in Figure 8.5.8.2.4-1.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC;

2.	The PINE already establishes the connection with PEGC;

3.	The PIN information to join is available at the PINE via e.g., PIN discovery procedure.

Figure 8.5.8.2.4-1: PINE join into PIN via PEGC

1.	The PINE sends PIN Management PINE join into PIN request to the PEGC. The PIN Management PINE join into PIN request contains the PIN ID which identifies the PIN to join, PINE client ID and credentials if available, PEMC identity/PIN server address.

2.	The PEGC identifies the received message is the PIN Management PINE join into PIN request and perform the authorization. If authorized, the PEGC determines to forward the PIN Management PINE join into PIN request to the PEMC or the PIN server.

3.	The PEGC forwards the PIN Management PINE join into PIN request to the PEMC in step 3a, based on the PEMC identity in step 1 or by resolving the PIN ID. The PEMC authorized the PINE to join the PIN, and returns the PIN Management PINE join into PIN response to the PEGC in step 3b.

4.	(Optional) The PEGC forwards the PIN Management PINE join into PIN request to the PIN server in step 4a based on the PIN server address in step 1 or by resolving the PIN ID. The PIN server forwards the PIN Management PINE join into PIN request to the PEMC in step 4b, and the PEMC authorized the PINE to join the PIN, and returns the PIN Management PINE join into PIN response to the PIN server in step 4c. Further the PIN server return the PIN server to the PEGC in step 4d.

5.	After the join, the PEMC update the PIN and may notify other entities (e.g., existing joined members, PIN server). The PEMC triggers the PIN status notify to PEGC/PINE as indicated in step 5-6 of procedure Figure 8.5.8.2.2-1.

6.	The PEGC return the PIN Management PINE join into PIN response to the PINE.

* * * End of Change * * *