SA WG6 Meeting #56	S6-232683

Goteborg, Sweden, August 21 – 25, 2023	(revision of S6-232324)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0005 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of information flow of PIN status notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In PIN status notify, that there are some issues unclear and need to clarification. Firstly, in Table 8.5.9.3.4-1, there are no information under the PIN profiles update. So, it better to add the updated PIN profile under this and refer to the former paragraph. Secondly, for each event ID that in Table 8.5.9.3.4-1, it better to not have the description. Thirdly, due to in some of the procedure, that the PINE (including PEMC or PINE or PEGC) should be notified with the dynamic PIN profile, in order to save the message design, the updated dynamic PIN profile should be added in the PIN status notify message. At last, for PIN modification bullet, only the updated PIN profile is enough, and no need to further describe the PEGC/PEMC information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Updated PIN profile sub-bullet under the PIN profile update in Table 8.5.9.3.4-1. Remove the description of each event ID. Remove the PEGC/PEMC information. Add dynamic PIN profile update to output of PIN status notify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear of the relationship between updated PIN profile and PEMC/PEGC information. There is no details information under the PIN profiles update. How to update the dynamic PIN profile is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.9.3.4	PIN status notify

Table 8.5.9.3.4-1 describes information elements in the PIN status notify from the PEMC to PEMC/PEGC/PINE/PIN server.

Table 8.5.9.3.4-1: PIN status notify

| Information element | Status | Description |
| --- | --- | --- |
| PIN Status event type | M | Identifies PIN status event types contained in the notification; more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave) - PIN profile update - PIN state (Activated or de-activated) |
| PINE management | O | Parameters for PINE management event |
| > PINE join into the PIN | O | Indicates the PINEs join into the PIN |
| >> PIN client ID | M | The identifier of the joined PINE |
| >> PIN client profile(s) | O | Profiles of PIN clients that joins the PIN. The PIN client profiles are further described in Table 8.2.2.3. |
| > PINE leaves the PIN | O | Indicates the PINEs leaves from the PIN |
| >> PIN client ID | M | The identifier of the leaved PINE |
| PIN modification | O | Parameters for PIN modification event |
| > Updated PIN profile | M | Updated PIN profile information (indicated in clause 8.2.2) that introduced by PIN modification that indicated in section 8.5.10. |
| PIN profiles update | O | Parameters for PIN profiles update |
| > Updated PIN profile | M | Updated PIN profile information that as indicated in section 8.2.2. |
| Dynamic profile information of a PIN | O | Referred to section 8.2.2.2. |
| Status of PIN state | O | Parameters for PIN state, including activate or deactivate |

* * * End of Change * * *