3GPP TSG-SA WG6 Meeting #56	S6-232707

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232458)

Source:	China Mobile, Huawei, HiSilicon

Title:	Pseudo-CR on Analysis of OMA OneAPI for enabling eMMTel service

Spec:	3GPP TR 23.700-92 v0.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for Analysis of gaps between OMA OneAPI and requirements of enabling  eMMTel service.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.0.0.

* * * First Change * * * *

4	Analysis of gaps

4.1	Introduction

The subsequent sub-clauses contain the results of an analysis of existing architecture and APIs in application layer frameworks against the requirements of enabling eMMTel service. This analysis has been performed as a starting point of the eMMTelAPP study to get a better understanding of existing functionality and the gaps.

4.x	Analysis of OMA OneAPI framework

4.x.1	Description

OMA OneAPI framework provides Call Control and Configuration capabilities via NGSI-4, NGSI-5, NGSI-6 and NGSI-7 interfaces (see OMA AD NGSI [y1]) in enabler layer. The interfaces extend the interface provided by Parlay X.  The OneAPI Call Control profile of the RESTful Network APIs defines a subset of the HTTP operations of OMA REST_NetAPI_3PC[y2], OMA REST_NetAPI_CallNotif [y3] and OMA REST_NetAPI_AudioCall [y4].

4.x.2	Detailed analysis

- This clause provides the results of a high-level gap analysis of the OMA OneAPI framework needed to support the enabling of eMMTel service, based on OMA inputs. Table 4.x.2-1 provides the results of this analysis and lists the gaps identified that may be applicable to the eMMTelAPP.

Table 4.x.2-1: OMA OneAPI framework analysis results

| Capabilities needed to enable eMMTel service | OMA OneAPI supported capabilities | OMA specification | Gap |
| --- | --- | --- | --- |
| Call control | OMA OneAPI provide Third Party Call capabilities for the call control. The operations include Create new call session, Terminate call Session, Get information of call session, Get a list of participants of a call session, Get information of an individual call session participant, add/remove/move participant etc. | OMA REST_NetAPI_3PC [y1] | Partial All of the Call control capabilities provided by OMA OneAPI is used for audio call. Video call and IMS data channel in the eMMTel are not supported. |
| Call Notification | OMA OneAPI provide Call Notification capabilities. The operations include the client retrieve/manage the Call Notification Subscriptions, server to inform the client about a call event etc. | OMA REST_NetAPI_CallNotif [y2] | Partial All of the Call event subscription/notification provided by OMA OneAPI is used for audio call. Video call and IMS data channel related events are not supported. |
| Application layer procedures needed for eMMTel service | Not available | Not available | Gap Application layer procedures needed for eMMTel service, e.g: download data channel application to UE, are not needed in audio call, therefore they are not supported by OMA OneAPI. |

In addition, the architecture of OMA OneAPI is based on the OSA architecture specified in 3GPP TS 23.198 [y5]. The  OSA architecture has not been updated since Rel-9. Therefore, OMA OneAPI cannot support the underlying 3GPP IMS Architecture supporting eMMTel service specified in Annex AC of 3GPP TS 23.228 [y6].

Based on the analysis above, this framework can not support the enabler layer capabilities exposure of the architecture and procedures specified in Annex AC.2 of 3GPP TS 23.228 [y6] and SA WG2 Rel-18 NG_RTC WID. It is recommended to study a new application layer architecture and related APIs in this study.  Some ideas of the basic functionalities of OMA OneAPI can be considered to be used for reference of this study.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[y1]	OMA-AD-NGSI-V1_0-20120529-A: "Next Generation Service Interfaces Architecture".

[y2]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[y3]	OMA-TS-REST_NetAPI_CallNotification-V1_0-20200226-C: "RESTful Network API for Call Notification".

[y4]	OMA-TS-REST_NetAPI_AudioCall-V1_0-20200226-C: "RESTful Network API for Audio Call".

[y5]	3GPP TS 23.198: "Open Service Access (OSA)".

[y6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".