3GPP TSG-SA WG6 Meeting #56	S6-232709

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232460)

Source:	China Mobile, Huawei, HiSilicon

Title:	Pseudo-CR on key issue of downloading data channel application to UE

Spec:	3GPP TR 23.700-92 v0.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes text for key issue of downloading data channel application to UE.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.0.0.

* * * First Change * * * *

6.1	Key Issue #X: download data channel application to UE

6.1.1	Description

In eMMTel service, different specific service may have different requirement for the data channel application. E.g. in the pre-call service, e.g. Multimedia CLIP (Calling Line Identification Presentation), COLP (Connected Line Identification Presentation) described in 3GPP TR 22.873 [x1], the data channel application should be downloaded before the call session is established and needed to be downloaded in a very short time to reduce the delay of call establishment. In some in-call application, e.g. Real-time screen sharing, Real-time visual interactive menu described in 3GPP TR 22.873 [x1], the download of data channel application needs to fulfil the real-time requirement. These kind of requirements cannot be left to implementation specific since the interaction between application traffic and IMS signalling and media are needed, i.e. the data channel application needs to know some information of the IMS signalling and media to decide its behavior.

In additional, the downloading and processing of data channel application is associated with an IMS audio/video session, the capability of indicating how to handle the downloading and processing of data channel application is needed to be provided by MNO to ensure the service quality, e.g. avoid the unpredictable service conflict in multiple data channel scenario.

The requirements for the data channel application may include but not limited to:

- download time, e.g. pre-call or in-call;

- whether download and launched to the UE automatically; and

- whether loaded to the native dialler automatically.

Based on the architecture and procedures specified in 3GPP TS 23.228 [x2], the data channel applications (bootstrapping) is downloaded via DCMF and/or MRF to the UE based on UE subscription. The detailed procedures to download data channel applications to the UE are out of scope of 3GPP TS 23.228 [x2].  Therefore, application layer procedures are needed to be specified to fulfill the service requirements for the data channel applications. The capabilities of handling the application layer procedure of downloading data channel application to UE may also be provided in enabler layer to the application providers/Vertical service provider to control the behavior of their data channel applications.

The following architecture requirements have been identified to fulfil the service requirements:

- The application layer should support to provide the capability of indicating how to handle the downloading and processing of a specific data channel application on UE based on the service requirement of the specific data channel application and MNO.

- The application layer should support to provide and control the capability to data channel application provider to set this indicator.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[x2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".