SA WG6 Meeting #56	S6-232711

Goteborg, Sweden, August 21 – 25, 2023	(revision of S6-232688)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0009 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PIN status service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In PIN status subscription request, that using PIN status event types to identify the subscribed event. But sometimes, the event ID is often used, just like the design in EDGEAPP. So, in this paper, make a replacement from event type to event ID. In PIN status update, the PIN ID is missing in information flow. In PIN status update response, maybe some of the subscription event is accepted, and some are rejected. So, in the successful response, accepted event ID should be included, and also the rejected event ID should be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the PIN status event types to event ID. In PIN status update, the PIN ID is added in information flow. In PIN status update response, accepted event ID and also the rejected event ID should be included individually. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details are missing and not correspond to the process. Sometimes not use event types to identify the subscribed event. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.9.3.2, 8.5.9.3.5, 8.5.9.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.5.9.3.2	PIN status subscribe request

Table 8.5.9.3.2-1: PIN status subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| PIN client ID | M | Unique identifier of the PINE/PEGC/PIN server. |
| Subscribed Events IDs | M | Identifies PIN status event for which the subscriber is notified, more than one PIN status event IDs can be provided. Event IDs: - PIN modification - PINE management (PINE join/leave) - PIN profile update - PIN state |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the PINE/PEGC/PIN server should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

* * * Start of Change 2 * * *

8.5.9.3.5	PIN status update request

Table 8.5.9.3.5-1: PIN status update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Subscribed Events | O | Identifies PIN status event types for which the subscriber is notified, more than one PIN status event type can be provided. PIN status event types - PIN modification - PINE management (PINE join/leave) - PIN profile update - PIN state |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the PINE/PEGC/PIN server should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

* * * Start of Change 3 * * *

8.5.9.3.3	PIN status subscribe response

Table 8.5.9.3.3-1: PIN status subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Successfully subscribed event IDs | O | Indicates the successfully subscribed event IDs. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |
| > Unsuccessfully subscribed event IDs | O | Indicates the unsuccessfully subscribed event IDs. |

NOTE: If some of the event ID is accepted to subscribe and others are rejected, the service provider should indicate the event ID that accepted and rejected individually to service consumer.

* * * End of Change * * *