3GPP TSG-SA WG6 Meeting #55	S6-232717

Berlin, 22th – 26th May 2023	(revision of S6-232442,2572)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0425 | rev | 2 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on connection authorisation of an MC gateway client hosted by an MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 11.5.1.3.3 Connection authorisation for non-3GPP devices that do not host an MC client, this procedure requires the MC gateway client is also within the MC gateway UE as indicated in figure 11.5.1.3.3-1 and the descriptions. However, in figure 11.2.1-1 functional model, there is no such MC gateway client in the MC gateway UE. So the procedure 11.5.1.3.3 needs to be updated. The MC gateway UE server will interact with the gateway UE functions in the server to achieve the connection authroization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the above text from 11.5.1.2.1, 11.5.3.1, 11.5.1.3.3, 11.5.4.2.1, 11.5.4.3.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion and misunderstandings will be caused.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.2.1, 11.5.3.1, 11.5.1.3.3, 11.5.4.2.1, 11.5.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

11.5.1.2.1	General

The solution is applied to non-3GPP devices which can host an MC client and the MC gateway client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC gateway client is authorized and controlled by the MC server. The MC gateway client informs MC clients about the connection status.

For the period of association between MC server, MC gateway client and MC gateway UE, the MC server maintains the assignment between MC clients to the MC gateway UE used. This assignment is cancelled again with the disconnection.

# /********************Next Changes ********************/

11.5.1.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client and the MC gateway client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC client is authorized and controlled by the MC server.

NOTE:	The interworking between the MC gateway client hosted at the MC gateway UE and an MC service user is out of scope of the present document, nevertheless, the connection authorisation performed by the MC gateway UE shall enable the non-3GPP devices to get the access to MC services requested by the service user.

# /********************Next Changes ********************/

11.5.1.3.3	Connection authorisation procedure

The procedure for connection authorisation of the MC gateway UE towards an MC service server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device which cannot host the MC client.

-	The MC gateway UE has been selected.

-	The MC gateway UE server, which is hosted by the MC gateway UE, has been configured with the necessary parameters needed for connectivity with the MC gateway UE function within the MC service server.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

MC gateway UE server hosted by MC gateway UE

MC gateway UE function within

MC service server

1. Connection authorization request

2. Authorization check

3. Connection authorization response

Figure 11.5.1.3.3-1: Connection authorisation of an MC gateway UE connected by non-3GPP device cannot host MC clients

1.	The MC service client, hosted by the MC gateway UE, requests connection authorization with an MC service server via the MC gateway UE server function by providing the GW MC service ID. The MC gateway UE sends the connection authorization request to the MC service server.

2.	The MC service server performs an authorization check, to verify that access using the MC gateway UE is permitted. An MC service server shall reject the connection authorization when the MC service server receives connection authorization from a MC gateway client for a particular MC service for which the connection already exists with the same or different MC gateway UE.

3.	The MC service server sends the connection authorization response to the MC gateway UE server residing on the MC gateway UE.

The MC service client has now access to the MC service server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

# /******************** Next Changes ********************/

11.5.4.2.1	General

The solution is applied to non-3GPP devices which can host an MC client and the MC gateway client. The MC gateway UE forwards the disconnection request to the corresponding MC server to disconnect the MC gateway UE to MC client connection.

# /******************** Next Changes ********************/

11.5.4.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client and the MC gateway client. The MC server is requested to disconnect the MC gateway UE to MC client connection on demand.