3GPP TSG-SA WG6 Meeting #56	S6-232736

# Goteborg, Sweden, 21st August – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0228 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN resolution for the subscribe-notify service operations for the SS_VALServiceAreaConfiguration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The EN in clause 9.4.1 needs to be resolved to freeze the Release-18. Also, the note in clause 9.3.13.4 contradicts the VAL service area ID purpose, i.e., the VAL service area ID(s) update shall affect the existing subscriptions to avoid unnecessary API invocations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the subscribe-notify service operations in SEAL-X reference point for the SS_VALServiceAreaConfiguration API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The defenition of the VAL service area functionality cannot be completed in the stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.13.4, 9.3.2.42, 9.3.2.43, 9.3.2.44, 9.3.13.6, 9.3.13.7, 9.3.13.8, 9.4.1, 9.4.8.6, 9.4.8.7, 9.4.8.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.13.4	Update VAL service area identifier procedure

Figure 9.3.13.4-1 illustrates the high level procedure of the VAL service area update.

Figure 9.3.13.4-1: Update VAL service area procedure

1.	The VAL server sends the VAL service area update request to the location management server to update the VAL service area identifier(s) configured by the VAL server. In the request message, the VAL server includes the information as specified in table 9.3.2.28-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL service area update request.

3.	The location management server checks whether the VAL service area IDs exist and then updates the VAL service area identifier(s).

4.	The location management server sends the VAL service area update response with the information as specified in table 9.3.2.29-1.

Editor's note:	The security aspect related to the common usage of VAL service area information amongst different service APIs of LMS is FFS and is in SA3 scope.

* * * Next change * * * *

#### 9.3.2.42	VAL service area subscription request

Table 9.3.2.42-1 describes the information flow from a SEAL server to a location management to subscribe VAL service area identifier update events at the LM server.

Table 9.3.2.42-1: VAL service area subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Indicates the SEAL server identity. |
| List of the subscribed events | M | Indicates the list of the requested events. |
| > Event type | M | Indicates the subscribed event (e.g., update/deletion of the VAL service area ID) |
| > VAL service area IDs | O | Indicates the list of the VAL service area identifiers. This information element shall be provided when the requested event type is update or delete VAL service area ID. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL service area update events. |
| Subscription duration | O | It indicates the subscription duration. |

* * * Next change * * * *

#### 9.3.2.43	VAL service area subscription response

Table 9.3.2.43-1 describes the information flow from a location management server to a SEAL server for VAL service area subscription response.

Table 9.3.2.43-1: VAL service area subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |

* * * Next change * * * *

#### 9.3.2.44	VAL service area notification

Table 9.3.2.44-1 describes the information flow for VAL service area notification from a location management server to a SEAL server.

Table 9.3.2.44-1: VAL service area notification

| Information element | Status | Description |
| --- | --- | --- |
| List of events | M | Indicates the list of the occurred events. |
| > Event type | M | Indicates the subscribed event (e.g., update/deletion of the VAL service area ID). |
| > List of VAL service area information | M | List of one or more VAL service area identifiers. |
| >> VAL service area ID | M | The VAL service area identifier. |
| >> Geographical locations list | M | List of one more geographical locations (e.g., co-ordinates) corresponding to the VAL service area ID. |

* * * Next Change * * * *

#### 9.3.13.6	VAL service area identifier subscribe procedure

Figure 9.3.13.6-1 illustrates the high level procedure of the subscribe VAL service area update events.

Figure 9.3.13.6-1: Subscribe VAL service area procedure

1.	The SEAL server sends the VAL service area subscription request to the location management server to subscribe for the VAL service area identifier(s) update events. In the request message, the VAL server includes the information as specified in table 9.3.2.42-1.

2.	The location management server shall check if the SEAL server is authorized to susbcribe for the VAL service area identifier(s) update event(s).

3.	if the SEAL server is authorized to subscribe for the VAL service area identifier(s) update events, the location management server creates the subscription based on the provided parameters in the request.

4.	The location management server sends the VAL service area subscription response with the information as specified in table 9.3.2.43-1.

* * * Next Change * * * *

#### 9.3.13.7	VAL service area identifier notify procedure

Figure 9.3.13.7-1 illustrates the high level procedure for notification of a SEAL server about the VAL service area identifier update events at the location management server.

Pre-conditions:

-	the SEAL server has an active VAL service area subscription at the location management server; and

-	the subscribed event has occurred.

Figure 9.3.13.7-1: Notify VAL service area procedure

1.	The location management server identifies the list of the occurred events for which the SEAL server subscribed.

2.	The location management server sends a VAL service area notification to the SEAL server. In the notification message, the location management server includes the information as specified in table 9.3.2.44-1.

* * * Next Change * * * *

#### 9.3.13.8	VAL service area identifier unsubscribe procedure

Figure 9.3.13.8-1 illustrates the high level procedure for notification of a SEAL server about the VAL service area identifier update events at the location management server.

Pre-conditions:

-	the SEAL server has an active VAL service area subscription at the location management server.

Figure 9.3.13.8-1: Unsubscribe VAL service area procedure

1.	When the SEAL server decides to terminate a VAL service area subscription, it sends a VAL service area unsubscribe request to the location management server. In the request message, the SEAL server includes the information as specified in table 9.3.2.21-1.

2.	If the SEAL server is authorized to terminate the subscription, the location management server terminates the VAL service area subscription.

3.	The location management server sends a VAL service area unsubscribe request to the SEAL server, the location management server includes the information as specified in table 9.3.2.22-1.

* * * Next change * * * *

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |

* * * Next Change * * * *

#### 9.4.8.6	Subscribe_VAL_Service_Area_Change_Event

API operation name: Subscribe_VAL_Service_Area_Change_Event

Description: Subscription to the VAL service area identifiers change events.

Known Consumers: SEAL server.

Inputs: See subclause 9.3.2.42

Outputs: See subclause 9.3.2.43.

See subclause 9.3.13.6 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.7	Notify_VAL_Service_Area_Change_Event

API operation name: Notify_VAL_Service_Area_Change_Event

Description: Notifying the SEAL server about the VAL service area identifier change events.

Known Consumers: SEAL server.

Inputs: See subclause 9.3.2.44

Outputs: None.

See subclause 9.3.13.7 for the details of usage of this API operation.

* * * Next Change * * * *

#### 9.4.8.8	Unsubscribe_VAL_Service_Area_Change_Event

API operation name: Unsubscribe_VAL_Service_Area_Change_Event

Description: Unsubscribing from VAL service area identifier change events.

Known Consumers: SEAL server.

Inputs: See subclause 9.3.2.21

Outputs: See subclause 9.3.2.22.

See subclause 9.3.13.8 for the details of usage of this API operation.

* * * End of changes * * * *