3GPP TSG-SA WG6 Meeting #56	S6-232759

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232701, S6-232347)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0001 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Procedure on Service experience |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on LS S6-232250, - For pushing service experience report, - For direct reporitng, ADAE client uses data repporting mechanism to send service experience report, as defined in TS 26.531; - For pull service experience report, ADAE server can request pulling of report over R8 interface and then ADAE client can use DDCC to send direct report. - Need to added reference from TS 26.531 for service configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For push service experience report - For direct reporting , added reference to TS 26.531 - For pull service experience report, specfiied mechanism as per the suggestion from SA4 LS - Added reference from TS 26.531 for ADAE server cnfigurating ADAE client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Serice experience reporing from UE to AS will be incomplelte in Rel-18. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.1, 8.9.2.1, 8.9.2.2, 8.9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 26.531 CR 0007, TS 26.532 CR 0005 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.9.2	Procedure

#### 8.9.2.1	Push service experience information

The ADAE client determines the service experience information based on information received from the VAL client. The service experience information includes application specific performance measurements like end-to-end response time, connection bandwidth, request rate, server availability time, etc. On request VAL client or any other trigger conditions, the ADAE client sends the service experience report about a VAL server to the ADAE server using the mechanism defined in 3GPP TS 26.531 [3]. The information elements as defined in clause 8.9.3.1 are used by ADAE client to send the request and clause 8.9.3.2 are used by ADAE server to send the response.

For direct reporting, if data reporting session is not available, the ADAE client creates data reporting session as specified in clause 5.4 of 3GPP TS 26.531 [3]. Once data reporting session is available, the ADAE client (acting as a UE client) sends reports to ADAE server using direct reporting method as specified in clause 5.5 of 3GPP TS 26.531 [3].

The ADAE server may take further actions based on the analysis of the report as shared by the ADAE client. A service experience information from certain UEs, can trigger the ADAE server to fetch further service experience information other UEs. Use the service experience information report from other UEs, to determine/predict analytics.

-	If most of the UE side entities report a similar service experience, then it could be the application server problem across globally.

-	If only some UEs report a bad service experience, the problem could be localized among a group of UEs.

-	If the bad service experience from only a UE, the problem is localized to the UE.

* * * Next Change * * * *

#### 8.9.2.2	Pull service experience information

The procedure can be initiated by the ADAE server upon receiving a service experience from an ADAE client, to fetch service experience information other ADAE clients or upon receiving VAL server performance analytics request from application service provider (application server) or any other event that requires the ADAE server to determine the service experience data.

Figure 8.x.2.2-1: Pull service experience information from UE

1.	The ADAE server sends Pull service experience request to the ADAE client. The request contains identity of the specific VAL server and VAL service ID, for which the service experience report is required, as mentioned in Table 8.9.3.3-1.

2.	Upon receiving the Pull service experience request from the ADAE server, the ADAE client may take user consent to send the report if the user consent is not available already.

3.	The ADAE client sends the Pull service experience response to the ADAE server. The ADAE client instructs the Direct Data Collection Client to prioritise immediate delivery of a UE data report to the Data Collection AF as specified in clause 5.5 of 3GPP TS 26.531 [3]. The service experience contains parameters as specified in Table 8.9.3.4-1.

4.	The ADAE server uses the service experience report for derivation of VAL server performance analytics.

* * * Next Change * * * *

#### 8.9.2.3	Service experience information based on triggers

The ADAE server configures triggers to the ADAE client to send the service experience report using the mechanism defined in clause 5.4 of 3GPP TS 26.531 [3].

The procedure can be initiated by the ADAE server upon receiving VAL server performance analytics request from application service provider (application server). The information elements as defined in clause 8.9.3.5 are used by ADAE server to send the request and clause 8.9.3.6 are used by ADAE client to send the response.

* * * End of Change * * * *