3GPP TSG-SA WG6 Meeting #56	S6-232766

Gothenburg, Sweden 21st – 25th August 2023	(revision of S6-232761)

Source:	Huawei, HiSilicon, China Mobile

Title:	Pseudo-CR on key issue of Application calling service based on eMMTel service enhanced with Data Channel

Spec:	3GPP TR 23.700-92 v0.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

The FS_eMMTelAPP SID includes the following objective:

Study the application layer APIs which integrated with the underlying 3GPP core network architectures to allow exposure of relevant capabilities and other core network interactions to application providers and vertical service provider.

2. Reason for Change

SA2 already finished Rel-18 study on IMS Enhancement to support Data Channel, the IMS Architecture supporting Data Channel is listed in Annex AC of 3GPP TS 23.228 and Telco network capability APIs are listed in Annex AA.2.4 and Annex AA.2.5.

For SA6 it's very meaningful to hide telco complexity making APIs easy to consume for vertical customers and offer interactive features to application providers and vertical service provider.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.0.0.

* * * First Change * * * *

## 6.1	Key Issue #X: Application calling service based on eMMTel service enhanced with Data Channel

### 6.1.1	Description

Currently, in order to perform a call application providers and vertical service providers have to develop applications using specific protocols to access Call Control functions provided by network elements, this approach requires a high degree of network expertise, or they can use Third Party Call specified in 3GPP TS 23.198[x], but all the call service provided by TS 23.198 are only used for audio call. So, the overall scope of this application calling service is to provide functions to application providers and vertical service providers to create call in a simple way. Using the application calling service, application providers and vertical service providers can invoke call functions without detailed network architechtures.

In TS 23.228 [y], UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel. From the application providers perspective of view, there are two cases listed bellow:

-	Take the Contact Center as an example, the Contact Center can establish a call based on eMMTel service enhanced with Data Channel;

-	The customer has established a call without Data Channel based on eMMTel service, during the call, Contact Center needs to use Data Channel capabilities, then the Contact Center can utilize the service provided by the eMMTel service enabler layer to update the call support Data Channel.

The requirements for the application calling service may include but not limited to:

-	eMMTel service enabler layer needs to provide control capabilities of calling service to application providers and vertical service providers. The control capabilities include the creation, updating, and terminating of a call which includes audio, video and Data Channel.

So, the Key Issue is to study the application calling service enhanced including:

-	Identify whether and how to provide API based Call control capabilities utilizing architecture and procedure provide by IMS subsystem.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.198: "Open Service Access (OSA)".

[y]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

* * * End of Changes * * * *