3GPP TSG-SA WG6 Meeting #60	S6-241081

Changsha, China 15th – 19th April 2024	(revision of S6-xxxxxx)

Source:	NTT Docomo

Title:	CAPIF implementation of O-RAN Non-RT RIC application protocols

Spec:	3GPP TR 23.946

Agenda item:	9.7

Document for:	Approval

Contact:	kouhei.nozaki.rv@nttdocomo.com

1. Introduction

This pCR provides the use of the CAPIF API in the O-RAN WG2 Non-RT RIC as an example of CAPIF utilization.

2. Reason for Change

CAPIF implementation of O-RAN is not described in this TR. To understand the example of implementation, it is necessary to add information in O-RAN Non-RT RIC application.

3. Proposal

It is proposed to add the text of TR 23.946 as follows.

* * * First Change * * * *

6.x	O-RAN Non-RT RIC R1 application protocols based on CAPIF

O-RAN defines the RAN Intelligent Controller (RIC) for the purpose of RAN intelligence, conducting base station parameter design and setting, automation of operations, optimization, and more. As per the document [r1], the Non-Real time RIC (Non-RT RIC) is deployed within the Service Management and Orchestration (SMO) that oversees and maintains the RAN, enabling non-real-time control and optimization of RAN elements and resources, AI/ML workflow that includes model training and updating, and policy-based guidance for applications/functions in Near-RT RIC. Furthermore, in Non-RT RIC, applications referred to as Non-RT RIC Applications (rApp) are utilized to perform various information analyses and policy generation. The rApp architecture is independent of the Non-RT RIC framework, and is connected to the Non-RT RIC framework via the R1 interface. The registration, discovery, and subscription of service APIs in the R1 interface stipulated in Clause 6 of [r2] utilize the CAPIF API. Specifically, within the three procedures of service registration, service discovery, and service events subscription, CAPIF_Publish_Service_API, CAPIF_Discover_Service_API, and CAPIF_Events_API are respectively used as CAPIF APIs.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 V3.1.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[r1]	O-RAN Working Group 2 (Non-RT RIC and A1 interface WG): "Non-RT RIC Architecture".

[r2]	O-RAN Working Group 2 (Non-RT RIC and A1 interface WG): "R1 interface: Application Protocols for R1 Services".

* * * End of Changes * * * *