3GPP TSG-SA WG6 Meeting #60	S6-241094

Changsha, China 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0418 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove GW MC service ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In alignment with the LS reply S6-240551 from SA3, this CR removes the GW MC service ID from 23.379. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing GW MC service ID from A.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Violating security mechanism specified by SA3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.5	MCPTT service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [16]. The MCPTT service configuration data is stored in the MCPTT server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCPTT service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCPTT service. Data in tables A.5-1and A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCPTT service configuration data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [17] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [17] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.8-002] of 3GPP TS 22.280 [17] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCPTT administrator. | Y | N | Y |

Table A.5-2: MCPTT service configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an on-network private call | N | Y | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an on-network MCPTT group | N | Y | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max on-network private call (with floor control) duration | N | Y | Y |
| [R-6.2.4-003] of 3GPP TS 22.179 [2] | Hang timer for private calls | N | Y | Y |
| [R-6.7.2-008] of 3GPP TS 22.280 [17] | Max duration of private call (without floor control) | N | Y | Y |
| [R-6.2.3.3.1-001] of 3GPP TS 22.179 [2] | Hierarchy of participant rights to override | N | Y | Y |
| [R-6.2.3.5-002] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call transmission | N | Y | Y |
| [R-6.2.3.5-003], [R-6.2.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (on-network) | N | Y | Y |
| [R-6.2.4-005] of 3GPP TS 22.179 [2] | Configuration of warning time before call hang time (on-network) | N | Y | Y |
| [R-6.2.3.2-006] of 3GPP TS 22.179 [2] | Depth of floor control queue | N | Y | Y |
| [R-6.2.3.2-012] of 3GPP TS 22.179 [2] | Max time for a user's floor control request to be queued | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use signalling protection between MCPTT servers (see NOTE 1) | N | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | Use floor control protection between MCPTT servers (see NOTE 1) | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Functional alias | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [17] | > Communication priority (see NOTE 2) | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > This functional alias can be taken over | N | Y | Y |
|  | > List of users |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >> MCPTT ID | N | Y | Y |
| [R-5.6.3-015], [R-6.7.4-016] of 3GPP TS 22.179 [2] | Max number immediate forwardings | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [17] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | Ad hoc group call configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [17] | > Support of ad hoc group call (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [17] | > Maximum number of particpants allowed to participate in an ad hoc group call | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [17] | > Hang timer for ad hoc group call | N | Y | Y |
|  | > Maximum duration for ad hoc group call | Y | Y | Y |
|  | > List of preferred voice codecs for ad hoc group call | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [19]. NOTE 2: The usage of this parameter by the MCPTT server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

Table A.5-3: MCPTT service configuration data (off-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.3.2-002] of 3GPP TS 22.179 [2] | Timeout value for the cancellation of an in-progress emergency for an off-network private call | Y | N | Y |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | Time limit for an in-progress emergency related to an off-network MCPTT group | Y | N | Y |
| [R-5.6.5-004] of 3GPP TS 22.179 [2] | Max off-network private call (with floor control) duration | Y | N | Y |
| [R-7.4-002] of 3GPP TS 22.179 [2] [R-7.4-003] of 3GPP TS 22.280 [17] | Hang timer for private calls in off-network | Y | N | Y |
| [R-7.3.3-001], [R-7.3.3-002], [R-7.3.3-003] of 3GPP TS 22.179 [2] | Priority hierarchy for floor control override in off-network | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-002], [R-7.3.5-003] of 3GPP TS 22.179 [2] | Transmit time limit from a single request to transmit in a group or private call. | Y | N | Y |
| [R-7.3.5-001], [R-7.3.5-004] of 3GPP TS 22.179 [2] | Configuration of warning time before time limit of transmission is reached (off-network) | Y | N | Y |
| [R-7.4-004] of 3GPP TS 22.280 [17] | Configuration of warning time before hang time is reached (off-network) | Y | N | Y |
| [R-7.7-001], [R-7.7-003] of 3GPP TS 22.280 [17] [R-7.7-002] of 3GPP TS 22.179 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCPTT private call signalling | Y | N | Y |
|  | > MCPTT private call media | Y | N | Y |
|  | > MCPTT Emergency private call signalling | Y | N | Y |
|  | > MCPTT Emergency private call media | Y | N | Y |
| [R-7.15-001], [R-7.7-003] of 3GPP TS 22.280 [17] | Configuration of metadata to log | Y | N | Y |

# * * * * End of changes * * * *