3GPP TSG-SA WG6 Meeting #60	S6-241138

Changsha, China 9th – 13th April 2024	(revision of S6-241xxx)

Source:	Huawei, HiSilicon

Title:	pCR on key issue of data channel application related capability exposure

Spec:	3GPP TR 23.700-92 v0.4.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This contribution proposes a new KI for the FS_eMMTelAPP.

2. Reason for Change

It is specified in clause 6.2.10 of 3GPP TS 26.114[x] that the data channel application is created prior to the DCMTSI call where it is intended to be used, as the data channel workflow depicts below.

Normally, before the data channel application is used, the data channel application should be provided by the application provider or some other authorized party, then the MNO verifies the data channel application and stores it in the Data Channel Application Repository, as specified in clause AC.2.2.2 of 3GPP TS 23.228[y], The Data Channel Application Repository stores the verified data channel applications which are retrieved by the DCSF when required.

Furthermore, the profile configuration via eMMTel Enabler Server procedure has been approved in the Athens meeting, if the data channel application upload procedure is a offline procedure or implemented by SA2, The profile of the dat channel application and data channel application are delivered to the UE through two different paths, and extra interaction will be added between the eMMTel Enabler layer and the IMS core network, e.g. the eMMTel Enabler Server needs to check whether the application has been uploaded to the IMS Core before download the profile to UE, etc. So, it will be better that the profile of data channel application and data channel application both through the eMMTel Enabler layer. The following figure shows the process comparison between the two modes:

Figure 1: Uploading applications through the DCSF directly

Figure 1: Uploading applications through the eMMTel Enabler

As a result, eMMTel enabler can provide unified data channel application-related capabilities to enable the application provider or some other authorized party to manage data channel applications, including application upload, application upgrade, application deletion, application information retrieval etc.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.4.0..

* * * First Change * * * *

## 6.X	Key Issue #X: data channel application related capability exposure

6.X.1	Description

It is specified in clause 6.2.10 of 3GPP TS 26.114[14] that the data channel application is uploaded prior to the DCMTSI call where it is intended to be used. And as specified in clause AC.5 of 3GPP TS 23.228[3], the DC Application is uploaded to DCAR of DCSF. However, how to upload the data channel application to the network was not mentioned.

The eMMTel service enabler can provides unified data channel application related capabilities to enable application provider or some other authorized party to manage data channel application, including application upload, application upgrade, application deletion, application information retrieval etc.

Open issue:

-	How does the eMMTel service enabler layer provide service for the DC application provider to support application upload, application upgrade, application deletion, application information retrieval etc, based on the capabilities of the IMS subsystem.

Editor's note:	Whether and how cooperation with SA2 and SA5 is FFS.

* * * End of Changes * * * *