3GPP TSG-SA WG6 Meeting #60	S6-241193

Changsha, China 9th – 13th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0055 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for DAA ground-based UAE layer assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-03-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusions of FS_UAV_Ph3, SA1 specified 5G service requirements to further support UAV application in 3GPP system in TS 22.125. The SA6-WID has the following task, which refers to the general requirements in 3GPP TS 22.125 to be analysed for potential solutions: “analyse the new requirements in 3GPP TS 22.125 and provide potential enhancements to the overall architecture, procedures, information flows and APIs of the application layer architecture as required” Hence, enhancements to the application layer architecture are proposed to enable DAA (Detect And Avoid) ground-based UAE layer assistance, which is critical to safety and optimal flight-path planning of a UAV. DAA ground-based assistance is enabled by a Local DAA Ground Station (LDGS UAE client), that provides USS with assistance for UAVs monitoring and DAA. The proposed CR enables UAE layer support for DAA ground-based assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes comprises: - Introduction of LDGS capability. - Enhancement to Support of DAA management and configuration procedures when UAE client has LDGS capability. - Enhancement to Support of DAA client event information procedure when UAE client has LDGS capability. - Enhancement to Support of the DAA server event information procedure when UAE client has LDGS capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ground-based DAA assistance is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 7.1a.3.1, 7.1a.3.5, 7.7, 7.7.1, 7.7.2, 7.7.2.1, 7.7.2.2, 7.7.3, 7.7.3.1, 7.7.3.2, 7.7.3.3, 7.7.3.4, 7.7.3.5, 7.7.3.6, 7.7.3.7, 7.7.3.8, 7.7.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

DAA application policy: The configuration provided to the UAS application specific client by a UAS application specific server for handling of detect and avoid.

DAA assist capability: The functionality for the UAE layer to assist the UAV application specific layer with handling of detect and avoid during flight.

LDGS capability: A UAE client with LDGS capability offers the ground based DAA support for the UAVs in the monitoring range of the LDGS. The Local DAA Ground-Station (LDGS) can be operated by a USS and can serve one or more USSes.

Multi-USS capability: The functionality for the UAE layer to assist at change of USS during flight.

NOTE:	A UAV with Multi-USS capability can be controlled by more than one USS during a flight, but at any given time, the UAV is under the control of only one USS.

Multi-USS policy: The configuration provided by a UAS application specific server to assist at change of USS.

Remote Identification (Remote ID) of UAS: The ability of a UAS to provide identification and tracking information that can be received by other parties, to facilitate advanced operations for the UAS (such as Beyond Visual Line of Sight operations as well as operations over people), assist regulatory agencies, air traffic management agencies, law enforcement, and security agencies when a UAS appears to be flying in an unsafe manner or where the UAS is not allowed to fly.

UAS Service Supplier (USS): An entity that provides services to support the safe and efficient use of airspace by providing services to the operator / pilot of a UAS in meeting UTM operational requirements. A USS can provide any subset of functionality to meet the provider's business objectives (e.g., UTM, Remote Identification). In the scope of this specification, the term USS refers to both USS and USS/UTM.

UAV: The Uncrewed Aerial Vehicle (also called remotely piloted aircraft or drone) of a UAS.

For the purposes of the present document, the following terms given in 3GPP TS 22.125 [2] apply

Command and Control (C2) Communication

Uncrewed Aerial System (UAS)

Uncrewed Aerial System Traffic Management (UTM)

UAV controller

For the purposes of the present document, the following terms given in clause 4.2 of 3GPP TS 22.125 [2] apply

Direct C2 Communication

Network-Assisted C2 communication

UTM-Navigated C2 communication

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BVLOS	Beyond Visual Line Of Sight

C2	Command and Control

CAPIF	Common API Framework for northbound APIs

DAA	Detect And Avoid

LDGS	Local DAA Ground Station

QoS	Quality of Service

SEAL	Service Enabler Architecture Layer

UAE	UAS Application Enabler

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

UAV-C	Uncrewed Aerial Vehicle-Controller

USS	UAS Service Supplier

UTM	UAS Traffic Management

* * * Next Change * * * *

#### 7.1a.3.1	Registration request

Table 7.1a.3.1-1 describes the information flow for a UAE client to register with the UAE server.

Table 7.1a.3.1-1: Registration request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C/LDGS (e.g. 3GPP UE ID or CAA level UAV ID) which initiates the registration request. |
| UAS UE information | O | UAS UE information like IP address, Multi-USS capability, DAA assist capability, Dual Network-Assisted C2 communication link capability, real time flight path monitoring assistance capability, LDGS capability, etc. |
| Proposed registration lifetime (NOTE) | O | Proposed registration lifetime. |
| NOTE: If Proposed registration lifetime IE is not included, then the registration lifetime is valid until explicit deregistration is performed. |  |  |

* * * Next Change * * * *

#### 7.1a.3.5	Registration update request

Table 7.1a.3.5-1 describes the information flow for a UAE client to update registration with the UAE server.

Table 7.1a.3.5-1: Registration update request

| Information element | Status | Description |
| --- | --- | --- |
| UAV ID | M | The identifier of the UAV/UAV-C/LDGS (e.g. 3GPP UE ID or CAA level UAV ID) which initiates the registration request |
| UAS UE information | M | UAS UE information like IP address, Multi-USS capability, DAA assist capability, Dual Network-Assisted C2 communication link capability, real time flight path monitoring assistance capability, LDGS capability, etc. |
| Proposed registration lifetime | O | Proposed registration lifetime. |

* * * Next Change * * * *

## 7.7	UAE layer support for DAA services and applications

### 7.7.1	General

This feature enables the UAS application enablement services for assisting the UAS application with DAA handling. In particular, the UAE layer provides support for the following operations:

-	Support of the registration of the UAE clients DAA assistance capability and, if supported, the LDGS capability to the UAE server as described in clause 7.1a.

-	Support the distribution of the DAA application policy from the UAS application specific server to the UAE server and the UAE client, as described in clause 7.7.2.1.

-	Support the UAS application with DAA, as described in clause 7.7.2.2.

NOTE:	The Detect and Avoid operations are out of scope of the present specification.

### 7.7.2	Procedures

#### 7.7.2.1	Configuration of DAA policies to the UAE layer and the UAS client

##### 7.7.2.1.1	Management of DAA support configuration

Figure 7.7.2.1.1-1 illustrates the DAA support management procedure where the UAE server receives an application request for managing the DAA application policy from the UAS application specific server.

Pre-condition:

1.	The UAV has received its UAS ID from the UAS application specific server.

2.	The UAV has performed the UAS UE registration procedure.

Figure 7.7.2.1.1-1: DAA support management procedure

1.	The UAS application specific server sends to the UAE server a DAA support management request. The request includes the UAV (UAE client) identifier and the DAA application policy. If the procedure is for LDGS assisted DAA, then the request may include a list of UAE client identifiers (LDGS IDs) and/or geographical area so UAE server may select the relevant UAE client(s). The DAA application policy includes parameters for the control of LGDS assisted DAA.

2.	The UAE server shall send to the UAS application specific server a DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server shall timestamp and store the DAA application policy and execute the DAA configuration according to clause 7.7.2.1.2.

4.	After execution of DAA configuration, the UAE server shall send a DAA support management complete to the UAS application specific server.

##### 7.7.2.1.2	DAA support configuration procedure

Figure 7.7.2.1.2-1 illustrates the DAA support configuration procedure. This procedure enables the configuration of the UAE client, based on a request from UAS application specific server to configure the DAA application policy to the UAE client.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE server has established a UAE session with the respective UAE clients as the UAE clients are successfully registered to the UAE server.

3.	UAE server has performed the DAA support management procedure according to clause 7.7.2.1.1.

Figure 7.7.2.1.2-1: DAA support configuration procedure

1.	The UAE server shall send a DAA support configuration request to the UAE client. The UAE client receives a DAA support configuration request from the UAE server that includes the DAA application policy. If the UAE client has LDGS capability, and the procedure is for LDGS assisted DAA, then the DAA application policy includes parameters for the control of LGDS assisted DAA.

NOTE:	The UAE client can serve multiple USSes, hence, the UAE client receives configuration information pertaining to each USS.

2.	The UAE client shall store or remove the DAA application policy as per the information received in step 1. The DAA application policy is forwarded to the UAS application.

3.	The UAE client shall send a DAA support configuration response to the UAE server.

#### 7.7.2.2	UAE layer support for DAA applications

##### 7.7.2.2.1	DAA support involving UAVs with U2X support

Figure 7.7.2.2.1-1 illustrates the procedure with DAA support involving UAVs with U2X support. If the UAE client has LDGS capabilities, the UAE client provides ground based DAA assistance for the target UAVs in its monitoring range.

Pre-conditions:

1.	UAE server has provided the DAA application policy to the UAE client.

2	The UAE client can communicate with the target UAVs using A2X communication.

Figure 7.7.2.2.1-1: DAA support involving UAVs with U2X support

1.	The UAE layer has, e.g. based on information provided by the U2X layer, detected UAVs in proximity, see 3GPP TS 23.256 [4] clause 5.6. The UAV informs its own UAS application specific server about the detected collision. If the UAE client has LDGS capability, and the procedure is for LDGS assisted DAA, then based on the configuration/policy received from the USSes (e.g., DAA triggering thresholds), LDGS enforces deconfliction for the detected UAVs with a detected collision.

2.	The UAE client shall send a DAA client event information to the UAE server with information about one or more UAVs in proximity as detected or about collision resolution performed in step 1.

3.	The UAE server shall record the DAA client event information with current timestamp. UAE server shall request UAE client location information from the SEAL location services. The UAE server shall record the received location information with current timestamp. The UAE server shall send the DAA client event information to the UAS application specific server.

4.	The UAS application specific server provides a DAA client event information acknowledge to the UAE server. The UAS application specific server may include more information in the acknowledgement (e.g., other UAVs detected information by UAS application layer mechanisms).

5.	The UAE server shall send a DAA client event information acknowledge to the UAE client, and the UAE client shall provide the application layer (i.e. UAS client) with the consolidated information from the UAS application specific server.

##### 7.7.2.2.2	DAA support involving UAVs without U2X support

Figure 7.7.2.2.2-1 illustrates the procedure with DAA support involving UAVs without U2X support and the procedure DAA support involving UAS application specific server managed DAA, where UAS application specific server determines target UAVs in the proximity of LDGS and sends DAA flightpath update request for the target UAVs to an LDGS. LDGS then broadcast/unicast the change of flight path information to respective target UAVs.

Pre-conditions:

1.	UAS application specific server has provided DAA configuration parameters to the UAE client.

2.	The UAV does not support U2X layer.

3	The LDGS based on input from UAS application specific server suggests flight path update to target UAVs for deconfliction purpose.

Figure 7.7.2.2.2-1: DAA support involving UAVs without U2X support

1.	The UAS application specific server has received information about UAVs (e.g. via UAS application layer mechanism or as in step 3 of clause 7.7.2.2.1. The UAS application specific server determines the target UAVs that have potential flightpath conflict and the information of the UAVs which may be in the proximity or the monitoring range of the UAE client.

2.	The UAS application specific server sends a DAA server event information to the UAE server which includes information of other UAVs in the proximity or the monitoring range of the UAE client. The UAE server shall verify that the request is authorized before sending the DAA server event information to the UAE client.

3.	The UAE server performs coordination with Real-Time UAV connection status monitoring and location reporting is performed by the UAE server, see clause 7.5 and 3GPP TS 23.434 [5], clause 9.3. The UAE server shall send a DAA server event information to the UAE client comprising the UAVs information, corrected flightpath for each target UAV and location of each UAV in proximity of the UAE client.

4.	The UAE client shall send to the UAE server a DAA server event information acknowledge.

5.	The UAE server shall send a DAA server event information acknowledge to the UAS application specific server.

### 7.7.3	Information flows

#### 7.7.3.1	DAA support management request

Table 7.7.3.1-1 describes the information flow DAA support management request from the UAS application specific server to the UAE server.

Table 7.7.3.1-1: DAA support management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the DAA management. This ID can be the USS identifier, when the UAS application specific server is the USS. |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA support management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| List of UAS ID | O | A list of UAS identification information, e.g., CAA level UAV ID, of the UAVs that are identified as target UAVs, is included when UAE client has LDGS capability. |
| Geographical area | O | The area where the DAA client configuration applies to locate relevant LDGS capable UAE clients. This can be geographical area, or topological area. |
| The DAA application policy | O | The DAA application policy. |
| >DAA triggering thresholds | O | Indicate threshold for DAA, that determine when an LDGS may trigger detect and/or resolution of flightpath conflict. |
| >Time validity | O | Time validity where the DAA client configuration applies for an LDGS capable UAE client. |
| > Reporting frequency | O | Indication on how often the LDGS collect target UAVs information and how often UAS application specific server would like to be reported. This can be event based and / or time based. |

#### 7.7.3.2	DAA support management response

Table 7.7.3.2-1 describes the information flow DAA support management response from the UAE server to the UAS application specific server.

Table 7.7.3.2-1: DAA support management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the DAA support management request. |

#### 7.7.3.3	DAA support management complete

Table 7.7.3.3-1 describes the information flow DAA support management complete from the UAE server to the UAS application specific server.

Table 7.7.3.3-1: DAA support management complete

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of provision of the DAA application policy to the UAS application. |

#### 7.7.3.4	DAA support configuration request

Table 7.7.3.4-1 describes the information flow DAA support configuration request from the UAE server to the UAE client.

Table 7.7.3.4-1: DAA support configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| List of UAS ID | O | A list of UAS identification information, e.g., CAA level UAV ID, of the UAVs that are identified as target UAVs, is included when UAE client has LDGS capability. |
| The DAA application policy | O | The DAA application policy. |
| >DAA triggering thresholds | O | Indicate threshold for DAA, that determine when an LDGS may trigger detect and/or resolution of flightpath conflict. |
| >Time validity | O | Time validity where the DAA client configuration applies for an LDGS capable UAE client. |
| > Reporting frequency | O | Indication on how often the LDGS collect target UAVs information and how often UAS application specific server would like to be reported. This can be event based and / or time based. |

#### 7.7.3.5	DAA support configuration response

Table 7.7.3.5-1 describes the information flow DAA support configuration response from the UAE client to the UAE server.

Table 7.7.3.5-1: DAA support configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of provision of the DAA application policy to the UAS application. |

#### 7.7.3.6	DAA client event information

Table 7.7.3.6-1 describes the information flow DAA client event information from the UAE client to the UAE server and from the UAE server to the UAS application specific server.

Table 7.7.3.6-1: DAA client event information

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA client support information applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| UAE layer detected information | M | List of UASes where e.g. U2X layer or LDGS has detected possible flight path conflict. |
| > UAS identity | M | The identification of e.g. a U2X-UAS where U2X layer has detected possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Location information | M | Location of e.g. a U2X-UAS where U2X layer has detected possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Associated alert | M | Alert based on the action performed by an LDGS, e.g., collision detected, collision resolved etc. |
| > Updated flight path | O | Updated flight path, if the deconfliction is performed by the LDGS |
| Time of arrival | O | The time when UAS enters the monitoring range of an LDGS, reported by LDGS as event to USS. |
| Time of departure | O | The time when UAS leaves the monitoring range of an LDGS, reported by LDGS as event to USS. |

#### 7.7.3.7	DAA client event information acknowledge

Table 7.7.3.7-1 describes the information flow DAA client event information acknowledge from the UAS application specific server to the UAE server and from the UAE server to the UAE client.

Table 7.7.3.7-1: DAA client event information acknowledge

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA client support information acknowledge applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| UAE layer detected information | M | List of UASes where the UAS application specific server has confirmed possible flight path conflict. |
| > UAS identity | M | The identification of a UAS where UAS application specific server has confirmed possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Location information | M | Location of a UAS where UAS application specific server has confirmed possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Associated alert | M | Alert based on the action performed by an LDGS, e.g., collision detected, collision resolved etc. |
| > Updated flight path | O | Updated flight path, if the deconfliction is performed by the LDGS |
| Time of arrival | O | The time when UAS enters the monitoring range of an LDGS, reported by LDGS as event to USS. |
| Time of departure | O | The time when UAS leaves the monitoring range of an LDGS, reported by LDGS as event to USS. |

#### 7.7.3.8	DAA server event information

Table 7.7.3.8-1 describes the information flow DAA server event information from the UAS application specific server to the UAE server and from the UAE server to the UAE client.

Table 7.7.3.8-1: DAA server event information

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS (UAV or LDGS) for which the DAA server support information applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| UAE layer detected information | M | List of UASes where the UAS application specific server has confirmed possible flight path conflict. |
| > UAS identity | M | The identification of an UAS where UAS application specific server has confirmed possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Location information | M | Location of an UAS where UAS application specific server has confirmed possible flight path conflict or an LDGS where LDGS has detected possible flight path conflict via A2X or Uu communication. |
| > Associated alert | M | Alert based on the action performed by an LDGS, e.g., collision detected, collision resolved etc. |
| > Updated flight path | O | Updated flight path, if the deconfliction is performed by the LDGS |
| Time of arrival | O | The time when UAS enters the monitoring range of an LDGS, reported by LDGS as event to USS. |
| Time of departure | O | The time when UAS leaves the monitoring range of an LDGS, reported by LDGS as event to USS. |

#### 7.7.3.9	DAA server event information acknowledge

Table 7.7.3.9-1 describes the information flow DAA server event information acknowledge from the UAE client to the UAE server and from the UAE server to the UAS application specific server.

Table 7.7.3.9-1: DAA server event information acknowledge

| Information element | Status | Description |
| --- | --- | --- |
| Reason | M | Acknowledgement of DAA server event information. |

* * * End of Changes * * * *