3GPP TSG-SA WG6 Meeting #60	S6-241244

Changsha, P. R. China, 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0355 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of GW MC service ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Based on the LS from SA3 (S6-240551/S3‑240828), the connection authorization is not required in R18, and the related GW MC service ID should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the List of permitted GW MC service ID(s) item from table A.5-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between MC specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

# A.5	MCData service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [5]. The MCData service configuration data is stored in the MCData server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCData service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCData service. Data in tables A.5-1 and A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCData service configuration data (on and off network)

| Reference | Parameter description | MCData UE | MCData Server | Configuration management server |
| --- | --- | --- | --- | --- |

Table A.5-2: MCData service configuration data (on network)

| Reference | Parameter description | MCData UE | MCData Server | Configuration management server |
| --- | --- | --- | --- | --- |
| Subclause 6.2.2 of 3GPP TS 22.282 [3] | Transmission and reception control |  |  |  |
|  | > Maximum data size for SDS | Y | Y | Y |
|  | > Maximum payload data size for SDS over signalling control plane (see NOTE 1) | Y | Y | Y |
|  | > Maximum data size for FD | Y | Y | Y |
| [R-6.2.2.1-002d], [R-6.2.2.4-003] of 3GPP TS 22.282 [3] | > Time limit for the temporarily stored data waiting to be delivered to a receiving user | N | Y | Y |
| [R-6.2.2.3-001] of 3GPP TS 22.282 [3] | > Timer for periodic announcement with the list of available recently invited data group communications | N | Y | Y |
|  | > Maximum data size for auto-receive | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | > Functional alias | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >> Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >> This functional alias can be taken over | N | Y | Y |
|  | >> List of users who can activate this functional alias |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [17] | >>> MCData ID | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [17] | >> Communication priority (see NOTE 2) | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [2] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | MCData notification server |  |  |  |
|  | > Server URI(s) | Y | Y | Y |
|  | Ad hoc group data communication configurations |  |  |  |
| [R-6.15.5.3-005] of 3GPP TS 22.280 [2] | > Support of ad hoc group data communication (enabled/disabled) (see NOTE 3) | Y | Y | Y |
| [R-6.15.5.3-002] of 3GPP TS 22.280 [2] | > Maximum number of particpants allowed to participate in an ad hoc group data communication | Y | Y | Y |
| [R-6.15.5.3-004] of 3GPP TS 22.280 [2] | > Hang timer for ad hoc group data communication | N | Y | Y |
|  | > Maximum duration for ad hoc group data communication | Y | Y | Y |
|  | > List of preferred media codecs for ad hoc group data communication | Y | Y | Y |
| NOTE 1: The maximum payload data size for SDS over signalling control plane shall be less than or equal to the maximum data size for SDS. NOTE 2: The usage of this parameter by the MCData server is up to implementation. NOTE 3: If the support for ad hoc group data communication is disabled by the MC system then all other configurations related to ad hoc group data communication are not applicable |  |  |  |  |

Table A.5-3: MCData service configuration data (off network)

| Reference | Parameter description | MCData UE | MCData Server | Configuration management server |
| --- | --- | --- | --- | --- |
| Subclause 10.10 of 3GPP TS 23.280 [5] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCData one-to-one call signalling | Y | N | Y |
|  | > MCData one-to-one call media | Y | N | Y |